3GPP TSG RAN WG4 Meeting #115	R4-2506467

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.21.4

Source:	vivo

Title:	Discussion on RRM core requirement and testing framework for beam management

Document for:	Discussion

- Introduction

In RAN4#114bis meeting, there were some further discussions on key issues including KPIs related, the update on simulation assumption of measurement error, and the potential RRM requirements for beam prediction. The latest outcome was recorded in the latest WF [1].

In this contribution, we will further analyze the impacts on requirements for beam prediction and update some simulation results on proposed KPIs/ Test Metrics with consideration of the measurement error modelling derived from the updated simulation assumption [2].

- Requirements for beam predictions

In the previous meetings, the RRM impacts on beam management was discussed. The package of requirements for beam predictions was identlified as follows:

- Measurement period for prediction

- The KPIs on prediction accuracy

- TCI state known/unknown

- Performance monitoring

Based on the latest progress of each bullet, we will further provide our analysis on the requirements.

  - Prediction delay requirements

Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS

For the latency on prediction, the above agreement was reached in the RAN4#113 meeting. It was agreed that RAN4 would define the requirement for an overall delay. Next, we will analyze these two parts of delay in more detail.

- Measurement for prediction

This process refers to UE performing the L1-RSRP measurements for set B and obtaining the corresponding results as model inputs. Basically, the legacy measurement period on L1-RSRP can be as the reference for defining requirements on measurements of set B.

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

To be specific, the related parameters to be considered at least include:

- The number of samples for set B measurements

The number of samples will affect the prediction accuracy.

For spatial prediction, previous discussions and simulations have been based on the assumption that the measurement of set B relies on one-shot measurement.

For time domain prediction, since this case is more complex compared to the spatial domain, there has been a lack of discussion on this scenario. If we are to study how to define the corresponding delay, it must at least take into account the characteristics of time-domain prediction, including the number of samples corresponding to the latest measurement instances and the interval between time instances. Considering that the core part only has two meetings remaining, it would be very difficult to conduct an in-depth discussion on the time domain within such a short timeframe. Therefore, we believe that at least in Rel-19, we should not consider defining the core part and performance part requirements related to time-domain prediction.

Proposal 1: For spatial prediction, the measurement of set B relies on one-shot measurement

- Beam sweeping factor

For DL Tx beam prediction evaluations, RAN1 had some preliminary discussions and simulations of the hypothesis on the Rx beam as AI/ML model input for training and/or inference during the SI phase, including:

- Option 1: Measurements of the "best" Rx beam with exhaustive beam sweeping for each model input sample.

- Option 2: Measurements of specific Rx beam(s).

- Option 3: Measurements of random Rx beam(s) per model input sample.

- Option 4: Measurements of quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the "best" Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping.

Among them, option 1 is implemented in the same way as the existing requirement definition, which requires beam sweeping to find the best Rx beam. The value of the existing beam sweeping factor can be reused. However, since RAN1 did not further discuss the Rx beam assumption in the SI and WI phases, it is possible to implement it as any of the above four options. However, from the perspective of defining requirements, Option 1 can be considered as the worst-case assumption, where beam sweeping is required to find the best Rx beam. The definition of the beam sweeping factor follows existing requirements.

Proposal 2: For L1-RSRP measurement of set B, the definition and value of the beam sweeping factor follows existing requirements.

- sharing factor P

The value of this parameter is related to factors such as measurement gap and SSB/SMTC period. However, due to the introduction of multiple feature enhancements in previous releases, such as measurement gap enhancement, significant improvements have been made to P. For example, the value of P may now be related to configurations like MUSIM gap and NCSG. This raises a question: when defining AI/ML-based L1-RSRP measurement, should these enhanced features introduced by other projects in previous releases be supported? Or, for the sake of protocol simplification, should the requirement definition here be based on the R15 spec version。

Observation 1: Due to the introduction of multiple feature enhancements in previous releases, such as measurement gap enhancement, significant improvements have been made to P.

Proposal 3: When defining AI/ML-based L1-RSRP measurement, specifically for sharing factor P, RAN4 will discuss the coexistence supportability of the prediction with other previously enhanced features, the candidate Options include:

- Option 1: The AI/ML-based BM can be configured simultaneously with enhanced features introduced in previous releases. The corresponding L1-RSRP requirements need to take these enhancements into account, such as MUSIM gap, NCSG, etc.

- Option 2: Fom the perspective of spec simplification, the requirement definition here will be based on the R15 spec version and the enhanced features introduced in previous releases are not supported by default

- inference delay

For model inference delay, the inference process is mainly from the data input to the model to the corresponding inference result is derived. Additionally, there may be some data processing time on top of that, which part of the delay may be related to the specific inference scenario (the number of Set B and Set A, time domain prediction or spatial prediction, etc.) and the processing capability of the model. It needs to be further discussed how to determine the value by RAN4.

Based on the above considerations, taking CSI-RS as the example, the prediction delay TL1-RSRP_Prediction_Period_CSI-R is defined as

Table : Prediction period TL1-RSRP_Prediction_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS+ Tprediction_processing) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)+ Tprediction_processing) |
| DRX cycle > 320 ms | ceil(M*P*N)*TDRX + Tprediction_processing |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tprediction_processing is time for prediction post-processing. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

Proposal 4: For prediction delay requirements TL1-RSRP_Prediction_Period_CSI-RS for BM, it can be defined as

Table : Prediction period TL1-RSRP_Prediction_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS+ Tprediction_processing) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)+ Tprediction_processing) |
| DRX cycle > 320 ms | ceil(M*P*N)*TDRX + Tprediction_processing |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tprediction_processing is time for prediction post-processing, FFS on the value. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

2.2	TCI state known/unknown

Agreement in RAN4#113 meeting Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions Agreement in RAN4#114 meeting Issue 2-4: QCL Source RS for TCI states Agreement: - wait for RAN1 discussion to conclude without sending any LS

On whether the TCI state associated with a predicted Tx beam is known or unknown, the main divergence lies in whether the Rx beam is implicitly considered as known information for DL Tx beam prediction. In RAN4#113 meeting, RAN4 further discussed the issue of UE Rx beam knowledge and reached the further agreement as above. And in RAN4#114 meeting, RAN4 briefly discussed the QCL Source RS for TCI states.

On how to know Rx beams information, it is somehow implementation dependent. In some implementations, UE first predicts the Tx beam and further determines the best Rx beam paired with the Tx beam by beam sweeping. For some UEs with specific model implementation, the information of Rx beam may be implicitly indicated or included as the model input, and then UE can directly know Rx beam corresponding to the best TX beam. Basically, it is not preferred to use the latter implementation as the default assumption for the DL Tx beam prediction.

To move forward, we suggest firstly identifying the case that no beam sweeping is needed after Tx beam prediction Regarding the relationship between Set A and Set B. There are two cases in RAN1 evaluation, including:

Case i): Set A and Set B are different (Set B is NOT a subset of Set A).

Case ii): Set B is a subset of Set A.

对于Case ii)，At the stage of data collection for inference, UE performs measurements on set B with a specific Rx beam assumption. As we mentioned in clause 2.1, there are several options of hypothesis on the Rx beam as AI/ML model input for training and/or inference. Based on RAN1 simulation, Option 1 with the measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample, AI/ML provides better performance than other methods. If we take Option 1 as the base assumption, when Set B is a subset of Set A and the best predicted Tx beam is in Set B, the Rx information should be known because the best Rx beam corresponding to Set B has been found by beam sweeping before the prediction. It can be served as the known condition of TCI state.

For Case i), consider the scenario where the best predicted beam is not in set B but is in set A. In this case, methods to leverage Rx beam knowledge include:

- Introducing UE capability.

- Considering SSB-predicted CSI-RS, where the best Tx beam is not in set B but has a QCLed relationship with a beam in set B.

These two cases can also be captured as known conditions within the "Known condition of TCI state.". Based on the above analysis, the corresponding "Known condition of TCI state" can be reflected as follows:

Known condition of TCI state The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state or used for L1-RSRP measurement for predicted RS quantities reporting to the completion of active TCI state switch, - where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state, or the RS resource for L1-RSRP measurement for predicted RS quantities reporting is the RS in target TCI state or QCLed to the target TCI state - or where the RS resource for L1-RSRP measurement for predicted RS quantities reporting is the RS in target TCI state or QCLed to the target TCI state, but UE supports [UE capability for fast beam sweeping], - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command or predicted RS quantities reporting - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB Otherwise, the TCI state is unknown.

Proposal 5: For the UE Rx beam knowledge,

- at least for the case that the Top-K beams to be reported are in set B and set B is a subset of set A, if beam sweeping has been done during measurements on set B, UE Rx beam information is known by UE and no further beam sweeping is needed.

- for other cases, e.g., set B is not a subset of set A, RAN4 to further discuss how to obtain Rx beam information:

  - introducing UE capability

  - Considering SSB-predicted CSI-RS, where the best Tx beam is not in set B but has a QCLed relationship with a beam in set B, it can be also regard as known

Proposal 6: RAN4 to update the known TCI state conditions as

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state or used for L1-RSRP measurement for predicted RS quantities reporting to the completion of active TCI state switch, - where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state, or the RS resource for L1-RSRP measurement for predicted RS quantities reporting is the RS in target TCI state or QCLed to the target TCI state - or where the RS resource for L1-RSRP measurement for predicted RS quantities reporting is not the RS in target TCI state or QCLed to the target TCI state, but UE supports [UE capability for fast beam sweeping], - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command or predicted RS quantities reporting - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB Otherwise, the TCI state is unknown.

2.3	Performance monitoring

The details for performance monitoring have been basically finalized by RAN1. For information, we copy the latest RAN1 conclusions on performance monitoring for BM as follows.

Agreement For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB Agreement For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type: Monitoring report type Inference report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. P report; Support; Support; Support. Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling Agreement For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model: - Support the size of a set for monitoring is the same as the size of Set A, - The n-th resource in the set for monitoring is linked to the n-th resource in Set A. - Support the size of a set for monitoring is smaller than the size of Set A Working Assumption At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report. Working Assumption For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring - wherein N (N>=1) is configured in CSI-ReportConfig - FFS on additional rule for counting N linked pair For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring. Conclusion For UE-sided model, for BM-Case 2, for inference, AP CSI-RS for Set B is not supported. Working Assumption For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring, - only one resource set is configured in the CSI-ReportConfig - the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring - FFS on whether to configure more than one time instance - the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring - N (N>=1) is configured in the CSI-ReportConfig - FFS on additional rule for counting N linked pair - measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance

First, regarding performance monitoring, the discussions in RAN1 primarily focus on Type 1 Option 2, UE-assisted monitoring for BM.

For the KPI of performance monitoring, RAN1 defines it as at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A. Based on this, the content ultimately reported by the UE to the network is the number of correct predictions made by the UE out of N KPI calculations.

Agreement: - Delay requirements for performance monitoring reporting from the UE (e.g.., Type 1 Option2, UE-assisted monitoring for BM) in case-by-case manner - FFS on which use cases the requirement will be defined for - FFS on the details for each use case in WID

In the LCM discussions of the general part, the related conclusion as above was reached in the last meeting: for performance monitoring reporting, RAN4 agrees to define the relevant delay requirement. According to our understanding, this delay measures whether the UE can report the performance metrics within a certain latency upon performance monitoring is required. Compared to the measurement delay for prediction discussed in Section 2.1 above, the delay requirement for performance monitoring should be similar, with the difference being:

For the KPI of performance monitoring, RAN1 defines it as at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A. Based on this, the content ultimately reported by the UE to the network is the number of correct predictions made by the UE out of N KPI calculations. Therefore, this process monitors the prediction results over N instances, and thus the delay should correspondingly scale to N times. Specifically, the delay  for performance monitoring Tmoitoring_period_BM can be defined as:

Tmoitoring_period_BM = Nmonitor* TL1-RSRP_Prediction_Period_CSI-RS,

Where Nmonitor is the configured number of the latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring

Proposal 7: For performance monitoring for BM, the correasponding delay is defined as

Tmoitoring_period_BM = Nmonitor* TL1-RSRP_Prediction_Period_CSI-RS,

Where Nmonitor is the configured number of the latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring

  - KPIs related

2.4.1	RSRP prediction accuracy

2.4.1.1	Definition alignment

Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

- Absolute RSRP accuracy

According to the following conclusion passed in the previous RAN1 meeting, it can be seen that only the RSRP reported for the predicted Top 1 beam corresponds to the absolute predicted RSRP. Therefore, for the absolute predicted RSRP, it can be applied to the Top-1 of predicted beams.

Agreement For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #K. RSRP #1. differential RSRP #2. …. differential RSRP #K. - CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K - RSRP #1 is absolute predicted RSRP - Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1

Proposal 8: RAN4 to define absolute RSRP accuracy as one of metrics for beam management requirements/tests, the corresponding definition is:

- The absolute RSRP accuracy = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i corresponds to the index of Top-1 beam of predicted beams

- Relative RSRP accuracy

According to the RAN1 reporting format for RSRP mentioned above, except for the Top 1 predicted beam, the other beams report the differential values of the predicted RSRP relative to the Top 1 predicted beam. Therefore, for beams other than the Top 1 predicted beam, the requirement to be met is the Relative RSRP accuracy, where the relative value refers to the comparison with the Top 1 predicted beam.

Regarding whether the Reported L1-RSRP can be predicted, based on the following RAN1 conclusion, the Reported L1-RSRP can only be predicted (assuming beam index n is also one of the predicted best Top K beams).

RAN1#116bis Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. RAN1#120 Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion.

"Regarding FFS whether the reported L1-RSRP can be measured RSRP, that is, considering beam index n is not a predicted beam but a purely measured beam, the currently agreed ground truth definition is based on the UE's reported measurement time. In this case, the reported L1-RSRP of beam index n will cancel out with the ground truth of L1-RSRP of beam index n."

Proposal 9: RAN4 to use the follow definition to define the relative RSRP accuracy requirement:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - Reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value

    - Reported L1-RSRP refers to predicted L1-RSRP

2.4.1.2	Simulation results with consideration of measurement error

In this section, we present simulation results based on the latest assumptions [2] to evaluate the defined absolute RSRP accuracy requirements. We consider the performance impact of the following three cases on beam prediction accuracy in each KPI:

- Case 1: No error will be considered in training dataset, model input during inference and ground-truth.

- Case 2:

  - Case 2a No error will be considered in training dataset and ground-truth. Error will be considered in model input during inference

  - Case 2b No error will be considered in training dataset and ground truth for training. Error will be considered in model input during inference and ground-truth for inference

- Case 3: Error will be considered in training dataset, model input during inference and ground-truth.

Here, baseband (BB) error is modeled using LLS-based measurement error modeling with an assigned SNR of -3 dB. For RF error, Truncated Gaussian distribution under ±4.5dB RF error is used.

Table 1 90%-tile RSRP difference between the predicted L1-RSRP of the Top-K (K=1,2,3,4,5) predicted beams and the ideal L1-RSRP of the same beams

| Simulation results | L1-RSRP difference (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Top K | 1 | 2 | 3 | 4 | 5 |
| Case 1 |  |  |  |  |  |
| No error | 6.30 | 5.97 | 5.52 | 4.87 | 4.75 |
| Case 2a |  |  |  |  |  |
| BB error | 7.04 | 6.77 | 6.27 | 5.79 | 5.87 |
| RF error | 9.50 | 9.03 | 8.92 | 8.01 | 7.80 |
| Both BB and RF | 10.05 | 9.77 | 9.08 | 8.98 | 8.46 |
| Case 2b |  |  |  |  |  |
| BB error | 7.01 | 6.83 | 6.37 | 5.75 | 5.73 |
| RF error | 9.62 | 9.25 | 9.30 | 8.73 | 8.40 |
| Both BB and RF | 10.38 | 10.25 | 9.51 | 9.67 | 9.11 |
| Case 3 |  |  |  |  |  |
| BB error | 6.69 | 6.10 | 5.93 | 6.01 | 5.53 |
| RF error | 8.59 | 7.70 | 7.71 | 7.48 | 7.30 |
| Both BB and RF | 9.07 | 8.20 | 8.11 | 7.71 | 7.54 |

Table 2 show the simulation results of RSRP difference between the predicted L1-RSRP of the Top-K predicted beams and the ideal L1-RSRP of the same beams.

For baseline scenario (Case 1: No Error), the RSRP difference for Top-1 to Top-5 ranges from 6.30 dB to 4.75 dB, decreasing as K increases.

For Case 2a (Errors Only in Inference Input), with only BB Error, the difference increases by 0.7–1.12 dB (Top-1 is most sensitive). And for RF error, a significant increase in difference (+3.2–3.9 dB), showing that RF errors have a much larger impact than BB errors. When both B and RF error are considered: the difference reaches 10.05 dB (Top-1), demonstrating a clear additive effect of errors. And for Case 2b (Errors in Both Inference Input and Ground Truth), compared to Case 2a, the difference further increases (e.g., under RF error, Top-1 rises from 9.50 dB to 9.62 dB).

For Case 3 (Errors in Training, Inference, and Ground Truth), with only BB Error is considered, the difference (6.69 dB–5.53 dB) is close to Case 1 and even outperforms Case 2a/2b for some K values. The possible reason is that when training data includes errors, the model may learn the error distribution, improving robustness. For RF Error, the difference (8.59 dB–7.30 dB) remains significant but is lower than in Case 2b.

To sum up the analysis as above, the key conclusions are:

- Error Sensitivity Ranking:

RF errors > BB errors, indicating that noise in the RF chain has a more significant impact on system performance.

- Impact of Error Propagation:

- Errors in only the inference input (Case 2a) already cause significant performance degradation.

- Introducing ground truth errors (Case 2b) further deteriorates results, particularly in RF scenarios.

- Robustness of Error-Inclusive Training:

When training data includes errors (Case 3), the model demonstrates improved generalization against similar errors, but RF errors still require further optimization.

2.4.2	beam prediction accuracy

2.4.2.1	Definition alignment

Regarding the definition of beam prediction accuracy, significant progress was made in the last meeting, and the following conclusions were agreed upon through discussion.

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Regarding this conclusion, there are still several points to be discussed. We will analyze these points one by one.

Issue 1: Regarding Top-N

The currently approved KPI definition compares the predicted largest beam's ideal RSRP with the actual largest beam's ideal RSRP. Based on our understanding of Top-N here (the conclusion above does not clearly elaborate on Top-N; this understanding is derived from the papers submitted by companies and the discussions during the last meeting), in addition to this comparison, it is also necessary to compare the predicted i-th largest beam's ideal RSRP (i = all predicted beams) with the actual i-th largest beam's ideal RSRP (i = all predicted beams). This method is intended to verify whether the difference between the reported beam's actual RSRP and the corresponding ranked beam's RSRP falls within a certain range, ensuring to some extent that all beams are usable.

However, we believe the necessity of verifying this KPI is not very strong, mainly for the following reasons:

This definition is very similar to what we previously discussed in the RSRP difference KPI: comparing the predicted RSRP of the Top-K beams with the actual RSRP of the corresponding ranked beams. The only difference here is that the comparison is between actual RSRP values. According to the conclusions of previous meetings, the RSRP difference has already been agreed upon as a KPI, and its definition is quite similar to the two definitions mentioned above.

To some extent, this involves redundant comparisons. For example, the second-largest predicted RSRP beam not only needs to be compared with the second-largest actual RSRP beam but also, based on the approved part of the definition, if the second-largest predicted RSRP beam happens to be the largest actual RSRP beam, it must also be compared with the largest actual RSRP beam. This leads to too many comparison combinations to consider.

Proposal 10: For Metrics/KPIs for beam ID prediction agreed in the last meeting, RAN4 not to further consider the verification of top-N (N</=K) predicted beams in the success rate evaluation

Issue 2: Whether Beam ID Prediction Is Only Applied When Only Beam ID Is Reported

For cases where both RSRP and beam ID are reported, we prefer that beam ID accuracy should also be checked. The main reasons are:

- RSRP prediction (same ID) only ensures that the difference between the predicted and actual values falls within a certain range but does not guarantee that the Tx beam is the best. Legacy methods only consider RSRP accuracy because all values are measured. If the reported beam only satisfies RSRP accuracy (e.g., Beam 1 meets the requirement), Beam 1 may not necessarily be the strongest beam but could merely be among the actual Top-K beams. This prediction result does not reflect whether the model can correctly predict the best beam.

- If only RSRP is considered when both beam ID and RSRP are reported, and only the predicted Top-1 RSRP is checked, a possible scenario is: the predicted Top-1 is Beam 1, and its RSRP meets the accuracy requirement. However, Beam 1 might not even be among the reported Top-K beams (extreme case), making this beam practically unusable.

- From a monitoring perspective, the existing RAN1 metric for monitoring requires that at least one of the actual top-N beams (N=1 or 2) is within the predicted Top-K beams. It is unlikely that RSRP-related KPIs will be introduced. If the prediction metric is defined solely based on RSRP, a situation may arise where RSRP meets the accuracy requirement, but monitoring still fails because the two KPIs are evaluating different aspects.

Proposal 11: For Metrics/KPIs for beam ID prediction agreed in the last meeting, it will apply to the case only beam ID is reported and the case both beam ID and L1-RSRP are reported.

Issue 3: FFS on X Values

Regarding the value of X, in the simulation assumptions approved during the last meeting [2], we included three candidate values for X: 1, 2, and 3. We believe this value can be determined based on the simulation results and analysis submitted by each company.

KPI 1: Beam prediction accuracy - The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 1, 2, 3dB

Proposal 12: For the X value of Metrics/KPIs for beam ID prediction agreed in the last meeting, it can be determined based on the simulation results

Specifically, according to the results discussed in RAN1#120 meeting, the step size of RSRP is 1dB. Therefore, the possible values of X can only be positive integers, which are relatively straightforward to determine.

Agreement For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size. Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

2.4.2.2	Simulation result with consideration of measurement error

Here, we also conducted simulation verification on the successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 1, 2, 3dB. The simulation results are shown below.

Table 2 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction

| Beam prediction accuracy (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Top-1 | Top-2 | Top-3 | Top-4 | Top-5 |
| Case 1 |  |  |  |  |  |  |
| No error | X=1 | 86.01 | 90.32 | 93.40 | 94.64 | 95.86 |
|  | X=2 | 87.58 | 91.05 | 93.86 | 95.00 | 96.12 |
|  | X=3 | 88.70 | 91.78 | 94.26 | 95.32 | 96.37 |
| Case 2a |  |  |  |  |  |  |
| BB error | X=1 | 84.42 | 89.40 | 92.87 | 94.26 | 95.60 |
|  | X=2 | 86.94 | 90.50 | 93.40 | 94.77 | 96.08 |
|  | X=3 | 88.49 | 91.56 | 93.84 | 95.03 | 96.30 |
| RF error | X=1 | 73.56 | 83.20 | 89.24 | 91.80 | 93.65 |
|  | X=2 | 78.92 | 85.44 | 90.48 | 92.63 | 94.28 |
|  | X=3 | 82.23 | 87.14 | 91.41 | 93.27 | 94.74 |
| Both BB and RF | X=1 | 72.38 | 82.77 | 89.21 | 91.45 | 93.21 |
|  | X=2 | 78.20 | 85.46 | 90.75 | 92.51 | 94.12 |
|  | X=3 | 81.78 | 87.37 | 91.79 | 93.31 | 94.71 |
| Case 2b |  |  |  |  |  |  |
| BB error | X=1 | 81.35 | 88.89 | 92.55 | 94.01 | 95.49 |
|  | X=2 | 85.57 | 90.18 | 93.23 | 94.54 | 95.77 |
|  | X=3 | 87.58 | 91.32 | 93.80 | 94.96 | 96.04 |
| RF error | X=1 | 62.34 | 79.86 | 87.07 | 90.95 | 93.16 |
|  | X=2 | 68.91 | 82.80 | 88.82 | 91.87 | 93.80 |
|  | X=3 | 74.58 | 85.18 | 90.20 | 92.68 | 94.34 |
| Both BB and RF | X=1 | 59.43 | 79.32 | 86.90 | 90.71 | 92.80 |
|  | X=2 | 66.46 | 81.90 | 88.53 | 91.60 | 93.50 |
|  | X=3 | 71.81 | 84.49 | 89.97 | 92.49 | 94.12 |
| Case 3 |  |  |  |  |  |  |
| BB error | X=1 | 70.35 | 85.99 | 91.47 | 94.16 | 95.47 |
|  | X=2 | 78.07 | 88.15 | 92.63 | 94.84 | 95.96 |
|  | X=3 | 82.94 | 89.69 | 93.42 | 95.17 | 96.19 |
| RF error | X=1 | 58.40 | 78.65 | 86.94 | 91.13 | 93.46 |
|  | X=2 | 65.49 | 81.90 | 88.77 | 92.09 | 94.07 |
|  | X=3 | 71.55 | 84.59 | 90.38 | 92.97 | 94.64 |
| Both BB and RF | X=1 | 58.54 | 78.46 | 86.79 | 90.84 | 93.12 |
|  | X=2 | 65.12 | 81.74 | 88.61 | 91.89 | 93.86 |
|  | X=3 | 71.24 | 84.47 | 89.90 | 92.72 | 94.46 |

For No-Error Baseline (Case 1), Performance Upper Bound for Top-1 prediction success rate reaches 88.7% at X=3dB, while Top-5 achieves 96.37%, demonstrating the model’s potential under ideal conditions. The Success rate increases monotonically with K (e.g., for X=1dB: Top-1→Top-5: 86.01%→95.86%), as expected.

And for Impact of Introduced Errors (Case 2a vs. 2b vs. 3), RF error causes the most severe degradation (e.g., Case 2a-RF, Top-1 X=1dB: 73.56% vs. Case 1 86.01%). BB error has a milder impact (Case 2a-BB vs. Case 1 difference <2dB); combined errors behave similarly to RF alone. Besides, Case 2b performs worse (e.g., RF error, Top-1 X=1dB: 62.34% vs. 73.56%), indicating ground truth with error significantly affects evaluation metrics. For Case 3, the performance aligns with Case 2b, suggesting training data errors are overshadowed by inference-stage errors.

Regarding the threshold X influence, it improves success rates across all scenarios (e.g., Case 3-RF, Top-1: 58.40%→71.55%). Relaxing the tolerance (X) partially compensates for error effects.

- Testability issues for beam management

3.1 Channel model

In the RAN4#114 meeting, the following agreements about the channel models are captured

Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point

As we discussed in the previous meeting, a flat channel (e.g., TDL channel model) could not reflect the real performance of AI model and channel models with spatial characteristic need to be emulated in the test system. Thus, RAN4 should at least focus on the CDL channel as the test environment. However, CDL channel with multi-clusters are too complicated in the whole sphere space to emulate. Fig. 1. shows the power angular spectrum (PAS) of AoA and ZoA of CDL-A, CDL-B, CDL-C channel model with NLOS clusters in a sphere space.

| (a) CDL-A | (b) CDL-B | (c) CDL-C |
| --- | --- | --- |

Fig. 1 Illustration of PAS of CDL-A, CDL-B and CDL-C channels

Based on the theory of prefaded signal (PFS) method to emulate the channel in OTA chamber, a great number of probes and high-number ports channel emulator will be implemented, which is not feasible for vendors. Thus, RAN4 needs to find a way to simplified the test system. For option 5, it is a feasible way to simplified the CDL channel model with a small number of clusters, but the test dependability will degrade. Based on the AI BM characteristic, we could have other feasible way and here is the analysis.

Observation 2: The simplified CDL channel with small number clusters will degrade the test dependability.

3.1.1 Feasible way to simplify the test system

UE always recognize the AoA direction of the strongest power path as the reference path for beamforming. When some certain different Tx beams are sweeping and transmitted in the channel environment, the AoA of the strongest power path will not change while the power is still changing. Fig. 2 shows an example of the barlett power spectrum for AoA estimation with different Tx beams in fading channel environment, where the UE is assumed as a 4×4 antenna array. It can be observed that at least for beam sets 2-3, 5-10, 13-14-… the AoAs of the strongest path in UE side are the same.

Fig. 2. Bartlett power spectrum for AoA estimation with different Tx beams

Based on the above understanding, some Tx beams can be grouped together (e.g., beam 5-10) if the strongest receive power AoA are the same. Besides, there are different ways to group the Tx beams and needs to be studied. For example, setting a range for AoA of the strongest path and using this range to group Tx beams. Strongest path AoA based grouping is just an example for introducing this theory. An illustration of the Tx beam grouping is shown in Fig. 3.

Fig. 3. Tx beams grouping

Since only AoA of the signal needs to be emulated in the anechoic chamber while different Tx beams in one group are corresponding to only one AoA, which means that only one probe with one AoA to emulate multiple Tx beams in one group is feasible, and the number of groups equals to the number of probes. TE only needs to change the power of the specific probe in the specific time to emulate the power of the different Tx beams transmitted through fading channel, which makes the test system much easier. Fig. 4. shows an illustration of multi-probe anechoic chamber test system with the Tx beam grouping method. In this MPAC system, only power and direction need to be emulated which is applicable for AI/ML based BM case.

In Fig. 4., the probe 1, 2, …, L is for emulating the Tx beams in group 1, 2, …, L, respectively. Only one probe is allowed to be active at the same time for transmitting the beams in the corresponding group. The Tx beams are realized in BS emulator and the fading channel is realized in channel emulator (CE). When the probe 1 is active and probe 2 to L are deactive, probe 1 could emulate the same AoA and different power of different beams in group 1, which are the basic elements for AI/ML based BM testing (L1-RSRPs and spatial characteristics).

Fig. 4. Illustration of MPAC test system with Tx beam grouping for emulating AoA and power

Besides, if other channel characteristics needs to be reconstructed in quiet zoom, the test system in Fig. 5 can be referred to reconstruct the channel environment, e.g., spatial correlation, PAS, etc. However, more probes will be needed in this MPAC system.

In Fig. 5, the probe set [1, 2, …,K] is for emulating the Tx beams in group 1, and another set […, i, j,…] is for emulating the Tx beams in group 2, and so on. Only one set of probes is allowed to be active at the same time for transmitting the beams in the corresponding group. When the probes in probe set [1, 2, …,K] are active and other probe sets are deactive, probe set [1, 2, …,K] could emulate the spatial correlation, PAS. This test system could emulate a more accurate environment that the test system in Fig. 5., but more probes are needed.

Fig. 5. Illustration of MPAC test system with Tx beam grouping for reconstructing channel environment

Therefore, following procedures can be used for simplification:

Step 1. Generate the BS beamforming power pattern of every Tx beam

Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster

Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA

Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range)

Step 5. Using one or small number of probes to emulate one beam group.

Step 6. Delete the unnecessary probes and merge some adjacent probes.

Observation 3: The number of probes can be greatly reduced by grouping different Tx beams together. Only one probe or one set of probes can emulate different Tx beams in one group.

Observation 4: Different test system with reduced cost and complexity can be introduced by using the Tx beam grouping method.

Proposal 13: RAN4 to consider methods to group Tx beams together to reduce the number of probes in the test system.

Proposal 14: RAN4 to consider the following procedures to simplify the test system:

- Step 1. Generate the BS beamforming power pattern of every Tx beam

- Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster

- Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA

- Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range)

- Step 5. Using one or small number of probes to emulate one beam group.

- Step 6. Delete the unnecessary probes and merge some adjacent probes.

Proposal 15: RAN4 to consider the following two approaches for design the test system

approach a

approach b

3.2 Test setup

In RAN4#114 meeting, the following agreements have been achieved.

Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

For this issue, we provide some simulation results for the OTA probe setup under the NLOS channel CDL-A, B, C based on the method, where the 128 Tx beams set A beamforming characteristics are given as:

Azimuth angle in degree: [-84.375 -73.125 -61.875 -50.625 -39.375 -28.125 -16.875 -5.625 5.625 16.875 28.125 39.375 50.625 61.875 73.125 84.375]

Zenith angle in degree: [81 84 87 90 93 96 99 102]

The power patterns of the 128 Tx beams in azimuth cut and elevation cut (elevation angle in degree = zenith angle in degree – 90degree) are shown in Fig. 6.

(a)

(b)

Fig. 6 BS beamforming power pattern of 128 Tx beams

CDL-A

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 7 Probe allocation for CDL-A

CDL-B

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 8 Probe allocation for CDL-B

CDL-C

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 9 Probe allocation for CDL-C

Based on the probe allocation in Fig 7-9, it can be seen that some allocated probes are really close in angle-domain and can be merged and replaced by only one probe. Figure 10 shows an example for how to merging the probes in angle domain.

(a) original probe allocation for CDL-A, B, C (red: CDL-A, blue: CDL-B, green: CDL-C)

(b) new probe allocation after UE rotation in zenith angle domain

(c) 5 probe allocation after merge considering UE Rx resolution

Fig. 10 probe allocation after merge and rotation

In Fig. 10 (a), the allocated probes for CDL-A, B, C are illustrated and the total number is 31, which is not feasible. However, test should be performed under different channel models and no need to consider channel model switch (e.g., CDL-A to CDL-B) in one test. Thus, we can rotate the channel (rotate DUT in 3D-turnable) to narrow the channel model range in angular domain, which is shown in Fig. 10 (b). In Fig. 10 (b), all the channels are rotated in zenith angular domain and probes are symmetry about zenith angle = 90°. Besides, UE Rx beamforming has its resolution in angular domain, e.g., 30 deg. Fig. 10 (c) shows an example of probe allocation when considering UE Rx 30 degrees resolution, which eventually provided 5 candidates probe locations and greatly decreases the cost of the test system.

Proposal 16: Companies to provide candidate probe locations for CDL-A/B/C for merging and rotating. E.g.,

| (a) CDL-A | (b) CDL-B | (c) CDL-C |
| --- | --- | --- |

Proposal 17: RAN4 to consider the UE Rx beamforming resolution in angular domain to perform OTA probe allocations.

- Summary

Proposal 1: For spatial prediction, the measurement of set B relies on one-shot measurement

Proposal 2: For L1-RSRP measurement of set B, the definition and value of the beam sweeping factor follows existing requirements.

Observation 1: Due to the introduction of multiple feature enhancements in previous releases, such as measurement gap enhancement, significant improvements have been made to P.

Proposal 3: When defining AI/ML-based L1-RSRP measurement, specifically for sharing factor P, RAN4 will discuss the coexistence supportability of the prediction with other previously enhanced features, the candidate Options include:

- Option 1: The AI/ML-based BM can be configured simultaneously with enhanced features introduced in previous releases. The corresponding L1-RSRP requirements need to take these enhancements into account, such as MUSIM gap, NCSG, etc.

- Option 2: Fom the perspective of spec simplification, the requirement definition here will be based on the R15 spec version and the enhanced features introduced in previous releases are not supported by default

Proposal 4: For prediction delay requirements TL1-RSRP_Prediction_Period_CSI-RS for BM, it can be defined as

Table : Prediction period TL1-RSRP_Prediction_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS+ Tprediction_processing) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)+ Tprediction_processing) |
| DRX cycle > 320 ms | ceil(M*P*N)*TDRX + Tprediction_processing |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tprediction_processing is time for prediction post-processing, FFS on the value. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

Proposal 5: For the UE Rx beam knowledge,

- at least for the case that the Top-K beams to be reported are in set B and set B is a subset of set A, if beam sweeping has been done during measurements on set B, UE Rx beam information is known by UE and no further beam sweeping is needed.

- for other cases, e.g., set B is not a subset of set A, RAN4 to further discuss how to obtain Rx beam information:

  - introducing UE capability

  - Considering SSB-predicted CSI-RS, where the best Tx beam is not in set B but has a QCLed relationship with a beam in set B, it can be also regard as known

Proposal 6: RAN4 to update the known TCI state conditions as

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state or used for L1-RSRP measurement for predicted RS quantities reporting to the completion of active TCI state switch, - where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state, or the RS resource for L1-RSRP measurement for predicted RS quantities reporting is the RS in target TCI state or QCLed to the target TCI state - or where the RS resource for L1-RSRP measurement for predicted RS quantities reporting is not the RS in target TCI state or QCLed to the target TCI state, but UE supports [UE capability for fast beam sweeping], - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command or predicted RS quantities reporting - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB Otherwise, the TCI state is unknown.

Proposal 7: For performance monitoring for BM, the correasponding delay is defined as

Tmoitoring_period_BM = Nmonitor* TL1-RSRP_Prediction_Period_CSI-RS,

Where Nmonitor is the configured number of the latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring

Proposal 8: RAN4 to define absolute RSRP accuracy as one of metrics for beam management requirements/tests, the corresponding definition is:

- The absolute RSRP accuracy = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i corresponds to the index of Top-1 beam of predicted beams

Proposal 9: RAN4 to use the follow definition to define the relative RSRP accuracy requirement:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - Reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value

    - Reported L1-RSRP refers to predicted L1-RSRP,

Proposal 10: For Metrics/KPIs for beam ID prediction agreed in the last meeting, RAN4 not to further consider the verification of top-N (N</=K) predicted beams in the success rate evaluation

Proposal 11: For Metrics/KPIs for beam ID prediction agreed in the last meeting, it will apply to the case only beam ID is reported and the case both beam ID and L1-RSRP are reported.

Proposal 12: For the X value of Metrics/KPIs for beam ID prediction agreed in the last meeting, it can be determined based on the simulation results

Observation 2: The simplified CDL channel with small number clusters will degrade the test dependability.

Observation 3: The number of probes can be greatly reduced by grouping different Tx beams together. Only one probe or one set of probes can emulate different Tx beams in one group.

Observation 4: Different test system with reduced cost and complexity can be introduced by using the Tx beam grouping method.

Proposal 13: RAN4 to consider methods to group Tx beams together to reduce the number of probes in the test system.

Proposal 14: RAN4 to consider the following procedures to simplify the test system:

- Step 1. Generate the BS beamforming power pattern of every Tx beam

- Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster

- Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA

- Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range)

- Step 5. Using one or small number of probes to emulate one beam group.

- Step 6. Delete the unnecessary probes and merge some adjacent probes.

Proposal 15: RAN4 to consider the following two approaches for design the test system

Proposal 16: Companies to provide candidate probe locations for CDL-A/B/C for merging and rotating. E.g.,

Proposal 17: RAN4 to consider the UE Rx beamforming resolution in angular domain to perform OTA probe allocations.

- References

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm

- R4-2505152, Updated simulation assumptions for AI/ML BM, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT