3GPP TSG RAN WG4 Meeting#115                                    R4- 2506468

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.21.5

Source:	vivo

Title:	Discussion on RRM core requirement and testing framework for Positioning accuracy enhancement

Document for:	Discussion

- Introduction

RAN4 has studied many aspects about AI/ML use case aspects in Rel-18 SI stage, including CSI feedback enhancement, beam management enhancement and positioning accuracy enhancement. The outcomes of the study for positioning are captured in TR 38.843 [1]. In last meeting, the following agreement is achieved.

Issue 3-2: LOS/NLOS indicator Agreement: the LOS/NLOS indicator is out of scope of WID. Stop discussions on it. Issue 3-3: Rx-Tx time difference for case 3a Agreement: - Introduce gNB Rx-Tx time difference reporting accuracy requirements - Reuse legacy accuracy requirements Issue 3-4: Report mapping for gNB Rx-Tx time difference Agreement: - Reuse the existing report mapping for gNB Rx-Tx time difference measurements

In this contribution, we further provide our views on RRM core requirements and test framework for positioning accuracy enhancement.

- Discussion

For AI/ML positioning cases, 5 sub-use cases are identified and summarized in Table 1.

- A detailed list of various positioning cases

| Cases | Priority | Model deployment | Positioning methods | Measured by which entity | Model output | Position calculation |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1st priority | UE-side | Direct AI/ML positioning | UE (Downlink positioning) | Position | UE-side |
| Case 2a | 2nd priority | UE-side | AI/ML assisted positioning | UE (Downlink positioning) | Intermediate feature | LMF |
| Case 2b | 2nd priority | LMF-side | Direct AI/ML positioning | UE (Downlink positioning) | Position | LMF |
| Case 3a | 1st priority | gNB-side | AI/ML assisted positioning | gNB (uplink positioning) | Intermediate feature | LMF |
| Case 3b | 1st priority | LMF-side | Direct AI/ML positioning | gNB (uplink positioning) | Position | LMF |

- 2.1	Potential RAN4 requirement for case 1

In RAN2#126 meeting, agreements about UE side model are reached as

Agreements - The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. - wait for RAN1 for associate ID discussion - At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements

For case 1, RAN2 agreed to use the existing LPP procedures related to Location Information Transfer to provide the UE sided model inference results, which is the UE position, to the NW side. It means that there is a potential interactive process between NW and UE. RAN4 could consider define a delay requirement from the start of UE measuring PRS, through the AI processing, to the end of UE report location to LMF. Since further signaling enhancements may needed in RAN2, more progresses are also needed for defining delay requirement for case 1.

Proposal 1: RAN4 to define delay requirement for case 1 at least considering the following procedures: PRS measuring, AI processing. Legacy requirement (if exists) can be reused.

- 2.2	Potential RAN4 requirement for case 2

For case 2, RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024 according to the RANP, which is case 2. For case 2b, the channel measurements (power information, timing information, phase information) are used as the model input and need to be reported to LMF and RAN1 is discussing whether the measurements are sample-based or path-based since the last meeting. Now the discussing has to be postponed and RAN4 does not need to discuss this case anymore in the meeting.

Proposal 2: RAN4 to postpone the discuss of requirement for positioning case 2b.

However, UE needs to report intermediate features to LMF in case 2a and the procedure is similar as case 1, including PRS measuring, AI processing, and UE reporting. RAN4 could define delay requirements for case 2a by considering the mentioned aspects.

Proposal 3: RAN4 to define delay requirement for case 2a at least considering the following procedures: PRS measuring, AI processing, UE reporting intermediate features to LMF. Legacy requirement (if exists) can be reused.

- 2.3	Potential RAN4 requirement for case 3

For the UL positioning method case 3a and 3b, the model is deployed in gNB side and LMF side, respectively. RAN4 does not need to define any core requirement for such 2 cases.

Proposal 4: RAN4 not to define requirements for positioning case 3.

- 2.4 Potential RAN4 requirement for performance monitoring

In RAN1 meetings, following agreements about performance monitoring for case 1 and case 3a are reached as

RAN1#116bis Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. RAN1#118 For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. RAN1#119 Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

For case 3a, the monitoring procedures are performed at NW side and no need to define delay requirement for RAN4. For case 1, there are many options under discussion that may have RAN4 impacts for delay requirements.

For option A-1, the label-based monitoring metrics calculation is performed at UE side and the ground truth label is generated by LMF and provided to the target UE. In that case, LMF may indicate UE to perform monitoring procedure through signaling and then the target UE may inform LMF the monitoring decision or monitoring metrics calculation results (NW decision) through report. These procedures imply a delay requirement and the details are depending on RAN1/2 progress.

For option A-2 and 3, the label-based monitoring metrics calculation is performed at UE side and some assistance data can be transferred from LMF to UE side. These procedures may also have delay requirement and more progress of RAN1/2 are needed for discussing.

For option A-4, the PRU measurement are sent from PRU to the target UE, it is more like implementations and vendors could use different ways for sending information, e.g., OTT server, sidelink. Since RAN1 has agreed that it can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. RAN4 would not discuss this option for defining any core requirements.

For option B-1, the label-based monitoring metrics calculation is performed at LMF side and the model inference needs to be sent from target UE to LMF. In that case, it is very clear for RAN4 to identify the delay requirement. For delay requirement, the LMF may indicate UE to perform AI operations, and then UE reports the inference results (e.g. AI estimated UE position) to LMF.

For option B-2, the target UE uses the PRU channel measurements as the model input and outputs the estimated PRU position as the model inference result to report to LMF for performing monitoring procedure. It is also very clear that these procedures may also have the delay requirement from RAN4 perspective.

It can be observed that the delay requirements for many options about the monitoring procedure in case 1 needs to be defined, but more progress and down-selection from RAN1/2 are needed for delated further discussion.

Proposal 5: RAN4 to wait for more progress from other WGs to consider define delay requirements for monitoring for positioning case 1.

- 2.5 Testability issues

Conformance and pre-deployment testing

For the conformance or the pre-deployment test, the issue of how to obtain the ground truth value needs to be first raised to considered. UE cannot derive its ground truth value of position and intermediate feature (except LOS/NLOS indicator) during the conformance testing by any methodology, if the legacy test method is used. The only way for DUT to derive the ground truth value is to deploy some PRUs in the test system.

For direct AI/ML positioning, it is not possible for UE to obtain its ground truth value for position, no matter which methodologies are used, e.g., GNSS, motion sensor, OTDOA, etc. All these methods would bring errors and cannot be treated as the ground truth value of target UE. However, the PRU (positioning reference unit), which is a type of use equipment with known location information, can perform positioning measurements, is introduced in Rel-18 positioning. With the known location and measurement information of nearby PRU, UE/NW can perform label-based performance validation by comparing the model output position with the known PRU location, while the model input is the channel measurement of PRU.

For AI/ML assisted positioning, PRU measurements together with the location information are still needed. Also, some other information, e.g. locations of TRPs stored in AMF, is also needed. Based on the assisted information, the ground truth value of intermediate feature can be calculated by geometry. Similar label-based validation procedure can be performed as the above contents for direct AI/ML positioning.

Based on the analysis, the test can also be feasible by performing label-based model output performance validation, and the test metrics should be KPIs defined in RAN4 accuracy requirement.

However, the label-based model input validation may be hard to conduct since the ground truth value of channel estimation is difficult to obtain. One potential feasible way is to transmit a package of additional reference dataset from LMF or server to the AI-deployed entity, and the dataset can be used as the ground truth for model input. In that case, the post-deployment and pre-deployment test can be feasible by comparing the provided ground truth value and the estimated channel measurements, and the test metrics should be KPIs defined in RAN4 accuracy requirement, if the model input KPI is identified by RAN4. Besides, if the reference dataset contains the ground truth value of model input and output, the test can be easily performed. The dataset can be generated by entity vendors or defined in the specifications.

Observation 1: PRUs can be deployed in the test scenario to provide measurements together with the location information.

Proposal 6: RAN4 to consider deploy PRUs in conformance or the pre-deployment test so as to derive the ground truth value of UE position and intermediate features.

Generalization

AI/ML based positioning is a very area specific use case, especially for direct AI/ML positioning. According to RAN1 evaluation, assisted positioning with multi-TRPs is the methodology with better generalization performance than single-TRP. RAN1 also discussed other ways to improve the generalization performance, e.g., fine-tuning, which increases the performance in different drops. From RAN4 perspective, the testability for direct AI/ML positioning and assisted AI/ML positioning needs to be considered respectively. For the direct AI/ML positioning, RAN4 first needs to discuss whether to introduce the reference scenario/model/dataset. The test under reference scenario/model/dataset can provide a reference performance in certain conditions to guarantee the positioning performance would not degrade to an unexpected level in the field. For the assisted AI/ML positioning, since the generalization performance is better than AI/ML direct positioning, there may be no need to introduce the reference scenario/model/dataset, and the legacy test method can be reused. Further discussion is needed for the generalization aspects, and RAN4 needs to find out what kind of key factors would impact the performance of positioning.

Proposal 7: RAN4 to discuss the key factors that affect the performance and consider introduce reference scenario/model/dataset for AI/ML based positioning test for generalization.

- Summary

In this contribution, we provided our views on AI/ML based positioning. Based on above analysis, following observations and proposals are present.

Proposal 1: RAN4 to define delay requirement for case 1 at least considering the following procedures: PRS measuring, AI processing. Legacy requirement (if exists) can be reused.

Proposal 2: RAN4 to postpone the discuss of requirement for positioning case 2b.

Proposal 3: RAN4 to define delay requirement for case 2a at least considering the following procedures: PRS measuring, AI processing, UE reporting intermediate features to LMF. Legacy requirement (if exists) can be reused.

Proposal 4: RAN4 not to define requirements for positioning case 3.

Proposal 5: RAN4 to wait for more progress from other WGs to consider define delay requirements for monitoring for positioning case 1.

Observation 1: PRUs can be deployed in the test scenario to provide measurements together with the location information.

Proposal 6: RAN4 to consider deploy PRUs in conformance or the pre-deployment test so as to derive the ground truth value of UE position and intermediate features.

Proposal 7: RAN4 to discuss the key factors that affect the performance and consider introduce reference scenario/model/dataset for AI/ML based positioning test for generalization.

- References

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18).