3GPP TSG RAN WG4 Meeting #115	R4-2506469

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.21.6

Source:	vivo

Title:	Discussion on RRM performance requirements for beam management and positioning accuracy

Document for:	Discussion

- Introduction

The RRM core requirements for AI/ML-based beam management were discussed in the past several meetings. According to the working plan, the performance part of this WI will begin to be discussed in this meeting. Therefore, in this paper, we will provide our preliminary analysis on the following aspects:

- Measurement accuracy requirement

- RRM test cases

- Discussion

- 2.1 measurement accuracy

Regarding the measurement accuracy, a relatively thorough analysis has already been conducted in the core part section. As for BM-Case 1, the supported reporting content is as follows, mainly including:

- CRI or SSBRI of beam K

- Absolute predicted RSRP for Top-1 beam

- Differential predicted RSRP with reference to the largest predicted RSRP

Agreement For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #K. RSRP #1. differential RSRP #2. …. differential RSRP #K. - CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K - RSRP #1 is absolute predicted RSRP - Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1

Regarding this prediction accuracy, the corresponding accuracy requirement needs to be defined. Since RAN4 has not previously established similar requirements, the basic framework and conditions for this accuracy requirement need further discussion within RAN4. To some extent, this also depends on the ongoing discussions regarding testability. As for deriving the specific accuracy values, based on our understanding, they should be determined through the simulations currently being conducted.

Secondly, for Absolute Predicted RSRP and Differential Predicted RSRP, when the predicted RSRP is required to be reported, its accuracy must be ensured. For Absolute Predicted RSRP, the existing Absolute RSRP accuracy requirements for FR2 can serve as a reference framework for defining this requirement. For Differential Predicted RSRP, similar to the existing Relative RSRP accuracy requirements for FR2, when the UE reports more than one beam, this requirement must also be checked. The specific accuracy values corresponding to these two requirements, as well as the beam prediction accuracy, should be derived from ongoing simulations.

Additionally, for performance monitoring, RAN1 has also defined corresponding performance metrics for Top 1 or Top K beam prediction accuracy. From the perspective of UE performance monitoring, the UE shall determine the RS-PAI (Reference Signal Prediction Accuracy Indicator) as the total count of accurate reference signal prediction instances. Accordingly, based on RAN1’s current discussions, the UE needs to report the number of correct predictions out of N inference attempts. Therefore, from a performance standpoint, we believe it is necessary to verify whether the UE correctly reports the number of accurate predictions out of N inference attempts. RAN4 needs to define the requirements for this.

Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling Agreement Working Assumption For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring - wherein N (N>=1) is configured in CSI-ReportConfig - FFS on additional rule for counting N linked pair For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

Proposal 1: For RRM performance requirements for beam management, RAN4 to consider the following requirements；

- Absolute Predicted RSRP accuracy

- Relative Predicted RSRP accuracy

- Monitoring accuracy for performance accuracy (to verify whether the UE correctly reports the number of accurate predictions out of N inference attempts.)

- 2.2 RRM test cases

For the RRM requirements to be verified, based on the currently agreed definitions or procedures under discussion, the main aspects include the following:

- For the prediction delay requirement, it primarily involves L1-RSRP measurement and potential model processing delays. For this requirement, relevant test cases need to be defined to verify whether the UE can complete the prediction behavior and report the prediction results within the specified delay.

- Test case for performance monitoring reporting delay requirements, mainly includes measurements configured by the network for monitoring and the calculation of performance metrics (e.g., Top 1 or Top K beam prediction accuracy). It verifies whether the UE can compute the performance metrics and report the monitoring results within the specified delay.

- Test case for functionality activation delay requirement, primarily covers the delay for activating the functionality. It checks whether the UE can complete the activation of the functionality and correctly perform inference within the specified delay.

Proposal 2: For test cases for AI/ML based beam management, RAN4 to define RRM test cases for the following requirements:

- Prediction delay requirement

- Performance monitoring reporting delay requirements

- Functionality activation delay requirement

- Summary

Proposal 1: For RRM performance requirements for beam management, RAN4 to consider the following requirements；

- Absolute Predicted RSRP accuracy

- Relative Predicted RSRP accuracy

- Monitoring accuracy for performance accuracy (to verify whether the UE correctly reports the number of accurate predictions out of N inference attempts.)

Proposal 2: For test cases for AI/ML based beam management, RAN4 to define RRM test cases for the following requirements:

- Prediction delay requirement

- Performance monitoring reporting delay requirements

- Functionality activation delay requirement

- References