3GPP TSG-RAN WG4 Meeting #115	R4-2506477

St. Julian’s, Malta, 19th – 23rd May, 2025

Agenda Item:	7.1.4

Source:	Ericsson

Title:	On UE demodulation and CSI reporting requirements for 6Rx

Document for:	Discussion

# 1	Introduction

Following the agreements made in the RF session below, the following UE demodulation and CSI reporting requirements will be defined but applied to handheld UE and FWA devices separately.

Issue 2-1-1: Way forward for 6-layers support for handheld UE and FWA devices Way Forward: - RAN4 Chair to take note of the following in the meeting report: - “RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19” - 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications.

In this paper, we will present our view on UE demodulation and CSI reporting requirements for enhanced 6Rx in FR1.

# 2	Discussion

After the meeting in Wuhan, RAN4 has made several agreements regarding the general issue and scope for ATG UE demodulation requirements.

Issue 1-1: Single carrier test - Agreement - Only define single carrier test for 6Rx, i.e. no-CA related tests. Issue 1-2: FR1 TDD test - Agreement - Only define test cases for FR1 TDD - 30kHz SCS - 40MHz CBW - TDD UL-DL pattern: 7DS2U, S: 6D+4G+4U Issue 1-3: PDCCH test - Agreement - Not define PDCCH demodulation requirements Issue 1-4: PBCH test - Agreement - Not define PBCH demodulation requirements

Regarding the test applicability rule for 6Rx test cases, following two candidates have been discussed:

Issue 1-5: Test applicability rule for 6Rx test cases - Way forward - Option 1: RAN4 can introduce UE declaration on the 6Rx UE type (e.g., handheld or FWA) without capability signaling and design proper test applicability in the specification. - Option 2: - 6 MIMO layers performance requirements will be only applicable for FWA devices - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

Since there is no clear definition on ‘handheld UE’ and ‘FWA’ in the core specification, it is preferred to resolve the test applicability issue within RAN4 performance scope. Similar to what RAN4 implemented for advanced receiver requirements in previous releases, it is feasible to define UE declarations without capability signaling to distribute different test cases to different UE/receiver types. Thus, we prefer option 1 so that RAN4 can introduce UE declaration on the 6Rx UE types without capability signaling, and design proper test applicability rules in the specification (TS38.101-4).

Followings are the proposed applicability:

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| [6Rx handheld UEs] | FR1 FDD | PDSCH | Clause 5.2.5.x.x(Tests 1-1,3-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.5.x.x(Tests 1-1,3-1) |  |
| [6Rx FWA UEs] | FR1 FDD | PDSCH | Clause 5.2.5.x.x(Tests 2-1,4-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.5.x.x(Tests 2-1,4-1) |  |

Proposal 1: RAN4 to introduce UE declaration on the 6Rx UE types without capability signaling, and design proper test applicability rules in the specification

## 2.1	PDSCH demodulation requirements

RAN4 has made following agreements for PDSCH requirements so far:

Issue 2-1: Receiver type assumed for UE performance requirements definition - Agreement - Single receiver type of MMSE-IRC with joint 6Rx MIMO detector Issue 2-3: Test cases for 6 MIMO layers for FWA - Agreement - A single test case for 6 MIMO layers for FWA Issue 2-5: Tx EVM assumption - Agreement - 6% for QPSK/16QAM/64QAM - 3% for 256QAM Issue 2-9: DMRS configuration for 4 MIMO layers case - Agreement - Rel-15 Type 1 DMRS 1+1 Type 1 Issue 2-10: Antenna configuration for 6 MIMO layers case - Agreement - 8Tx6Rx Issue 2-14: N1, N2 for codebook - Agreement - 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) - 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1) (if 4Tx6Rx configuration for 4 MIMO layers case is agreed)

There were proposals to consider interference cancellation along with introducing 6Rx requirements. Although the interference cancellation is not within the WID for now, it is important as well to consider it similar to the discussion to include interference cancellation for 8Rx in Rel-18. Different from the situation in Rel-18, now the interference cancellation was brought up in the first meeting of this WI, plus there is a mature interference model that can be directly reused (interference profiles for inter-cell interference and intra-cell inter-user interference for 8Rx) so that there will be less discussion effort for the 6Rx. Thus, it’s also our interest to consider test scenarios with interference.

Proposal 2: Consider test scenarios with interference

- PDSCH demodulation requirements with inter-cell interference

- PDSCH demodulation requirements with intra-cell inter-user interference

Proposal 3: Reuse the interference profile for 8Rx

As for the issue of how many test cases will be designed with 4 MIMO layers, there were two options under discussion:

Issue 2-4: Test cases for 4 MIMO layers for FWA and Handheld UE - Way forward - Option 1: One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE - Option 2: Two separate test cases for 4 MIMO layers for FWA and Handheld UE - Option 2a: One test case with ULA Low for FWA; Another test case with ULA Medium with β = 0.00718 for Handheld UE - Other options

In our opinion, a single test case is insufficient to cover two distinct UE types due to their differing characteristics. We recommend designing dedicated test cases for each UE type to ensure a more realistic approach. If RAN4 allows, two separate test cases could be created for the 4-MIMO layer, where different MIMO correlations, propagation models, or even MCS values can be selected for each UE type.

Proposal 4: Define two separate test cases for 4 MIMO layers for FWA and handheld UE, detailed correlation can be decided by evaluation results.

For the propagation condition, we propose to consider TDLA30-10 for the case of 6 MIMO layers; to consider both TDLA30-10 and TDLC300-100 for 4 MIMO layer cases.

Proposal 5: Consider TDLA30-10 for 6 MIMO layer case

Proposal 6: Consider TDLA30-10 for 4 MIMO layer FWA case, TDLC300-100 for 4 MIMO layer handheld case

Regarding the MIMO correlation, we have evaluated the following candidates for ULA medium:

Issue 2-7-2: MIMO correlation for 4 MIMO layers - Way forward - Option 1: ULA Medium RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1 - Option 1a: with β = 0.00718 - Option 1b: with β = 0.005154 - Option 2: ULA Low

Based on our initial results [3], we found both option 1a and 1b are feasible. For a dedicate test case for handheld UE with 4 MIMO layer, we propose to consider option 1a as it is proved to be closer to one of the UE implementations.

Proposal 7: Consider option 1a for the dedicate test case for handheld UE with 4 MIMO layer.

For the DMRS configuration of 6 MIMO layer case, the following options have been discussed:

Issue 2-8: DMRS configuration for 6 MIMO layers case - Way forward - Option 1: Rel-15 Type 1 DMRS 2+2 - Option 2: Rel-15 Type 1 DMRS 2+0 - Option 2: Rel-18 eType1 DMRS 1+1

We have evaluated all three configurations, and from our observation there are only minor differences among all three results, see details in [3]. We think it’s sufficient to consider option 1: Rel-15 Type 1 DMRS 2+2 for the 6-layer requirements.

Proposal 8: Consider option 1: Rel-15 Type I DMRS 2+2 for 6-layer cases

For MCS selection, there were also quite many candidates proposed in the last meeting:

Issue 2-12: MCS for 6 MIMO layers case - Way forward - Option 1: MCS13 (Table 2) - Option 2: MCS 13 as starting point (Table 1) - Option 3: 256QAM - Option 4: 64QAM (MCS Table 1) with the same MCS for both CW - Option 5: MCS17 as starting point (Table 1) Issue 2-13: MCS for 4 MIMO layer case - Way forward - Option 1: MCS13 (Table 2) : 64QAM with CR = 567/1024 - Option 2: MCS 13 as starting point (Table 1) : 16QAM with CR = 490/1024 - Option 3: 256QAM - Option 4: 64QAM (MCS Table 1) - Option 5: MCS17 as starting point (Table 1): 64QAM with CR = 438/1024 - Option 6: MCS13 and MCS 17 as starting point

We have also evaluated different MCS values with both 4 and 6 MIMO layers. For 6-layer cases, we prefer to consider MCS at least equal or higher than 64QAM. From our results [3], we can observe that the maximum throughput can not be reached with 256QAM. The performance with 64QAM is good and within a reasonable range.

Observation 1: For both 4-layer and 6-layer cases, the maximum throughput can not be reached with 256QAM

Observation 2: For 6-layer cases, the performance with 64QAM is good and within a reasonable range

Observation 3: For 4-layer case, the SNR point for 64QAM is quite high

RAN4 have defined PDSCH requirements with MCS17 (64QAM, 0.43) for rank 4 and rank 8 with 8Rx reception.

Table 5.2.4.1.1-5: Minimum performance for Rank 4 with Baseline SU-MIMO 8Rx receiver Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 3-1; R.PDSCH.1-3.7 FDD; 10 / 15; 64QAM, 0.43; TDLA30-10; 4x8, ULA Low; 70; 12.8. Table 5.2.4.1.1-6: Minimum performance for Rank 4 with Simplified SU-MIMO 8Rx receiver Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 4-1; R.PDSCH.1-3.7 FDD; 10 / 15; 64QAM, 0.43; TDLA30-10; 4x8, ULA Low; 70; 15.8. Table 5.2.4.1.1-7: Minimum performance for Rank 8 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 5-1; R.PDSCH.1-3.8 FDD; 10 / 15; 64QAM, 0.43; TDLA30-10; 8x8, ULA Low; 70; 22.5.

Based on the above results and analysis, we propose to consider 64QAM for 6-layer requirements.

Proposal 9: Consider 64QAM for 6-layer requirements

## 2.2	CSI reporting requirements

Similar to Rel-18 8Rx, it is necessary to define CQI reporting requirements for both handheld UEs and FWA UEs with 6Rx. For those handheld UEs and FWAs that only support up to 6Rx, the CQI reporting requirement can not be covered by the existing 8Rx CQI reporting requirements. Following main assumptions are proposed as follows:

- Rank: 4

- Propagation channel:  AWGN

- Antenna: 4T6R with static channel specified in Annex B.1

Reuse other parameter assumptions for 8Rx CSI reporting requirements.

In general, we propose to define one case and apply it to both handheld UEs and FWA UEs, which means there is no need to define additional applicability rules for 6Rx CQI reporting requirements.

Proposal 10: Consider following parameter assumptions:

- Rank: 4

- Propagation channel:  AWGN

- Antenna: 4x6 with static channel specified in Annex B.1

Proposal 11: RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rule.

## 2.3	SDR requirements

The purpose of the SDR requirements is to verify that Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. Following the same approach of Rel-18 8Rx, we propose to evaluate 6Rx SDR performance in the first stage with both 4 and 6 MIMO layers.

SDR requirements can be defined for 6 MIMO layers, and the MCS index table should be extended accordingly. For 1024QAM, it’s agreed to define SDR requirements with 2 layers. We propose to study the feasibility of 4 layers.

Proposal 12: Define SDR requirements and extend the MCS index table to 6 MIMO layers, considering

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 6’

  - RAN4 to study the performance of 6 MIMO layers with different maximum modulation format

- Update Table 5.5A-6 (1024QAM) to support 6Rx.

  - RAN4 to study the Maximum number of PDSCH MIMO layers, e.g. 4

# 3	Summary

In this contribution, we shared our views for the demodulation requirement discussion for 6Rx covering both handheld UEs and FWA. Proposals are summarized as follows:

Proposal 1: RAN4 to introduce UE declaration on the 6Rx UE types without capability signaling, and design proper test applicability rules in the specification

Proposal 2: Consider test scenarios with interference

- PDSCH demodulation requirements with inter-cell interference

- PDSCH demodulation requirements with intra-cell inter-user interference

Proposal 3: Reuse the interference profile for 8Rx

Proposal 4: Define two separate test cases for 4 MIMO layers for FWA and handheld UE, detailed correlation can be decided by evaluation results.

Proposal 5: Consider TDLA30-10 for 6 MIMO layer case

Proposal 6: Consider TDLA30-10 for 4 MIMO layer FWA case, TDLC300-100 for 4 MIMO layer handheld case

Proposal 7: Consider option 1a for the dedicate test case for handheld UE with 4 MIMO layer.

Proposal 8: Consider option 1: Rel-15 Type I DMRS 2+2 for 6-layer cases

Observation 1: The maximum throughput can not be reached with 256QAM

Observation 2: The performance with 64QAM is good and within a reasonable range

Observation 3: For 4-layer case, the SNR point for 64QAM is quite high

Proposal 9: Consider 64QAM for both 6-layer and 4-layer requirements

Proposal 10: Consider following parameter assumptions:

- Rank: 4

- Propagation channel:  AWGN

- Antenna: 4x6 with static channel specified in Annex B.1

Proposal 11: RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rule.

Proposal 12: Define SDR requirements and extend the MCS index table to 6 MIMO layers, considering

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 6’

  - RAN4 to study the performance of 6 MIMO layers with different maximum modulation format

- Update Table 5.5A-6 (1024QAM) to support 6Rx.

  - RAN4 to study the Maximum number of PDSCH MIMO layers, e.g. 4

# References

[1]	RP-240828: New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei), RAN #103, Maastricht, Netherlands

[2]	R4-2504750: Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, RAN4 #114bis, Wuhan, China

[3]	R4-2506478: Simulation results for 6Rx UE demodulation, Ericsson, RAN4 #115, St Julian’s Malta

# Annex

Parameter configurations and example test cases for information:

Table 5.2.5.1.1-2: Test parameters Parameter; Unit; Value. Duplex mode; FDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PDSCH aggregation factor; 1. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 0. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1 for rank <= 4 2 for rank > 4. Codebook configuration; CodebookType; typeI-SinglePanel. CodebookMode; 1. (CodebookConfig-N1,CodebookConfig-N2); (2,1) for 4Tx (4,1) for 8Tx. (CodebookConfig-O1,CodebookConfig-O2); (4,1). CSI-RS for tracking; First OFDM symbol in the PRB used for CSI-RS; l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4. Number of HARQ Processes; 4. The number of slots between PDSCH and corresponding HARQ-ACK information; 2. Table 5.2.5.1.1-3: Minimum performance for Rank 4 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; xxx; 10 / 15; MCS17; TDLA30-10; 4x6, ULA medium B; 70; xxx. 1-2; xxx; 10 / 15; MCS17; TDLC300-100; 4x6, ULA medium B; 70; xxx. 1-3; xxx; 10 / 15; MCS17; TDLA30-10; 4x6, ULA Low; 70; xxx. Table 5.2.5.1.1-4: Minimum performance for Rank 6 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 2-1; xxx; 10 / 15; MCS13; TDLA30-10; 8x6, ULA Low; 70; xxx.