3GPP TSG-RAN WG4 Meeting #115										   R4-2506482

Malta, 19th - 23th May, 2025

Agenda Item:	7.27.3.1

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for OD-SSB for network energy saving

WID/SID:        Netw_Energy_NR_enh-Core

Document for:	Discussion

# 1  Introduction

The remaining issues on On-demand SSB(OD-SSB) requirements [1] are discussed in this paper.

# 1 OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Sub-issue 1: The start point of T2

Wayforward

The start point of T2 is

- Option 1:

  - The time point of OD-SSB deactivation

- Option 2:

  - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

- Option 2a:

  - The time point of OD-SSB activation

Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs?

The T2 between multiple OD-SSB transmissions was discussed to limit the UE measurement complexity and prevent the UE from remining in FWM. Intuitively, the two fast modes should not be too close to each other. Considering that the network has no intention to trigger frequent OD-SSB transmissions from network power saving perspective, a simple and clear definition of T2 is preferred. Additionally, it is crucial to avoid dynamic changes in UE measurement behaviour, as it will require additional UE complexity.

Option 2a appears to be the simplest approach, but the problem needs to be slightly reformulated a bit as follows: After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement (based on measCycleSCell) until the T2 from the time point of OD-SSB activation.

Proposal 1: After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement (based on measCycleSCell) until the T2 from the time point of OD-SSB activation.

# OD-SSB general issues

When the OD-SSB is configured to the UE, the additional processing time to allow the UE to reschedule measurement resources was discussed in [1]. And there are two cases:

- Case 1: NW configures a single OD-SSB carrier with multiple legacy SSB carriers

- Case 2: NW configures a single OD-SSB carrier without multiple legacy SSB carriers

Case 1: NW configures a single OD-SSB carrier with multiple legacy SSB carriers

In this case, it was agreed (as copied as below) that the FMW window time is extended by one OD-SSB periodicity to allow the UE to reschedule measurement resources. Consequently, the corresponding L3 measurement delay for deactivated SCell should be extended by one OD-SSB periodicity in accordance with the FWM window length.

Sub-issue 2: CSSF Agreement - When NW configures single OD-SSB carrier with multiple legacy SSB carriers, CSSF =1 for OD-SSB layer - Extend the FMW window time by one OD-SSB periodicity - Longer measurement delay of other frequency layers with AO-SSBs are expected Agreement 2025.02: (FMW length) - The time window is defined with Tuncertain plus actual measurement duration. - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows. - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time. - Treport is the first OD-SSB based L3 measurement reporting - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow. - A detected cell: OD-SSB based measurement period - A newly detectable cell: OD-SSB based cell identification period

Proposal 2: For the case where NW configures a single OD-SSB carrier with multiple legacy SSB carriers, the L3 measurement delay for deactivated SCell should be extended by one OD-SSB periodicity in accordance with the FWM window length.

Case 2: NW configures a single OD-SSB carrier without multiple legacy SSB carriers

Our understanding of the agreement reached in the last meeting (copied as below) is that it is only applicable to Case 2, where the UE is configured with SINGLE SCell with OD-SSB. This agreement does not apply to Case 1, where the L3 measurement delay has been extended by one OD-SSB periodicity.

Issue 1-4-2: Additional Processing time for OD-SSB reception – value Agreement UE applies the additional processing timing only in deactivated SCell measurement. - Note: The additional UE processing timing is not applied to SCell activation procedure. The additional processing timing is T_processing, the value of T_processing: - Introduce different values with UE capability: [2 or 3]ms and 5ms.

Introduce s specific UE capability for this case seems unnecessary and would add extra burden to the signalling process, given that SCell activation is not a frequent event and the relaxation by one OD-SSB periodicity does not apply for this case. Therefore, we prefer not to introduce the UE capability and instead predefine a value as 5 ms.

However, if there are strong concern from companies about defining a single value of 5 ms, we can opt for 3 ms as an advanced UE capability.

Observation 1: The relaxation by one OD-SSB periodicity does not apply for the case where NW configures OD-SSB carrier with single SSB carriers. Introduce s specific UE capability would add extra burden to the signalling process.

Proposal 3: For the case where NW configures OD-SSB carrier with single SSB carriers, the additional processing timing for deactivated SCell measurement delay is defined as

- Option 1: not to introduce the UE capability and instead predefine a value as 5 ms

- Option 2: introduce different values with UE capability: 3 ms and 5ms.

# Conclusion

Proposal 1: After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement (based on measCycleSCell) until the T2 from the time point of OD-SSB activation.

Proposal 2: For the case where NW configures a single OD-SSB carrier with multiple legacy SSB carriers, the L3 measurement delay for deactivated SCell should be extended by one OD-SSB periodicity in accordance with the FWM window length.

Observation 1: The relaxation by one OD-SSB periodicity does not apply for the case where NW configures OD-SSB carrier with single SSB carriers. Introduce s specific UE capability would add extra burden to the signalling process.

Proposal 3: For the case where NW configures OD-SSB carrier with single SSB carriers, the additional processing timing for deactivated SCell measurement delay is defined as

- Option 1: not to introduce the UE capability and instead predefine a value as 5 ms

- Option 2: introduce different values with UE capability: 3 ms and 5ms.

# References

- R4-2504906, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, Apr. 2025.