3GPP TSG-RAN WG4 Meeting #115										   R4-2506484

Malta, 19th - 23th May, 2025

Agenda Item:	7.27.3.3

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for OD-SIB1

WID/SID:        Netw_Energy_NR_enh-Core

Document for:	Discussion

# 1  Introduction

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption

Way forward:

    - Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. (HW, OPPO, Apple, Nokia, Xiaomi, CATT, MTK, vivo)

    - Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. (CMCC, QC, LGE)

    - Option C: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paing time. (E///, LGE, MTK)

In [1], the RRM impact of OD-SIB1 will be discussed.

Since the UL WUS from a UE to NES cell would be transmitted for OD-SIB1, the NES cell may transmit SIB1 in response to the UL WUS (RAN1#116 agreement). This may lead to impact on paging interruption requirement, which is discussed in this paper.

# 2 Potential impact on paging interruption due to UL-WUS for OD-SIB1

During cell re-selection in IDLE mode, UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. And the interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms, as specified in 4.2.2.6 in 38.133 (as attached below).

#### 4.2.2.6	Maximum interruption in paging reception

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW.

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc.

At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms.

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell.

However, UL WUS is required for OD-SIB1 indication, as illustrated as below, and the UL WUS transmission would cause additional paging interruption, on top of the current requirement.

Figure 1: OD-SIB1 timeline

One possible approach is to extend the interruption time to include the UL WUS transmission.

Another approach is to avoid the overlapping between UL WUS and paging, because both are controllable by the network. In other words, the paging interruption is not extended, provided the UL-WUS resource is not overlapping with Paging Occasion.

Proposal 1: For paging interruption requirement in cell re-selection.

- Option 1: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition.

- Option 2: Paging interruption is not extended, provided the UL-WUS resource is not overlapping with Paging Occasion.

# Conclusion

Proposal 1: For paging interruption requirement in cell re-selection.

- Option 1: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition.

- Option 2: Paging interruption is not extended, provided the UL-WUS resource is not overlapping with Paging Occasion.

# References

- R4-2504907, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Apr. 2025.