3GPP TSG-RAN WG4 Meeting #115											R4-2506486

Malta, 19th - 23th May, 2025

Agenda Item:	7.32.4

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for IoT NTN phase 3

WID/SID:        IoT_NTN_Ph3-Core

Document for:	Discussion

# 1  Introduction

This paper presents our view on the procedural requirements for CB-Msg3-EDT.

# 2  Requirement for CB-msg3-EDT

The procedural requirements for CB-MSG3 transmission was agreed to be introduced as the following subclauses in WF [1].

Issue 1-1: Subclauses in Random Access requirement for CB-Msg3 Agreement: - Include the following subclauses in Random Access requirement for CB-Msg3 in IoT NTN - Correct behavior when transmitting CB-Msg3. - Correct behavior when receiving CB-Msg4 - Correct behavior when not receiving CB-Msg4 - The exact titles of the subclauses can be further updated to align with RAN2 specifications.

Besides, the relevant agreement in R2#129bis [2] are copied below:

Agreements;

…..

9.	RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

Agreements (part3):

…

6.	The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI)

7.	Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

8.	The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4.

9.	Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached.

10.	Backoff information could be included in CB-Msg4.

11.	L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported.

12.	HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

And the open issues are summarized in [4], with few relevant issues listed below:

Open issue MAC-6:  The power ramping parameters and how the power ramping is done.

Open issue MAC-7:  whether the HARQ operation is applicable to transmit CB-Msg3

Open issue MAC-11:  Whether L2 ACK is supported.

Open issue MAC-13:  FFS the Msg4 structure including Contention Resolution Identities, C-RNTIs, Backoff parameters, TA information in the CB-Msg4.

Open issue MAC-14:  FFS for the detail of HARQ operation on Msg4.

Open issue MAC-15:  Whether the UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails.

Open issue MAC-16:  whether UE will be in the next CE level when max re-attempt number has been reached.

….

Additionally, the RAN2 MAC Running CR for CB-Msg3-EDT [3] is included in the Appendix for reference. The procedural requirements for CB-Msg3-EDT should be extracted and aligned with RAN2 MAC spec.

Proposal 1: The procedural requirements for CB-Msg3-EDT should be extracted and aligned with RAN2 MAC spec.

Finally, the TP for “Correct behavior when transmitting CB-msg3-EDT” is provided in Appendix as a reference, based on PUR requirement outlined in section 4.6A.3 of TS 36.133.

# Summary

The observations and proposals are summarized as the following:

Proposal 1: The procedural requirements for CB-Msg3-EDT should be extracted and aligned with RAN2 MAC spec.

# References

- R4-2502606, WF on RRM requirements for IoT_NTN_Ph3, Apr. 2025.

- R2-2502983, Report from Break-out session on NR-NTN and IoT-NTN, Apr. 2025.

- R2-2504525, MAC Running CR for Rel-19 IoT NTN, Apr. 2025.

- R2-2504526, Remaining MAC open issues in IoT NTN, Apr. 2025.

# Appendix: MAC Running CR for CB-Msg3-EDT

### 5.1x CB-Msg3-EDT Procedure

#### 5.1x.1	CB-Msg3-EDT initialization

The CB-Msg3-EDT procedure described in this clause is initiated by the RRC sublayer and can only be performed in a non-terrestrial network. If the UE is an NB-IoT UE, the CB-Msg3-EDT procedure is performed on the anchor carrier or one of the non-anchor carriers for which CB-Msg3-EDT resource has been configured in system information.

The following information is assumed to be available before the procedure can be initiated for NB-IoT UEs, BL UEs or UEs in CE mode A, as specified in TS 36.331 [8]:

-	if the UE is a BL UE or a UE in CE mode A:

-	the available set of PUSCH resources associated with each enhanced coverage level for the transmission of the CB-Msg3, [FFS cb-Msg3-EDT-PUSCH-Config].

-	if the UE is an NB-IoT UE:

-	the available sets of PUSCH resources on the anchor carrier, [FFS cb-Msg3-EDT-StartTimeParameters] and on the non-anchor carriers, in [FFS ul-ConfigList].

Editor’s note: FFS which IE in SIB22-NB contains the CB-Msg3-EDT configuration.

-	the mapping of the PUSCH resources into enhanced coverage levels is determined according to the following:

-	the number of enhanced coverage levels is equal to one plus the number of RSRP thresholds present in [FFS cb-Msg3-EDT-RSRP-ThresholdList].

Editor’s note: FFS how the mapping of PUSCH resources and CE levels are done for NB-IoT.

Editor’s note: FFS how the NB-IoT UE selects a carrier for CB-Msg3-EDT when multiple carriers provide PUSCH resources for the same enhanced coverage level.

-	the criteria to select PUSCH resources based on RSRP measurement per enhanced coverage level, [FFS cb-Msg3-EDT-RSRP-ThresholdList].

-	the number of replicas for CB-Msg3 transmission corresponding to the selected enhanced coverage level, [FFS cb-Msg3-EDT-NumReplicas].

-	CB-Msg3 transmission window configuration corresponding to the selected enhanced coverage level, [FFS cb-Msg3-EDT-TransmissionWindow].

-	CB-Msg3 response window length corresponding to the selected enhanced coverage level, [FFS cb-Msg3-EDT-ResponseWindowLength].

-	the maximum number of transmission attempts per enhanced coverage level, [FFS maxNumCBMsg3AttemptCE].

-	[FFS other parameters]

Editor’s note: FFS the transmission power parameters and power ramping parameters.

The CB-Msg3-EDT procedure shall be performed as follows:

-	flush the Msg3 buffer;

-	set the CB_MSG3_TRANSMISSION_COUNTER_CE to 1;

-	if the UE is an NB-IoT UE and if the RSRP threshold of enhanced coverage level 2 configured by upper layers in [FFS cb-Msg3-EDT-RSRP-ThresholdList] and the measured RSRP is less than the RSRP threshold of enhanced coverage level 2:

-	the MAC entity considers to be in enhanced coverage level 2;

-	else if the measured RSRP is less than the RSRP threshold of enhanced coverage level 1 as configured by upper layers in [FFS cb-Msg3-EDT-RSRP-ThresholdList] then:

-	the MAC entity considers to be in enhanced coverage level 1;

-	else:

-	the MAC entity considers to be in enhanced coverage level 0;

Editor’s note: To confirm the CE level 2 and 3 are not applicable to eMTC UE operating in CE mode A.

-	set the CB-Msg3-EDT backoff parameter value to 0 ms;

-	proceed to the selection of the CB-Msg3 Resource (see clause 5.1x.2).

#### 5.1x.2	CB-Msg3 Resource selection

The CB-Msg3 Resource selection procedure shall be performed as follows:

Editor’s note: FFS whether the TBS is per CE level. If the TBS is per CE level, UE may need to cancel CB-Msg3-EDT if the size of pending UL data is more than the TBS when moving to the next CE level.

-	the UE shall select [FFS cb-Msg3-EDT-NumReplicas] UL grants for CB-Msg3 transmission as follows:

-	select the next upcoming CB-Msg3 transmission window provided by the [FFS cb-Msg3-EDT-TransmissionWindow] associated with the selected enhanced coverage level;

-	randomly select [FFS cb-Msg3-EDT-NumReplicas] PUSCH resources in the time domain within the selected CB-Msg3 transmission window from the CB-Msg3 resources provided by [FFS cb-Msg3-EDT-PUSCH-Config] associated with the selected enhanced coverage level;

-	randomly select a frequency domain PUSCH resource for each selected time domain resource provided by [FFS cb-Msg3-EDT-PUSCH-Config] associated with the selected enhanced coverage level.

-	select the [FFS cb-Msg3-EDT-responseWindowLength] associated with the selected enhanced coverage level.

-	proceed to the transmission of the CB-Msg3 (see clause 5.1x.3).

#### 5.1x.3	CB-Msg3 transmission

The CB-Msg3 transmission shall be performed as follows:

Editor’s note: FFS CB-RNTI calculation.

Editor’s note: FFS the power ramping parameters and how the power ramping is done.

-	if this is the first transmission of CB-Msg3-EDT procedure:

-	obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity and store it in the Msg3 buffer.

#### 5.1x.4	CB-Msg3 response reception

After the CB-Msg3 transmission, the MAC entity shall monitor PDCCH identified by CB-RNTI in the CB-Msg3 response window.

-	the CB-Msg3 response window starts at the subframe that contains the end of selected the CB-Msg3-EDT transmission window plus UE-eNB RTT and has length [FFS cb-Msg3-EDT-ResponseWindowLength] for the corresponding enhanced coverage level;

Editor’s note: FFS whether the timer or window is used.

Editor’s note: FFS processing time is needed.

Editor’s Note: To update when to start PDCCH monitoring after more FFS is resolved from below agreements:

- FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring.

If notification of a reception of a PDCCH transmission is received from lower layers within the CB-Msg3 response window:

-	if the MAC PDU is successfully decoded:

-	if the MAC PDU contains one or more UE Contention Resolution Identities; and

-	if there is a UE Contention Resolution Identity in the CB-Msg3 response that matches the 48 first bits of the CCCH SDU transmitted in CB-Msg3(s):

-	consider this CB-msg3 transmission successfully completed;

-	if there is no C-RNTI information in the CB-Msg3 response corresponding to the matched Contention Resolution Identity:

-	the MAC entity shall stop monitoring for CB-Msg3 Response(s).

Editor’s note: FFS whether L2 ACK is supported.

Editor’s note: To update the Msg4 structure including Contention Resolution Identities, C-RNTIs, Backoff parameters, TA information.

If none of all received CB-Msg3 responses within the CB-Msg3-EDT response window contains a Contention Resolution Identity that matchs the 48 first bits of the CCCH SDU transmitted in CB-Msg3(s).

-	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer;

-	increment CB_MSG3_TRANSMISSION_COUNTER_CE by 1;

-	if CB_MSG3_TRANSMISSION_COUNTER_CE = maxNumCBMsg3AttemptCE for the corresponding enhanced coverage level + 1:

-	reset CB_MSG3_TRANSMISSION_COUNTER_CE;

-	if the UE has reached the maximum enhanced coverage level:

-	consider the CB-Msg3-EDT procedure unsuccessfully completed.

Editor’s note: Whether the UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails.

-	else

-	consider to be in the next enhanced coverage level.

Editor’s note: FFS whether UE will be in the next CE level when max re-attempt number has been reached.

-	based on the CB-Msg3-EDT backoff parameter, select a random backoff time according to a uniform distribution between 0 and the CB-Msg3-EDT Backoff Parameter Value;

-	delay the subsequent CB-Msg3-EDT by the backoff time;

-	proceed to the selection of a CB-Msg3 Resource (see clause 5.1x.2).

Editor’s Note: To update how to do contention resolution based on below agreement:

- RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

<skip>

# Appendix: TP for correct behavior when transmitting CB-Msg3-EDT.

### 6.6A.4	Requirements for correct behavior when transmitting CB-Msg3-EDT.

#### 6.6A.4.1	Introduction

The requirements in this clause are applicable when the UE is configured with CB-Msg3-EDT as specified in [TS 36.331].

#### 6.6A.4.2	Requirements for CB-Msg3-EDT transmission

The requirements in this clause are applicable for the UE in normal coverage or in enhanced coverage.

The UE is allowed to transmit using CB-Msg3-EDT resources provided in [SIBxx] that the UE is synchronized towards the serving cell prior to transmission. The UE determines the CB-Msg3-EDT transmission occasion according to the received CB-Msg3-EDT configuration [TS 36.331].

The UE shall determine the uplink timing,  TTA, for transmitting on CB-Msg3-EDT using the valid values of NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE where TTA is defined in [16] which are the latest acquired values before CB-Msg3-EDT transmission.

…

#### 6.6A.4.3 Correct behavior when receiving CB-Msg4

TBA

#### 6.6A.4.4 Correct behavior when not receiving CB-Msg4

TBA