3GPP TSG-RAN WG4 Meeting # 115	                                      R4-2506500

Malta, MT, May 19 – 23, 2025

Agenda Item: 7.29.3.1

Source: MediaTek Inc.

Title: Discussion on event triggered L1 report for LTM

Document for: Discussion

- Introduction

RAN#105 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. According to WID, RAN4 is supposed to define requirements for the following measurement enhancements:

- Event triggered L1 report,

- CSI-RS L1 measurements,

- CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch.

Last meeting, there were some discussions [2]. In the next, we would like to provide our views on the open issues of event triggered L1 report.

- Discussion

2.1 Capabilities for Support of Event Triggering L1 Reporting

Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Yes (Xiaomi, CMCC, ZTE, CTC, vivo) - Option 2: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple) Issue 2-4: whether to define a separate capability for supported number of events triggered L1 measurement reporting Candidate options: - Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. (Xiaomi, CMCC, ZTE, E///, CTC) - Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///) - Option 2: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO) Issue 2-5: whether and how to coordinate with R19 MIMO Agreement: - RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

In our understanding, the goal of event triggered L1 measurement reporting is to replace L3 report and used to trigger LTM. With this understanding, for the frequency layers configured with event triggered L1 measurement reporting, it is not necessary to have event triggered L3 measurement reporting.

But on the other hand, NW may configure and adjust event triggered L1 measurement reporting based on L3 report. Then there will be both L3 event and L1 event for the same frequency layer. We think it is reasonable to introduce extra UE capability to support event triggered L1 measurement reporting.

Another issue is to update the table defined in 38.133 or introduce a reported UE capability. We prefer the latter given that most of L1 measurement related capability are reported by UE capability but not defined fixed in the spec.

Proposal 1: Introduce reported UE capabilities for supported number of events triggered L1 measurement reporting. FFS the candidate values and the details.

2.2 Impact on other LTM requirements

Agreement: - For FR1: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, - Case 1: leaving condition of the event is configured and corresponding condition is not met, - the related cell switch delay and TCI state activation requirements are applicable - Case 2: leaving condition of the event is NOT configured, FFS - For FR2: FFS

In the past two meetings, RAN4 discussed the impact of event-triggered L1 report on other LTM requirements and reached the above agreement.

From UE’s point of view, as long as the channel condition is not bad and UE can keep measuring the cell or beam, the cell and TCI state can be known to UE. For NW controlled LTM, it is also important to let NW know whether the cell/TCI state is known. The way to let NW know is through reporting or not reporting (leaving condition is configured and corresponding condition is not met). For Case 2, we don’t know how to let NW know whether the cell/TCI state is known or not. So, we prefer not to do further optimization for the case that leaving condition of the event is NOT configured.

In FR2, when NW activates some TCI state(s), UE will prioritize L1-RSRP measurement on those TCI state(s) and may never measure the beam which is not activated. So UE may stop measuring the beam if NW activates some other TCI state(s) after event-triggered L1 report. In this case, the above optimization seems not workable.

Proposal 2: For FR1, if UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, leaving condition of the event is configured and corresponding condition is not met, the related cell switch delay and TCI state activation requirements are applicable. Not to extend the optimization to other cases.

- Summary

In this paper, we provide some views on event triggered L1 report in R19 mobility. We have the following proposals:

Proposal 1: Introduce reported UE capabilities for supported number of events triggered L1 measurement reporting. FFS the candidate values and the details.

Proposal 2: For FR1, if UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, leaving condition of the event is configured and corresponding condition is not met, the related cell switch delay and TCI state activation requirements are applicable. Not to extend the optimization to other cases.

4 Reference

RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#114bis.