3GPP TSG-RAN WG4 Meeting # 115	                                                           R4-2506503

Malta, MT, May 19 – 23, 2025

Agenda Item:      7.29.3.3

Source:	MediaTek Inc.

Title:	Discussion on conditional LTM

Document for:	Discussion

- Introduction

In this contribution, we provide our views on the left issues of conditional LTM.

- Discussion

2.1 Conditional cell switch delay and interruption

Last meeting, RAN4 reached the following progress for conditional cell switch delay and interruption requirements.

Issue 1-2-3: The components of conditional LTM delay Issue 1-2-4: Tinterrupt < Agreement > - Keep either TCLTM_execution or TCLTM-RRC-processing as one component for the conditional LTM delay. - Note: The legacy value(s) in previous releases will be reused. - TCLTM_execution or TCLTM-RRC-processing is not included in Tinterrupt. - Not include the components for TCI state activation and/or PDCCH order PRACH in C-LTM delay. - TEvent_DU can be updated based on RAN2 progress if needed. - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell. - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered. - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO. - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19. < Way forward > FFS - Tfirst-RS and TRS-proc are included in Tinterrupt - Reuse the values in Rel-18 LTM, and further discuss whether the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can be reused or not.

The most controversial part is whether Tfirst-RS or part of Tfirst-RS can be excluded from Tinterrupt. In our view, it is workable. If UE supports early T/F tracking, it is also possible to perform T/F tracking before Tprocessing. In addition, the concept has been adopted in NTN for soft satellite switch with re-sync as specified in 6.1C.3.2.3 in 38.133.

Observation 1: It is workable for UE to perform T/F tracking before Tprocessing.

We also want to point out that the above optimization is only applicable when T/F tracking is still needed during conditional cell switch. If T/F tracking can be skipped, we don’t need the above optimization.

Last meeting, it was questioned whether the optimization is only for some corner cases since T/F tracking may be skipped. We would like to show that it is not a corner case in the next.

As well known, to skip T/F tracking, there are some conditions. In R18 LTM, if the time gap between UE activates TCI state and receives cell switch command is less than 160ms, then UE can skip T/F tracking during cell switch. As shown below, in R18 LTM, it is easy to make sure the time gap between UE activates TCI state and receives cell switch command is less than 160ms, as both TCI state activation and cell switch are under NW control. But here in conditional LTM, as NW don’t know when conditional cell switch will be triggered, it is quite difficult to guarantee the 160ms condition.

Observation 2: It is not easy for NW to guarantee that T/F tracking can be skipped during conditional cell switch.

In the next, we would like to discuss how the requirements can be. As shown in Fig. 1 and Fig. 2, the time introduced by T/F tracking (Tfirst-RS + TRS-proc) includes two parts: the time spent on waiting for the SSB occasion “Twait_RS” and the time to do T/F tracking on SSB “TRS + TRS-proc”. Theoretically, TRS is the time needed to get ready to receive SSB on target cell plus the SSB duration. For SSB to be measured within MG, RF retuning time is needed. The SSB duration is less than 1 slot. Most of the components in HO/cell switch delay are defined in the unit of ms. We understand that the time needed for intra-frequency case can be shorter. To make the spec simpler, we prefer to define a unified value of TRS for all the scenarios. Here we propose TRS as 2ms. TRS-proc is time for SSB post-processing which can be up to 2 ms.

Here we include “TRS + TRS-proc” into interruption time due to there will be interruption on source cell to receive SSB on target cell, e.g.,

- Inter-f: UE needs to retune to another frequency

- Intra-f in FR2: Rx beam may be different from source cell

- Intra-f in FR1 with mixed numerology and UE incapable of mixed numerology

As the optimization of UE performing T/F tracking before Tprocessing has reduced the average interruption time significantly, we prefer to define a unified requirements for all the scenarios for the sake of spec simplification and readability.

Fig.1

Fig. 2

Proposal 1: For C-LTM, separate Tfirst-RS to two parts:

- Twait_RS: the time spent on waiting for the SSB occasion

- TRS: the time needed to get ready to receive SSB on target cell plus the SSB duration, which is 2 ms.

Proposal 2: Exclude Twait_RS the time spent on waiting for the SSB occasion from conditional cell switch interruption.

2.2 UE based TA measurement

Last meeting, RAN4 discussed whether to define requirements for UE based TA measurement. The mechanism is to derive TA of neighbor cell based on TA of serving cell and RTD between serving cell and neighbor cell as shown in Fig.3. Considering that serving cell and neighbor cell are not ideally synchronized in the field, there will be some synchronization error (denoted as y). Then TA2=TA1+2*RTD = TA1+2*(y+TP2-TP1).

Fig. 3

The errors, for each component should be considered, i.e.,

- TA1

Consider the errors: UL to DL error + TA resolution error + TA adjustment accuracy

- TP2:

Consider DL timing error

- TP1:

Consider DL timing error

The total error is UL to DL error + TA resolution error + TA adjustment accuracy + 2*(y+2* DL timing error) = (UL to DL error +DL timing error) + TA resolution error + TA adjustment accuracy +2y +3* DL timing error = Te + TA resolution error + TA adjustment accuracy +2y +3* DL timing error. To make sure the degradation at gNB reception is acceptable, the total error should be less than 1/2 CP, which is used as a criterion to define UE based TA measurement accuracy requirements in FR2 HST. Then derive the synchronous requirement on serving cell and neighbor cell under different numerologies as below:

| SCS of SSB (kHz) | SCS of UL signals (kHz) | 1/2 CP of UL signals (64*Tc) | Te | TA resolution error 16*64*Tc/2u | TA adjustment accuracy 38.133 Table 7.3.2.2-1 | DL timing error 1/(2*BW) | y (ns) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 15 | 71.05 | 12*64*Tc | 16*64*Tc | 256 Tc | 139ns | 427.1 |
| 30 | 30 | 35.55 | 8*64*Tc | 8*64*Tc | 256 Tc | 69.4ns | 149.0 |
| 120 | 120 | 8.9 | 3.5*64*Tc | 2*64*Tc | 32 Tc | 17.4ns | 21.1 |

After calculation, it can be observed that only when the synchronization error between serving cell and target cell is less than 427.1ns for 15kHz SCS, 149ns for 30kHz SCS, and 21.1ns for 120kHz SCS, it is possible that UL signal on target cell after adjusting through UE based TA can be within 1/2 CP at gNB.

Observation 3: Only when the synchronization error between serving cell and target cell is less than 427.1ns for 15kHz SCS, 149ns for 30kHz SCS, and 21.1ns for 120kHz SCS, it is possible that UL signal on target cell after adjusting through UE based TA can be within 1/2 CP at gNB.

As the condition to use UE based TA measurement is too tight for almost all the cases, we prefer not to define the related requirements.

In addition, we don’t think the accuracy requirements defined for one shot large UL timing adjustment for FR2 HST can be applied directly, where at least two shots of T/F tracking or TRS based T/F tracking on the target TRP is needed. If to define the requirements, at least the following should be discussed:

- accuracy requirements of the estimated TA at UE side

- synchronization requirements between source cell and target cell

- Timeline to perform UE based TA measurement

  - We don’t think UE can start to perform UE based TA measurement at RRC configuration just following the signalling designed in R18 LTM to meet acceptable accuracy.

- The impact on cell switch delay requirements

  - Similar as the discussion on the conditions T/F tracking can be skipped during cell switch delay, the condition that TA measured at UE is valid should be discussed. If the measured TA is not valid, UE behaviour should be discussed further, e.g., measure TA during cell switch delay or fall back to RACH-based cell switch.

Considering the above work to do and there are only two meetings left, we propose not to define the requirements for UE based TA measurement.

Proposal 3: Not to define the requirements for UE based TA measurement in RAN4 in R19.

- Conclusion

In this contribution, we provide our views for conditional LTM and have the following proposals and observation.

Observation 1: It is workable for UE to perform T/F tracking before Tprocessing.

Observation 2: It is not easy for NW to guarantee that T/F tracking can be skipped during conditional cell switch.

Proposal 1: For C-LTM, separate Tfirst-RS to two parts:

- Twait_RS: the time spent on waiting for the SSB occasion

- TRS: the time needed to get ready to receive SSB on target cell plus the SSB duration, which is 2 ms.

Proposal 2: Exclude Twait_RS the time spent on waiting for the SSB occasion from conditional cell switch interruption.

Observation 3: Only when the synchronization error between serving cell and target cell is less than 427.1ns for 15kHz SCS, 149ns for 30kHz SCS, and 21.1ns for 120kHz SCS, it is possible that UL signal on target cell after adjusting through UE based TA can be within 1/2 CP at gNB.

Proposal 3: Not to define the requirements for UE based TA measurement in RAN4 in R19.