3GPP TSG-RAN WG4 Meeting # 115	                                                         R4-2506506

Malta, MT, May 19 – 23, 2025

Agenda item:	7.21.4

Source:	MediaTek Inc.

Title:	Discussion on RRM core requirement and testing framework for beam management

Document for:	Discussion

- Introduction

In this contribution, we provide our views on core requirements of AI/ML BM use case.

- Discussion

## 2.1 Whether Rx beam information is known for predicted beams

In the past few meetings, RAN4 discussed whether Rx beam information is known for predicted beams. We would like to provide our view here.

Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

As captured in TR38.843 [1], in R18 SI, both DL Tx beam prediction and beam pair prediction are studied.

TR38.843 For both sub-use cases, the following alternatives are studied for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction (deprioritized) - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) Using BM-case1 as an example:

In R19 WI, as pointed out in WID, only DL Tx beam prediction is in scope and beam pair prediction is not in the scope.

RP-243244 - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

Observation 1: Both DL Tx beam prediction and beam pair prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI.

In the next, we will focus on DL Tx beam prediction at first.

Take BM-case 1 as an example. Two alternatives had been studied:

- Alt. i): Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams.

- Alt. ii): Set B is a subset of Set A.

In our understanding, there are mainly two sub-use cases

  - Use part of SSB beams to predict all the SSB beams

  - Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off

In Alt. i), if CSI-RS to be predicted is QCL Type-D to SSB in Set B, then Rx beam of the associated known SSB can be applied to CSI-RS Tx beam. In this scenario, Rx beam of the predicted Tx beam can be known.

In Alt. ii), if the case is to use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off, the predicted CSI-RS beam is supposed to be QCL Type-D to a SSB beam. If Rx beam of the QCL-ed SSB is known, then Rx beam of the predicted CSI-RS beam is also known.

If the case is to use part of SSB beams to predict all the SSB beams, in R18 SI, the following options were evaluated for DL Tx beam prediction. It is possible to use a fixed Rx beam for all the beams at inference and Rx beam of the predicted beams can be assumed to be the fixed Rx beam. Also as argued last meeting, it is UE implementation whether to use a fixed Rx beam for inference or use Rx beam related information as model input.

TR38.843 For the purpose of DL Tx beam prediction evaluations, consider the following options for Rx beam as AI/ML model input for training and/or inference if applicable: - Option 1: Measurements of the "best" Rx beam with exhaustive beam sweeping for each model input sample. - Companies expected to report how to select the "best" Rx beam(s). - Option 2: Measurements of specific Rx beam(s). - Companies expected to report how to select specific Rx beam(s). - Option 3: Measurements of random Rx beam(s) per model input sample. - Option 4: Measurements of quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the "best" Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping. - Identify the quasi-optimal Rx beams to be utilized for measuring Set B/Set C based on the previous measurements. Companies can report the time information and beam type (e.g., whether the same Tx beam(s) in Set B) of the reference signal to use. Companies expected to report the measurement/RS overhead together with the beam prediction accuracy, as well as, how to find the quasi-optimal Rx beam with "previous measurement". - Other options are not precluded and can be reported by companies.

Proposal 1: Whether Rx beam of the predicted Tx beam which is not measured can be known or not depends on different cases.

- Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use known SSB beams to predict CSI-RS beams:

  - if the predicted Tx beam which is not measured is QCL Type-D to a known Tx beam in Set B, the corresponding Rx beam is known.

- Set B is a subset of Set A.

  - Use part of SSB beams to predict all the SSB beams: Whether Rx beam of the predicted Tx beam which is not measured can be known can be up to UE capability.

  - Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off: If Rx beam of the QCL-ed SSB is known, then Rx beam of the predicted CSI-RS beam is also known.

      - 2.2 Scenarios to define requirements for?

We had done some simulations with different Set B patterns when Set B is a subset of Set A. In the simulation, there are 32 Tx beams with 2 elevation angles and 16 azimuth angles in Set A and 8 Tx beams in Set B. We tried different set B patterns (e.g., in pattern 0, Set B = the first 8 beams in Set A) and got the Top 5/1 prediction accuracy requirements as in Fig. 1. It can be observed that the top 5/1 prediction accuracy of different patterns are varied. The gap is more than 10% between the best and worst.

(a)                                                                (b)

Fig. 1. Prediction accuracy for different Set B patterns. (a) the performance, (b) the patterns

Observation 2: In terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference.

Usually, when we define accuracy requirements in RAN4, the requirements defined is scenario agnostic. Take L1-RSRP measurement accuracy as an example. The requirements are applicable to all the channel models, all UE speeds, all BS radiation patterns and etc. But for AI/ML BM models, it seems not possible to define a scenario-agnostic performance requirements based on the above observations. In R18 MIMO, we faced similar issue when discussing how to define requirements for TDCP. At that time, due to the requirements vary with different UE speeds, different serving frequency as well as different channels, RAN4 decided to define the test cases for limited cases (i.e., under certain doppler shift and channel model) instead of defining scenario-agnostic accuracy requirements. So we propose to discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements.

Proposal 2: Further study and discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements.

As mentioned above, for each BM sub-case, there are different scenarios. If it is not possible to define scenario-agnostic performance requirements, we shall discuss which scenarios to define requirements and test cases for. Take BM case 1 as an example. There are several candidate scenarios:

- Scenario 1: Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams.

  - Scenario 1a: AI model inputs are the RSRP with best Rx beam for each Tx beam in set B

  - Scenario 1b: AI model inputs are the RSRP with specific Rx beam for each Tx beam in set B

- Scenario 2: Set B is a subset of Set A.

  - Scenario 2-1: use part of SSB beams to predict all the SSB beams.

    - Scenario 2a-1: AI model inputs are the RSRP with best Rx beam for each Tx beam in set B

    - Scenario 2b-1: AI model inputs are the RSRP with specific Rx beam for each Tx beam in set B

  - Scenario 2-2: Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off

According to the simulation assumptions agreed for AI/ML BM in R4-2505152, only scenario 2a-1 and 2-2 are included. Although we think scenario 1 is the most reasonable and promising scenario, to make the best use of the simulation efforts, we propose to only define the requirements for 2a-1 or 2a-2.

Also it is up to UE implementation whether to use beam pair prediction models. However, as beam pair prediction is not in the WI scope, we propose not to define separate requirements or test cases for beam pair prediction.

Proposal 3: Further discuss which scenario among the following to define requirements and test cases for:

- Scenario 1: Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams.

  - Scenario 1a: AI model inputs are the RSRP with best Rx beam for each Tx beam in set B

  - Scenario 1b: AI model inputs are the RSRP with specific Rx beam for each Tx beam in set B

- Scenario 2: Set B is a subset of Set A

  - Scenario 2-1: use part of SSB beams to predict all the SSB beams.

    - Scenario 2a-1: AI model inputs are the RSRP with best Rx beam for each Tx beam in set B

    - Scenario 2b-1: AI model inputs are the RSRP with specific Rx beam for each Tx beam in set B

  - Scenario 2-2: Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off

Proposal 4: Only define the accuracy requirements and test cases for one of the following two scenarios:

  - Scenario 2a-1: use part of SSB beams to predict all the SSB beams

    - AI model inputs are the RSRP with best Rx beam for each Tx beam in set B

  - Scenario 2-2: Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off

      - 2.3 Impact of measurement error

2.3.1 General analysis

Last meeting, the simulation settings were further discussed to evaluate the impact of measurement error in R4-2505152. In our simulations, Option 2 in R4-2505152 is adopted to get the distribution of BB error under different SNRs, i.e., TDL-C channel and single Tx beam. For a single SNR, Tx power is set as the normalized power 1 and noise is generated following Gaussian distribution. After simulations, the average and variance of BB error under a fixed SNR can be obtained. Following above procedures, we get the mean and variance of BB measurement error under SNR= [-24, 10] dB with step 3dB at first (as in Fig. 2).

Fig.2

During inference, the way to add measurement error to the inputs of model inference is shown in Fig. 3:

- In SLS simulation, SINR of each Tx-Rx beam pair is calculated dependently.

  - In the calculation of SINR, noise is generated as Gaussian distribution and UE receiver noise figure is set to 10dB which is as guided in TR38.843 Table 6.3.1-1. Useful signal and interferences from other cells are calculated considering Tx power, path-loss, channel gain and beamforming gains.

- After calculating SINR of each beam pair, we look up the table to get the corresponding variance.

  - If the SINR is not an integer or out of the range [-24:3:9] dB, we use interpolation method to obtain the mean and variance.

- A random BB error is generated following Gaussian distribution with the mean and variance corresponding to the SINR value.

Fig. 3 Method of adding measurement error

2.3.2 Simulation results

As guided in R4-2505152, we focus on the scenario with 100% outdoor UEs to check the performance of different metrics. The simulation results are summarized as in Table 3. To check the predicted RSRP accuracy, regression model is adopted in the simulations.

Table 1

| error consideration |  |  | case 1 | case 2a |  |  | case 2b |  |  | case 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KPI | K | X | N.A. | BB error only | RF error only | BB+RF error | BB error only | RF error only | BB+RF error | BB error only | RF error only | BB+RF error |
| KPI 1 | 1 | 0 | 68 | 64 | 61 | 58 | 64 | 51 | 50 | 65 | 52 | 51 |
|  | 2 | 0 | 84 | 81 | 79 | 78 | 81 | 73 | 71 | 83 | 73 | 72 |
|  | 3 | 0 | 91 | 89 | 87 | 86 | 89 | 83 | 81 | 90 | 84 | 82 |
|  | 4 | 0 | 94 | 92 | 91 | 90 | 92 | 89 | 87 | 93 | 90 | 88 |
|  | 5 | 0 | 95 | 95 | 94 | 93 | 95 | 92 | 91 | 95 | 92 | 92 |
|  | 1 | 1 | 75 | 71 | 68 | 66 | 71 | 59 | 57 | 73 | 59 | 58 |
|  | 1 | 2 | 81 | 78 | 76 | 73 | 78 | 67 | 65 | 80 | 67 | 65 |
|  | 1 | 3 | 86 | 83 | 81 | 79 | 83 | 73 | 71 | 85 | 74 | 72 |
| KPI 2 | 1 | N.A. | 4.9 | 6.1 | 6.2 | 7.6 | 5.7 | 7.2 | 7.7 | 5.7 | 7.1 | 7.5 |
|  | 3 |  | 5.4 | 7.8 | 6.4 | 8.3 | 6.7 | 7.2 | 8.2 | 6 | 7.3 | 7.8 |
|  | 5 |  | 5 | 8.5 | 6.4 | 9.3 | 6.6 | 7 | 8.4 | 6.2 | 7.2 | 8 |
| KPI 1: Beam prediction accuracy - The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 1, 2, 3dB KPI 2: Absolute RSRP accuracy - 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ground truth L1-RSRP of the same beams Note: For KPI 2, the statistics is carried out for the single predicted Kth strongest beam, rather than all beams within top K. |  |  |  |  |  |  |  |  |  |  |  |  |

It can be observed that measurement error has big impact on the performance. Both BB error and RF error should be considered when define the requirements and test case.

Proposal 5: Both BB error and RF error should be considered when define the requirements and test case.

Comparing the simulation results for case 2b (training without measurement error) and case 3 (training with measurement error), it can be observed that there is little impact on either Top K prediction accuracy or absolute RSRP accuracy training with measurement error or not.

Observation 3: There is little impact on either Top K prediction accuracy or absolute RSRP accuracy training with measurement error or not.

Comparing the simulation results for case 2a (Error will be considered in model input during inference) and case 2b (Error will be considered in model input during inference and ground-truth for inference), it can be observed that Top K prediction accuracy of case 2a is a bit high and absolute RSRP accuracy of the 1st and 3rd predicted beams are similar. If to use multiple AoA test systems, it is questionable to get the genie-aided RSRP, so the feasibility of case 2a is TBD. If to use single AoA test systems, it is possible to get the genie-aided RSRP, both case 2a and 2b can work.

Observation 4: Comparing to the genie-aided ground-truth or ground-truth with measurement error, Top K prediction accuracy is a bit different and absolute RSRP accuracy of the 1st and 3rd predicted beams are similar.

      - 2.4 Metrics

Last meeting, RAN4 agreed to use Top-K plus xdB margin as the metric for beam ID prediction. The value of K and x is supposed to be discussed further.

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%): the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Meanwhile, RAN1#120bis agreed to use Top-K/M for Type 1 option 2 monitoring.

Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling

As shown above, the metric agreed in RAN4 is not totally aligned with the metric agreed for monitoring in RAN1. One of the questions is whether to align with the metric for monitoring. We prefer to have aligned metric as monitoring, then the defined accuracy requirements and test cases can be used as a reference for NW to decide whether the model is working well. Otherwise, the to-be-defined accuracy requirements and test cases can only be used to test the performance of model. Given that there is slightly difference between the metrics agreed in RAN4 and RAN1. We prefer the defined accuracy requirements and test cases can be used as a reference for NW to decide whether the model is working well, i.e., the metric is also for monitoring. As RAN1 has concluded the metric for monitoring, we propose to align the metric discussed in RAN4 with RAN1.

In our understanding, RAN4 is supposed to conclude whether x and N (in last meeting’s agreement) are needed. The exact value can be further discussed during performance part.

Proposal 6: Prefer that the metric to be used in RAN4 is also used for monitoring, i.e., x shall be 0.

Last meeting, RAN4 further discussed the metrics “absolute RSRP accuracy and relative RSRP accuracy”. The main divergence lies in whether absolute RSRP accuracy is applicable to none Top-1 beams.

Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

From the point of necessity, we think the absolute RSRP accuracy requirements should be defined for none Top-1 beams too. Suppose the absolute RSRP accuracy requirement is 7dB, and the relative RSRP accuracy requirement is also 7dB. Assuming the predicted RSRP of the top 1 beam is -83dBm, to meet the relative RSRP accuracy requirement, the predicted RSRP of beam Y should be within [-98,-84]dBm. It means if the predicted RSRP of beam Y is -84dBm, which is 14dB high than the genie-aided RSRP, the model can pass the test. We don’t think it is reasonable.

| Beams | Genie-aided RSRP | Predicted RSRP |
| --- | --- | --- |
| Predicted Top 1 | -90dBm | -83dBm |
| Another beam: beam Y | -98dBm | [-98, -84] dBm |

Proposal 7: From the point of necessity, the absolute RSRP accuracy requirements should be defined for none Top-1 beams too.

But we totally understand other companies’ concern. The legacy L1 measurement requirements are defined at SNR= -3dB and the requirements are applicable under the side condition that SNR≥-3dB. In legacy L1-RSRP related tests, we can make sure SNR≥-3dB. For AI BM, regarding how to define the RSRP accuracy requirements, it seems there are two directions based on previous offline discussions. One is to define the RSRP accuracy requirements similar as legacy, i.e., the requirements are defined at SNR= -3dB and the requirements are applicable under the side condition that SNR≥-3dB. Another is filtering out the data with SNR of the strongest beam in set A no less than -3dB. We checked SNR distribution of the strongest beam in set A and found that there is no data with SNR of the strongest beam in set A equal to -3dB in our generated dataset as shown in Fig.4. We once tried to decrease the RSRP artificially to make SNR of the strongest beam in set A equal to -3dB. But we received some offline comments that this may lead to a different scenario and is not a good way. Considering the above fact, we think the RSRP accuracy requirements for the strongest beam can be defined by filtering out the data with SNR of the strongest beam in set A no less than -3dB. Even though, we may face an issue that SNR distribution of the top 1 beam in set A would be varied among different companies’ datasets, and also may be different from the final test dataset generated by TE vendors. How to handle this issue shall be further discussed.

Fig. 4

Proposal 8: RSRP accuracy requirements of Top-1 of predicted beams can be defined by filtering out the dataset with SNR of the Top-1 beam in set A no less than -3dB. Further discuss how to deal with the issue that the distribution of SNR of the Top-1 beam in set A vary among different companies’ dataset as well as the test dataset generated by TE vendors.

For none Top-1 beams, it can be observed that the absolute accuracy of each predicted beam varies as shown in Table 1 (KPI 2). We checked the CDF of the RSRP/SINR difference between the (x+1)th strongest beam and the strongest beam (E.g., 1th gap: strongest beam - 2nd strongest beam, 3th gap: strongest beam - 4th strongest beam) in Fig. 5. As shown by figure on the right in Fig. 5, there is a 60% probability that SINR of the 2nd strongest beam is lower than the strongest beam, i.e., SINR difference >0dB. Such a SINR difference will lead to the difference in predicted absolute RSRP accuracy of top-1 beam and other beams.

Observation 5: For different beams, e.g., predicted 1st strongest and 5th strongest beam, the predicted absolute RSRP accuracy varies. It may be the SINR difference that results in the difference between the predicted absolute RSRP accuracy of top-1 beam and other beams.

Fig. 5

Due to limited time, we don’t have time to check the predicted absolute RSRP accuracy of none top-1 beams if excluding those datasets with low SNR. We can further check it to see whether it is workable to define a single absolute accuracy requirement which is applicable to all/some of the predicted beams with some side conditions, e.g., SNR≥-3dB.

Proposal 9: Further discuss whether it is workable to define a single absolute accuracy requirement which is applicable to all/some of the predicted beams with some side conditions.

      - 2.5 Test setup for AI/ML BM

2.5.1 Testing goal

In R18 SI, the following principle was identified: The design of test should ensure performance is guaranteed and avoid that a UE can pass the test but perform poorly in the field (in TR38.843).

In General part, RAN4 discussed the testing goals of this WI and reached the following agreement in RAN4#112.

Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

As observed in the simulation results in Fig.1, in terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference. RAN4 once agreed to use synthetic channels in R19 AI/ML test. Now we wonder whether the simulation results under synthetic channels can reflect the performance of AI/ML models in the real field. If not, we think it may be necessary to design a test system that is possible to use field data in the test. The key issue to use field data in the test is how to get the test dataset. We understand that it is not easy to figure this out.

Considering the potential fact that the performance under synthetic channels may not reflect the performance of AI/ML models in the real field and the difficulty to use field data in the test, we think at least for AI/ML BM, RAN4 needs to discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

If the goal is to test UE can train a model that works well under a specific scenario, we prefer to choose a test system that is easy to implement and evaluate.

Proposal 10: For AI/ML BM, further discuss whether the performance under synthetic channels can reflect the performance of AI/ML models in the real field.

Proposal 11: For AI/ML BM, discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

Proposal 12: If the testing goal is to verify the model can work well in real field, and the performance under a specific channel/scenario cannot reflect the performance in the real field, it is necessary to design a test system that is possible to use field data in the test.

Proposal 13: If the testing goal is to test UE can train a model that works well under a specific scenario, prefer to choose a test system that is easy to implement and evaluate.

2.5.2 Test set up

2.5.2.1 Single AoA test system

For DL Tx beam prediction, the model inputs are L1-RSRP of Tx beams in Set B with Tx beam ID information as input or implicitly indicated, no matter RSRP of Tx beams in Set B is the one obtained from exhaustive sweeping or a specific fixed Rx beam. It is only the final L1-RSRP value that matters. All the spatial characteristics will be reflected on RSRP. Instead of emulating CDL channels and [32-128] Tx beams through multiple AoAs and AoDs, the spatial-selective and time-varying propagation conditions can be finally reflected on received RSRP at the test UE through adjusting Tx power at TE on each beam at different time. This is also the method which is widely used in legacy RAN4 RRM tests.

Proposal 14: In AI/ML BM test, emulate the spatial-selective and time-varying in propagation conditions through adjusting Tx power at TE on each beam at different time.

Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. In the next, we would like to explain that using real field data is also possible with single AoA test system.

As shown in Fig.4, in the data collected from real field, the measured RSRP (at baseband, including Rx beamforming gain) of Tx#1 and Tx#2 are P1 and P2, respectively. Using single AoA test system, with Rx beam lock, the Rx beamforming gain during the test can be fixed, assuming it is G. If the transmit RSRP at TE for Tx#1 and Tx#2 is P1-G and P2-G, then the received RSRP at UE baseband will be P1 for Tx#1 and P2 for Tx#2 which are exactly the same as the measured RSRP in real field. The Rx beamforming gain G can be derived by letting TE transmits a fixed high power P0, G is equal to P0’-P0 where P0’ is the RSRP measured at UE. This can be done multiple times to derive an average G to reduce the impact of noise.

Fig. 4

If to use field data, TE transmission power for each Tx beam in set B can be determined as shown in Fig.5.

Fig. 5

If to use synthetic channels defined in 38.901, BS/UE antenna configuration & radiation pattern defined in 38.901 can be used to determine TE transmission power for each Tx beam in set B (as shown in Fig.6).

Fig. 6

In computer simulations, it is possible to get RSRP of each Tx beam under a specific gNB settings (Tx spatial radiation pattern, BS height, downtilt angle and etc.), a specific channel (e.g., UMi) and a specific UE radiation pattern (defined in 38.901) at a random UE location with random UE rotation. Though computer simulation, it is possible to get a set of RSRP values, as shown in Table 4. Take Data 1 as an example, if the transmission power at the probe is RSRP#B1-1 for Tx beam B1, the channel between the probe and UE is purely AWGN channel, and UE beamforming gain is 1, then the measured RSRP of Tx beam B1 at UE is RSRP#B1-1. As the channel between the probe and UE may be not purely AWGN channel and UE beamforming gain is not 1, the real transmission power at the TE shall be calibrated.

Table 2

| Data set | Set B | Set A |
| --- | --- | --- |
| Data 1 | RSRP#B1-1, …,RSRP#BM-1 | RSRP#A1-1, …,RSRP#AN-1 |
| Data 2 | RSRP#B1-2, …,RSRP#BM-2 | RSRP#A1-2, …,RSRP#AN-2 |
| …… |  |  |
| Data L | RSRP#B1-L, …,RSRP#BM-L | RSRP#A1-2, …,RSRP#AN-L |

As the data set can be easily obtained through simulation for varied scenarios and settings, we think single AoA test system can be easily extended to different scenarios and settings.

It is observed that one of the concerns on single AoA test system is that using UE’s radiation pattern to get the inference dataset at TE will lead to high complexity. We don’t think it is necessary to use the test UE’s radiation pattern. We evaluated the performance of models trained with a UE antenna radiation pattern (case 2) and inference on UEs with varied antenna radiation patterns (case 1, 3, 4) as shown in Fig. 7. It can be observed that AI/ML BM Tx beam prediction model generalize well for different antenna radiation patterns. Therefore, we think it is enough to use a specified UE radiation pattern during the test.

Fig. 7

Observation 6: Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. It can be easily extended to different scenarios and settings.

For set A, TE can get the generated RSRP value for each beam as shown in Table 4. Then after some calibration, we think TE can also know the expected RSRP of Tx beams at UE baseband, i.e., roughly transmitted RSRP at TE + fixed Rx beamforming gain.

Observation 7: With single AoA, TE can know the expected RSRP of Tx beams at UE baseband.

With single AoA, existing IFF and enhance IFF test systems can be re-used for AI/ML BM test.

Proposal 15: Use existing IFF or enhance IFF test systems with single AoA to test R19 DL Tx beam prediction models.

Proposal 16: Standardized UE radiation pattern in 38.901 can be assumed in R19 AI/ML BM test with single AoA test system instead of using test UE’s radiation pattern.

The procedures of AI/ML BM test with single AoA can be as following. We are open to do necessary adjustments.

Proposal 17: The procedures of AI/ML BM test with single AoA:

Beam lock at test UE during the test.

- Step 1: Overall coupling loss calibration

  - TE transmit the signal with a fixed power, UE receive signal and report RSRP to TE. TE can get the overall coupling loss through the relationship between TE’s transmit power and the reported RSRP (i.e., including the antenna gain of the probe, OTA pathloss and UE’s Rx beamforming gain). TE can ask UE to report multiple times to reduce the impact of noise in the coupling loss calculation.

- Step 2: Main test

  - TE gets the test dataset. The dataset can be early prepared through computer-like simulation using standardized UE radiation pattern in 38.901 and is the same for all the test UEs.

  - TE transmit the calibrated power on each Tx beam in set B from the test dataset by the overall coupling loss. UE measures set B and predicts Top K or RSRP of set A. In such a way, the uncertainty of beamforming gain can be removed when deciding pass/fail for each predicted RSRP.

2.5.2.2 Multiple AoAs test system

In the past few meetings, there were some proposals to further enhance multiple AoAs test system for AI/ML BM test. It seems there are two candidates of multiple AoAs test systems, eIFF FR2 RRM test system and FR2 MIMO OTA test system. As guided, companies are invited to bring analysis on the following aspects.

For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

Last meeting, there were some simplified channels proposed for eIFF in R4-2504268, as well as the simulation results comparing the performance with the non-simplified (theoretical) channels. We plan to check the performance of simplified channels too.

In the system level simulation, UEs are casted in the cell randomly. For each UE, the channel is generated according to 38.901. The instant channels for different UEs are varied. To do the system level simulation with simplified channels, for each UE, we need to simply the channel following some agreed method. So the method of how to simplify the channel should be decided at first. Then companies are able to check the performance. We have to say that the workload will be very high, as a new channel model shall be coded, and all the simulations should be re-done. If there are multiple candidate methods to simply the channel, the workload will increase linearly.

Observation 8: Simulation workload will increase linearly with the number of candidate methods to simply the channels.

Proposal 18: Make a down-selection on the candidate methods of how to simply CDL channels.

Regardless the huge work to be done to verify the performance with simplified CDL channels using multi-AoA test systems, we think some potential issues should be discussed too.

In general, the channel that the chamber simulates is link level channel, which is just one realization, i.e., for UE at a certain location. For AI/ML BM test, it is necessary to check the performance at different UE locations in a cell. Especially for AI/ML BM case 2 (temporal prediction), it is necessary to emulate UE movement which is important for BM case 2 during the test. In our understanding, there may be two methods. One is through UE rotation. But according to the information provided by TE vendors [2-5], it may be difficult and costly to support frequent and flexible UE rotation, i.e., rotation 2o, 5o. Now, the time needed to rotate UE is about 1s, which is not acceptable during one iterative test. Another method may be through adjusting TE transmission power at each probe, which is similar as single AoA test system. We are open to other methods. If not, we don’t think it is meaningful to use multi-AoA as different UE locations will be emulated through adjusting TE transmission power at each probe.

Proposal 19: Discuss how to emulate UE movement in multiple AoA test systems.

To use multiple AoAs test system, the test will include two steps. In the first step, TE generates the channel and transmits Tx beams in Set B only. UE reports prediction results. In the next step, TE tries to generate the same channel as in the first step and transmits all the Set A Tx beams. The order of the above steps can also be reversed. In the test, as the measurement results may be 10dB lower or 20dB higher the transmitted RSRP at TE, it is difficult to check whether UE reports the real measurement results of Set A or reports fake values according to the previous predicted results to pass the test. Another issue is how to ensure the channels in these two steps are the same. We observed there were proposals to make the mapping relation of RS index and beam index different in the two steps. We admit it is workable to avoid UE cheating in the test. But this may also make model monitoring fail and is not preferred.

Proposal 20: How to avoid UE cheating in the test, such as reports fake values according to the previous predicted results, shall be further discussed to use the reported RSRP measurement as ground truth.

Another issue is how to get the training dataset. In computer simulations, a pre-defined UE radiation pattern can be used to generate the training dataset and inference dataset. But in the test, we are not clear about how to get the training dataset. We think there may be two ways. One is collecting the training data in the chamber for each test UE before training. This method is time-consuming and not preferred. Another is to generate the training dataset assuming a certain UE radiation pattern. According to the simulation in Fig.9 for single AoA test system, we guess this method may work for multiple AoA test system too. But before concluding, more simulations should be performed.

Proposal 21: Discuss how to get training dataset if to use multiple AoA test systems.

2.5.2.3 Dynamic range of Single AoA and Multiple AoA test systems

In early releases, dynamic range of IFF and DFF test systems had been analysed. Since several years have passed, there may be some new progress. But we are not experts, so here we just copy the previous results in TR38.810 here for information.

Table 3

| Test methods | Wanted signal (dBm/SCS) at reference point: upper bound | SNR at baseband (dB): upper bound | SNR at baseband (dB): upper bound – 30dB |
| --- | --- | --- | --- |
| IFF, peak fine beam | -83.5 | 25.1 | -4.9 |
| IFF, peak rough beam | -83.5 | 18.1 | -11.9 |
| IFF, non-peak fine beam | -83 | 13 | -17 |
| IFF, non-peak rough beam | -83 | 6 | -24 |
| DFF, peak fine beam | -83.8 | 24.8 | -5.2 |
| DFF, peak rough beam | -83.8 | 17.8 | -12.2 |
| DFF, non-peak fine beam | -83.3 | 12.7 | -17.3 |
| DFF, non-peak rough beam | -83.3 | 5.7 | -24.3 |

Considering the upper bound of SNR with rough beam are much lower than that with fine beam as shown in Table 1, fine beam should be used in AI/ML BM test to get a larger dynamic power range. Next we will only focus testing using fine beam.

Observation 9: The upper bound of SNR with rough beam are much lower than that with fine beam.

Proposal 22: Fine beam should be used in AI/ML test.

It can be observed that the power level at reference point (before UE Rx beam forming) for different cases are basically the same as it is determined by maximum Tx power at TE.

The upper bound of SNR at UE baseband is determined by several factors. One of the factors is Tx power at TE. If TE can do better now, the upper bound of SNR can be higher.

Regarding the lower bound, in our understanding, it is not quite a limitation. As observed through the simulation results in 2.2, the weak beams should be excluded otherwise the predict accuracy of AI/ML BM model will be very poor after considering the impact of measurement error.  In our simulation, we excluded the beams which is 30dB weaker than the best beam, and the Top 5/1 prediction accuracy is above 90%. Therefore 30dB dynamic range may be enough for the test based on our initial simulations. We can get the corresponding SNR at baseband in the last column. It can be observed that SNR in the last column is above -6dB only when using peak fine beam. It is important to have a large SNR dynamic range, otherwise the measurement error will be large for many beams. If to use multiple AoA test system, it is not possible to use peak fine beam only. With single AoA, it is possible to use peak fine beam. And we propose to use peak fine beam in AI/ML BM test. It can be observed that the upper bound of SNR with non-peak fine beam is 12.1dB lower than that of peak fine beam, which means SNR dynamic range of multiple AoA test system would be at least 12dB smaller than single AoA test system.

Observation 10: Among all the test methods, only using peak fine beam direction can make sure the lowest SNR above -6dB with 30dB dynamic power range.

Observation 11: Multiple AoA test systems cannot use peak fine beam direction only.

Proposal 23: Using peak fine beam direction in AI/ML BM test with single AoA test system.

Observation 12: SNR dynamic range of multiple AoA test system would be at least 12dB narrower than single AoA test system.

- Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Observation 1: Both DL Tx beam prediction and beam pair prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI.

Proposal 1: Whether Rx beam of the predicted Tx beam which is not measured can be known or not depends on different cases.

- Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use known SSB beams to predict CSI-RS beams:

  - if the predicted Tx beam which is not measured is QCL Type-D to a known Tx beam in Set B, the corresponding Rx beam is known.

- Set B is a subset of Set A.

  - Use part of SSB beams to predict all the SSB beams: Whether Rx beam of the predicted Tx beam which is not measured can be known can be up to UE capability.

  - Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off: If Rx beam of the QCL-ed SSB is known, then Rx beam of the predicted CSI-RS beam is also known.

Observation 2: In terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference.

Proposal 2: Further study and discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements.

Proposal 3: Further discuss which scenario among the following to define requirements and test cases for:

- Scenario 1: Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams.

  - Scenario 1a: AI model inputs are the RSRP with best Rx beam for each Tx beam in set B

  - Scenario 1b: AI model inputs are the RSRP with specific Rx beam for each Tx beam in set B

- Scenario 2: Set B is a subset of Set A

  - Scenario 2-1: use part of SSB beams to predict all the SSB beams.

    - Scenario 2a-1: AI model inputs are the RSRP with best Rx beam for each Tx beam in set B

    - Scenario 2b-1: AI model inputs are the RSRP with specific Rx beam for each Tx beam in set B

  - Scenario 2-2: Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off

Proposal 4: Only define the accuracy requirements and test cases for one of the following two scenarios:

  - Scenario 2a-1: use part of SSB beams to predict all the SSB beams

    - AI model inputs are the RSRP with best Rx beam for each Tx beam in set B

  - Scenario 2-2: Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off

Proposal 5: Both BB error and RF error should be considered when define the requirements and test case.

Observation 3: There is little impact on either Top K prediction accuracy or absolute RSRP accuracy training with measurement error or not.

Observation 4: Comparing to the genie-aided ground-truth or ground-truth with measurement error, Top K prediction accuracy is a bit different and absolute RSRP accuracy of the 1st and 3rd predicted beams are similar.

Proposal 6: Prefer that the metric to be used in RAN4 is also used for monitoring, i.e., x shall be 0.

Proposal 7: From the point of necessity, the absolute RSRP accuracy requirements should be defined for none Top-1 beams too.

Proposal 8: RSRP accuracy requirements of Top-1 of predicted beams can be defined by filtering out the dataset with SNR of the Top-1 beam in set A no less than -3dB. Further discuss how to deal with the issue that the distribution of SNR of the Top-1 beam in set A vary among different companies’ dataset as well as the test dataset generated by TE vendors.

Observation 5: For different beams, e.g., predicted 1st strongest and 5th strongest beam, the predicted absolute RSRP accuracy varies. It may be the SINR difference that results in the difference between the predicted absolute RSRP accuracy of top-1 beam and other beams.

Proposal 9: Further discuss whether it is workable to define a single absolute accuracy requirement which is applicable to all/some of the predicted beams with some side conditions.

Proposal 10: For AI/ML BM, further discuss whether the performance under synthetic channels can reflect the performance of AI/ML models in the real field.

Proposal 11: For AI/ML BM, discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

Proposal 12: If the testing goal is to verify the model can work well in real field, and the performance under a specific channel/scenario cannot reflect the performance in the real field, it is necessary to design a test system that is possible to use field data in the test.

Proposal 13: If the testing goal is to test UE can train a model that works well under a specific scenario, prefer to choose a test system that is easy to implement and evaluate.

Proposal 14: In AI/ML BM test, emulate the spatial-selective and time-varying in propagation conditions through adjusting Tx power at TE on each beam at different time.

Observation 6: Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. It can be easily extended to different scenarios and settings.

Observation 7: With single AoA, TE can know the expected RSRP of Tx beams at UE baseband.

Proposal 15: Use existing IFF or enhance IFF test systems with single AoA to test R19 DL Tx beam prediction models.

Proposal 16: Standardized UE radiation pattern in 38.901 can be assumed in R19 AI/ML BM test with single AoA test system instead of using test UE’s radiation pattern.

Proposal 17: The procedures of AI/ML BM test with single AoA:

Beam lock at test UE during the test.

- Step 1: Overall coupling loss calibration

  - TE transmit the signal with a fixed power, UE receive signal and report RSRP to TE. TE can get the overall coupling loss through the relationship between TE’s transmit power and the reported RSRP (i.e., including the antenna gain of the probe, OTA pathloss and UE’s Rx beamforming gain). TE can ask UE to report multiple times to reduce the impact of noise in the coupling loss calculation.

- Step 2: Main test

  - TE gets the test dataset. The dataset can be early prepared through computer-like simulation using standardized UE radiation pattern in 38.901 and is the same for all the test UEs.

  - TE transmit the calibrated power on each Tx beam in set B from the test dataset by the overall coupling loss. UE measures set B and predicts Top K or RSRP of set A. In such a way, the uncertainty of beamforming gain can be removed when deciding pass/fail for each predicted RSRP.

Observation 8: Simulation workload will increase linearly with the number of candidate methods to simply the channels.

Proposal 18: Make a down-selection on the candidate methods of how to simply CDL channels.

Proposal 19: Discuss how to emulate UE movement in multiple AoA test systems.

Proposal 20: How to avoid UE cheating in the test, such as reports fake values according to the previous predicted results, shall be further discussed to use the reported RSRP measurement as ground truth.

Proposal 21: Discuss how to get training dataset if to use multiple AoA test systems.

Observation 9: The upper bound of SNR with rough beam are much lower than that with fine beam.

Proposal 22: Fine beam should be used in AI/ML test.

Observation 10: Among all the test methods, only using peak fine beam direction can make sure the lowest SNR above -6dB with 30dB dynamic power range.

Observation 11: Multiple AoA test systems cannot use peak fine beam direction only.

Proposal 23: Using peak fine beam direction in AI/ML BM test with single AoA test system.

Observation 12: SNR dynamic range of multiple AoA test system would be at least 12dB narrower than single AoA test system.

- Reference

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2411177, Testability and interoperability issues for beam management, Korea Testing Laboratory, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411254, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight Technologies, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411279, Study on CDL channel model for AI/ML beam management in FR2, Anritsu Corporation, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411587, Discussion on test setup for AI/ML based beam management, Rohde & Schwarz, 3GPP TSG-RAN4 # 112, Aug. 2024.