3GPP TSG-RAN WG4 Meeting # 115	                                                      R4-2506509

Malta, MT, May 19 – 23, 2025

Agenda item:	7.23.4

Source:	MediaTek Inc.

Title:	Discussion on testability and interoperability of AI mobility

Document for:	Discussion

- Introduction

RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

According to the agreed work plan last meeting, this meeting RAN4 will focus on RRM measurement prediction use-case. In this contribution, we would like to discuss the related testability issues.

- Discussion

As guided by the agreement last meeting, we would like to discuss how to model different time-varying characteristics per cell/site due to moving UE trajectories.

Issue 4-1-2: Prediction consistency in time domain Agreement: - To ensure prediction consistency in time domain, RAN4 to discuss how to model different time-varying characteristics per cell/site due to moving UE trajectories. - Discuss how to incorporate controlled randomness and the extent of time-domain variation and correlation - RAN4 to prioritize discussion on FR1.

The fact is that it is possible to model different time-varying characteristics per cell/site due to moving UE trajectories in computer simulations. RAN4 has agreed “Conductive testing” as a baseline option for testing in FR1. With “Conductive testing”, as UE is connected to the channel simulator through cable, the test would be similar as computer simulations. TE can emulate the time-domain variation and spatial-domain variation due to moving UE trajectories through adjusting the transmit power per cell/site based on a predefined fading channel.

Take scenario#3 FR1 to FR1 inter-frequency prediction as an example. There may be several cells on different frequency layers. If the model is to predict measurement results of Cell 2 and Cell 3 on different frequencies from serving cell (Cell 1), a test setup as shown in Fig.1 can be used. TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model. SSB is transmitted on cell 1 at all the SMTC occasions.  For cell 2 and cell 3, SSB will not be transmitted on the SMTC/MG occasions to be predicted. We think it is sufficient to change TE Tx power every symbol to avoid any impact on UE’s algorithms. TE vendors may need to further check whether it is possible to change Tx power every symbol.

Fig. 1

Proposal 1: Further check whether it is possible to change Tx power every symbol at TE using conductive test in FR1 for AI/ML mobility.

Proposal 2: In FR1, computer simulations alike test setup can be adopted for AI mobility test to model different time-varying characteristics per cell/site due to moving UE trajectories. The following figure can be used to determine the power transmitted at TE on each cell, i.e., TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model.

Last meeting, RAN#4 reached the following progress on the testing setup.

Issue 4-1-3: Multiple cells in the testing setup Agreement: - RAN4 to consider two cells: serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction. - FFS: RAN4 to study if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

Inter-cell case is Scenario 3 (FR1 to FR1 inter-frequency (frequency domain)). We believe it is more meaningful to consider cluster-based approach. UE is supposed to measure inter-frequency neighbor cells per frequency layer. If UE needs to measure any of the inter-frequency neighbor cells on a particular frequency layer, gap is still needed and the benefit cannot be obtained. In RAN2 study, some companies including us, use the model as shown in Fig.2: the inputs are RSRP of 21 cells (assume Cell#0,1, …,20) on frequency#1, the outputs are RSRP of 21 cells (assume Cell#0’,1’, …,20’) on frequency#2. Cell#X and Cell#X’ are co-located.

Fig. 2

For the test or defining the requirements in WI, RAN4 need to discuss the cell layout for inter-frequency prediction. To fully reuse the work that has done by RAN2, we prefer to use the same cell layout as RAN2. However, for UE at a certain location, it is not possible to detect all the 21 cells. For the purpose of defining the requirements, UE may adopt some approach to handle the undetectable cells, e.g., set RSRP of those cells or use a model with less inputs. During the test, we don’t think it is necessary to transmit power at all the 21 cells for UE at a certain location. According to the core requirements for L3 measurement, “For each intra-frequency layer, the UE shall be capable of performing SS-RSRP measurements for at least 8 identified cells”, we think it is sufficient for TE to transmit the strongest 8 intra-f cells (serving cell included) of the 21 cells during the test.

Proposal 3:  For inter-cell case is Scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), prefer to reuse the same cell layout adopted in RAN2 evaluations.  It is sufficient for TE to transmit the strongest 8 intra-f cells (serving cell included) of the 21 cells during the test.

- Conclusion

In this contribution, we provide our views for testability of AI mobility. Observations and proposals are summarized as follows.

Proposal 1: Further check whether it is possible to change Tx power every symbol at TE using conductive test in FR1 for AI/ML mobility.

Proposal 2: In FR1, computer simulations alike test setup can be adopted for AI mobility test. The following figure can be used to determine the power transmitted at TE on each cell, i.e., TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model.

Proposal 3:  For inter-cell case is Scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), prefer to reuse the same cell layout adopted in RAN2 evaluations.  It is sufficient for TE to transmit the strongest 8 intra-frequency cells (serving cell included) of the 21 cells during the test.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.