3GPP TSG-RAN WG4 Meeting #115	R4-2506517

Malta 19th – 23rd May 2025

Source: 	Nokia

Title: 					n1 NS_05 A-MPR proposals

Agenda Item: 			4.2.1

Document for:	Approval

# 1	Introduction

We provide A-MPR proposals for band n1 and updated emission mask for NS_05. The proposals are based on our simulations presented in [1] and WF of the previous meeting [2].

# 2	Simulation assumptions and scenario

Here we provide the simulation scenario and parameters that were used in the A-MPR simulations [1] leading to the proposals.

- Operating band n1 (UL 1920 MHz – 1980 MHz)

- Power classes PC2 and PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- Only additional spurious emission mask for Japan is simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

- New additional spurious emission mask for Japan [3] signaled with NS_05:

  - CBW = 5MHz for PC2 and PC3

    - 1884.5-1910.0 MHz: -30 dBm@1MHz

    - 1910.0-1915.7 MHz: -25 dBm@1MHz

  - CBW = 10 MHz, 15 MHz, 20MHz for PC2 and PC3

    - 1884.5-1906.6 MHz: -30 dBm@1MHz

    - 1906.6-1915.7 MHz: -25 dBm@1MHz

# 3	A-MPR proposals

## 3.1	PC3

In the previous meeting it was agreed to study splitting of the IMD3 region A2. In the tables below we show our proposal making this split. The entries that are added or modified are highlighted with yellow color. Note that we have taken the spit also into account for 10 MHz and 15 MHz CBWs besides 20 MHz.

We note that when RBStart = 26 the simulated A-MPR is a bit smaller than for other parts of the new A3 region. This is because part of the IMD power falls outside the mask. For example, the simulated A-MPR for DFT-s-OFDM QPSK and RBStart = 26 is approximately 0.8 dB.

Proposal 1: RAN4 shall consider defining A-MPR for n1 PC3 NS_05 as shown in tables 1 and 2.

Table 1: A-MPR regions for NS_05 (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  | Region A3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart | LCRB | RBstart |
| 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | N/A | N/A | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs |
| 15 | 1927.5 ≤ fc < 1932.5 | ≥ RBstart*1.2 + 4.68MHz/12/scs | ≥ 0 | ≤ 3.96MHz/12/scs | 0 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs; > 0 |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28/12/scs | ≤ 0.54/12/scs | N/A | N/A | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs |
| 20 | < 1945.7 | ≥ RBstart*1.5 + 4.14MHz/12/scs; | ≥ 0 | ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs; | ≤ 1.62MHz/12/scs | ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs; | ≤ 4.68MHz/12/scs; > 1.62/12/scs |

Table 2: A-MPR values for NS_05

(Power Class 3, 20MHz, Fc<1945.7MHz)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 3 | 1.5 |
|  | QPSK | ≤ 4 | ≤ 3.5 | 1.5 |
|  | 16 QAM | ≤ 5 | ≤ 4 | 2 |
|  | 64 QAM | ≤ 5 | ≤ 4 |  |
|  | 256 QAM | ≤ 5 | ≤ 4.5 |  |
| CP-OFDM | QPSK | ≤ 6 | ≤ 5.5 | 3 |
|  | 16 QAM | ≤ 6 | ≤ 5.5 | 3 |
|  | 64 QAM | ≤ 6 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7 | ≤ 6 |  |

## 3.2	PC2

Since no agreement on the harmonized A-MPR region definition for PC2 was made during the previous meeting [2], we resubmit our proposal from [1]. In Appendix we show complete sets of the A-MPR triangles from the proposal. In Fig. 1 we show some examples of them.

We note that when RBStart = 26 the simulated A-MPR is a bit smaller than for other parts of the A2 region. This is because part of the IMD power falls outside the mask. For example, the simulated A-MPR for DFT-s-OFDM QPSK and RBStart = 26 is approximately 1.3 dB.

Proposal 2: RAN4 shall consider defining A-MPR for n1 PC2 NS_05 as shown in tables 3 and 4.

Table 3: The proposed A-MPR regions for n1 PC2 NS_05.

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | Fc < 1929.37 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.44 | ≤ 1.8 | A2 |
|  |  | ≤ 0.18 | > 2.7, ≤ 3.6 | A3 |
| 15 MHz | Fc < 1932.5 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | > 0, ≤ 3.26 | ≤ 3.42 | A2 |
|  |  | 0 | ≤ 1.62 | A4 |
|  | 1932.5 ≤ Fc < 1936.69 | ≤ 1.26 | ≥ 1.0*12*SCS*RBstart + 10.2 | A5 |
|  |  | ≤ 1.44 | ≤ 1.8 | A2 |
| 20 MHz | Fc < 1943.87 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.62 | ≤ 3.6 | A4 |
|  |  | > 1.62, ≤ 4.68 | ≤ 3.6 | A2 |
|  |  | ≤ 0.9 | > 4.14, < 1.0*12*SCS*RBstart + 4.7 | A6 |
|  |  | > 1.8, ≤ 3.42 | > 4.14, ≤ 5.76 | A7 |

Table 4: The proposed A-MPR values for n1 PC2 NS_05.

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 1.5 |  | 3 | 1.5 | 1 |  |
|  | QPSK | 4 | 2 | 1.5 | 3.5 | 2.5 |  | 1 |
|  | 16 QAM | 4 | 2.5 |  | 4 | 2.5 |  |  |
|  | 64 QAM | 4 | 3 |  | 4 |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |
| CP-OFDM | QPSK | 5 | 4 |  | 5.5 | 3.5 |  | 2.5 |
|  | 16 QAM | 5 | 4 |  | 5.5 | 3.5 |  |  |
|  | 64 QAM | 5 | 4 |  | 5.5 |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |

Figure 1: Examples of the proposed A-MPR to satisfy the new NS_05 additional spurious emission limit for 10 MHz and 20 MHz CBWs at the lower edge of the operating band (Fc=1925 MHz and Fc=1930 MHz) for PC2.

# 4	Conclusions

Based on our simulation results we have made the following proposals.

Proposal 1: RAN4 shall consider defining A-MPR for n1 PC3 NS_05 as shown in tables 1 and 2.

Table 1: A-MPR regions for NS_05 (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  | Region A3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart | LCRB | RBstart |
| 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | N/A | N/A | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs |
| 15 | 1927.5 ≤ fc < 1932.5 | ≥ RBstart*1.2 + 4.68MHz/12/scs | ≥ 0 | ≤ 3.96MHz/12/scs | 0 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs; > 0 |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28/12/scs | ≤ 0.54/12/scs | N/A | N/A | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs |
| 20 | < 1945.7 | ≥ RBstart*1.5 + 4.14MHz/12/scs; | ≥ 0 | ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs; | ≤ 1.62MHz/12/scs | ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs; | ≤ 4.68MHz/12/scs; > 1.62/12/scs |

Table 2: A-MPR values for NS_05

(Power Class 3, 20MHz, Fc<1945.7MHz)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 3 | 1.5 |
|  | QPSK | ≤ 4 | ≤ 3.5 | 1.5 |
|  | 16 QAM | ≤ 5 | ≤ 4 | 2 |
|  | 64 QAM | ≤ 5 | ≤ 4 |  |
|  | 256 QAM | ≤ 5 | ≤ 4.5 |  |
| CP-OFDM | QPSK | ≤ 6 | ≤ 5.5 | 3 |
|  | 16 QAM | ≤ 6 | ≤ 5.5 | 3 |
|  | 64 QAM | ≤ 6 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7 | ≤ 6 |  |

Proposal 2: RAN4 shall consider defining A-MPR for n1 PC2 NS_05 as shown in tables 3 and 4.

Table 3: The proposed A-MPR regions for n1 PC2 NS_05.

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | Fc < 1929.37 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.44 | ≤ 1.8 | A2 |
|  |  | ≤ 0.18 | > 2.7, ≤ 3.6 | A3 |
| 15 MHz | Fc < 1932.5 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | > 0, ≤ 3.26 | ≤ 3.42 | A2 |
|  |  | 0 | ≤ 1.62 | A4 |
|  | 1932.5 ≤ Fc < 1936.69 | ≤ 1.26 | ≥ 1.0*12*SCS*RBstart + 10.2 | A5 |
|  |  | ≤ 1.44 | ≤ 1.8 | A2 |
| 20 MHz | Fc < 1943.87 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.62 | ≤ 3.6 | A4 |
|  |  | > 1.62, ≤ 4.68 | ≤ 3.6 | A2 |
|  |  | ≤ 0.9 | > 4.14, < 1.0*12*SCS*RBstart + 4.7 | A6 |
|  |  | > 1.8, ≤ 3.42 | > 4.14, ≤ 5.76 | A7 |

Table 4: The proposed A-MPR values for n1 PC2 NS_05.

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 1.5 |  | 3 | 1.5 | 1 |  |
|  | QPSK | 4 | 2 | 1.5 | 3.5 | 2.5 |  | 1 |
|  | 16 QAM | 4 | 2.5 |  | 4 | 2.5 |  |  |
|  | 64 QAM | 4 | 3 |  | 4 |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |
| CP-OFDM | QPSK | 5 | 4 |  | 5.5 | 3.5 |  | 2.5 |
|  | 16 QAM | 5 | 4 |  | 5.5 | 3.5 |  |  |
|  | 64 QAM | 5 | 4 |  | 5.5 |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |

# 5	References

[1] R4-2419197, n1 Japan A-MPR simulations and proposal, Nokia

[2] R4-2505222, WF on PHS protection update, KDDI

[3] R4-2415439, NR A-MPR reevaluation scenarios on n1 for Japan, KDDI, NTT Docomo

# 6	Appendix: A-MPR triangles

### 6.1	PC2 A-MPR proposal