# 3GPP TSG-RAN WG4 Meeting #115	R4-2506519

# Malta, MT, 19-23 May, 2025

Agenda Item:	7.1.2.2.1

Source: 	Xiaomi

Title: 	Power domain enhancements for single carrier

Document for:	Discussion

- Introduction

In Rel-19 UE RF WI, one of major objective was to study and specify power enhancement on single carrier with emission requirements relaxtion:

- Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM

In this contributions, views on remaning open issues were provided.

- Discussion

Sub-topic 1-1: Approaches to enable MPR reduction for both scenario 1 and scenario2

RAN4#114bis agreements ### Sub-topic 1-1: Approaches to enable MPR reduction #### Issue 1-1-1: RB based or CBW based extension - WF - Symmetrical/ Asymmetric for maximum extension at each side are determined by 0.5*NRB - If a limited number of additional fixed values are introduced, the extension is determined by X*NRB at each side, where X < 0.5 - The extended RBs are aligned with common RB grid #### Issue 1-1-2: Fixed or flexible extension - Agreement in main session: - Fixed value (0.5*CBW and/or 0.5*NRB) for Symmetrical/ Asymmetric by default for UE supporting the MPR reduction feature - In addition, consider a limited number of additional fixed values with capability #### Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - FFS in next meeting #### Issue 1-1-4: Guard band for extended CBW - Options - Option 1: No need to define multiple UE guard bands for initial CBW and extended CBW, UE should have one guard band per each side. All frequency segments within UE extended CBW and outside UE maximum transmission bandwidth (number of RBs in 38.101-1) act as UE total guard bands (i.e., UE initial guard band + additional guard band according to UE extended CBW NW indication), where this UE cannot be allocated and OOBE edges are the outer edges of UE total guard band. - Option 2: NRB based extension and associated guard band: - maximum bandwidth extension: The extended RB BW NRBext inherits the guard-bands (GBext) from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext. - flexible NRB-based extension: Similarly to the ceil(NRB/2) extension, the guard band mapping for the fractional NRB extension can inherit the guard band from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext - Option 3: The GBs of extended UE CBW should only be retained on the side that is far away from the original CBW. The GBs of extended UE CBW reuse the size of original GB of UE CBW. - Option 4: The GBs of the extended CBW shall be the remaining portion obtained by subtracting the RB numbers from the specified channel bandwidth when the extended CBW is selected from Table 5.3.2-1 in 3GPP TS 38.101-1. - Others - WF - FFS in next meeting

Extended CBW approach was adopted for MPR reduction in last RAN4 meeting. From deployment scenario and network scheduling perspective, extended CBW/extended RB numbers on each side of UE CHBW can have wide range with the granularity from zero to half of maximum CHBW in FR1 i.e., 50MHz CHBW.  On the other side, the concept of extended CBW was used to introduce MPR reduction by converting outer RB allocation to inner RB allocation. From RAN4 requirements perspective, we shall consider the actual impact to MPR reduction, evaluation work and test effort to decide confined extend CBW range or fixed values for RAN4 requirement introduction.

Observation 1: Extended CBW approach adopted to serve following purpose

- Specify MPR applicability from outer RB allocation to inter RB allocation

- Specify requirement applicability between IBE and OOBE

Proposal 1: RAN4 shall decide confined fixed value(s) for extended CBW (fixed ratio to UE CHBW) with associated MPR and requirement applicability rules. RAN4 further evaluate follow candidate values on the feasibility

- Candidate value (NRB/2 (default value), NRB/3, NRB/4)

Regarding symmetric/asymmetric CBW extension, we didn’t see any needs to introduce different CBW extension ratio on two sides. Meanwhile, we also understand that in real deployment there is possibility that relaxation maybe only appliable to single side especially when UE CBW close to band edge or BS BW edge.

Proposal 2: CBW extension can be applied to one side or both sides

- if CBW extension is applicable to same side, the extend CBW value shall be same.

On the guard-band decision, we suggest that NW can indicate both RB extension ratio and CHBW extension ratio. With above indication, UE can automatically get the guard-band size on each side.

Proposal 3: The guard-band on extended CBW can be decided by extend CHBW and extended RBs which indicated by network.

Sub-topic 1-2: Applicable requirements

RAN4#114bis agreement #### Issue 1-2-1: A-MPR handling - Proposals - Option 1: Only apply the power domain enhancement scheme to channels which are not subject to A-MPR. - Option 2: - If A-MPR is specified for a given RB allocation X, then the UE performs max(MPRenh(X), A-MPR(X)) - If A-MPR is not specified for a given RB allocation, then the UE performs max(MPR(X), A-MPR(X)) - Option 3: In cases where additional emission should be met, the UE performs the max(MPR, A-MPR) function based on the specified A-MPR and the initial UE CBW MPR and thus set its backoff ignoring any MPR reduction. - Option 4: max(MPRenh(X), A-MPR(X)), where A-MPR(X) is determined based on a given RB allocation, and RB region with respect to initial CBW, WF and modulation etc, and MPRenh(X) is determined based on RB region/etc. within the extended UE CBW. - Others - WF - Only apply the power domain enhancement scheme to channels which are not subject to A-MPR in Rel-19. #### Issue 1-2-2: Whether the extended IBE is applied in the GB of original UE CBW Issue1 Issue 2; Option1. Allocate the extended RBs from the GB of the original UE; Option2. Allocate the extended RBs from the edge of the UE CBW. Option1. Individual evaluation of the extended RBs & Horizontal line; Case1; Case2. Option2. Combine evaluation of the extended RBs with the non-allocated RBs of the UE CBW & Stepped line; Case3; Case4. Note: It is assumed that extended RBs are RB aligned with the UE and BS RBs for the listed cases.. - WF: - Case 1 and case 3 could be further down selected - GB of UE CBW may or may not be scheduled to other UEs - Extended IBE can be applied in the GB of UE CBW - Relaxation can be considered for the GB, based on calculation from the existing IBE formula from the last RB of original UE CBW #### Issue 1-2-3: Derivation of the extended IBE - Options - Option 1: Fixed IBE used for extended CBW at each side of UE CBW, the value is derived from the outmost allocated RB of UE CBW (illustrated in the figure below) - Option 2: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs. - Option 3: Others - WF - Discuss together with issue 1-2-2, and down select the options in next meeting. #### Issue 1-2-4: Relaxation of the extended IBE - Proposals - Proposal 1: For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK. (Xiaomi) - Proposal 2: (vivo) - If the extended RBs are allocated from the GB of the original UE (case1/3), then the IBE in the GB should be relaxed to ensure the MPR of outer allocation reduces to the inner level: - Way 1: Skip the IBE evaluation for the extended RBs allocated in the GB. - Way2: Relax EVM in the IBE formula to that for BPSK for extended RBs. IBE_general = max(PRB - 30 dB, ) - WF - For case 1 in issue 1-2-2, use BPSK EVM in the IBE formula for all extended RBs - For case 3 in issue 1-2-2, FFS the relaxation

On above issue, the initial evaluation results from our side for case 1 and case 3 summarized in below table. We observed both case 1 and case 3 are feasible and case 1 have better performance.

Table1. MPR for case1 and case3

| DFT-S-OFDM, PC3, 15K SCS, QPSK, full allocation | Original | Case1 | Case3 |
| --- | --- | --- | --- |
| UE CBW=20M | 1.1 | -1.1 | -0.9 |
| UE CBW=40M | 0.5 | -1.2 | -0.9 |
| UE CBW=50M | 0.8 | -1.1 | -0.9 |

Proposal 4: Support case 1 for IBE requirements.

Sub-topic 1-3: Signaling and others

On signalling aspects i.e., how to indicate extended RBs/BW location is strongly depending on the discussion/decision on Sub-topic 1-1 for possible extended BW/RB number on one side and associated MPR applicability.

From deployment perspective, the extended BW needs to be combined with BS channel bandwidth or aggregated bandwidth for multi-carrier depending deployment scenario. However, from UE perspective, UE only needs to know the applicable requirements and MPR values with extended BW, the deployment scenario and BS channel bandwidth or aggregated bandwidth can be transparent to UE. Network shall take the responsibility to guarantee system performance without degradation on adjacent system and take proper scheduling to enable the feature or not.

If RAN4 agreed to fix the extended BW on one side as half/BW of UE CBW for inner RB MPR applicability, then UE only needs to know whether enabling power boosting or not and/or on which sides or both sides as well as network can guarantee the actual BS bandwidth or spectrum holding can be equal or large than the gap specified in RAN4 for extended BW.

Observation 2: Actual BS channel bandwidth/aggregated bandwidth and deployment scenarios can be transparent to UE.

Proposal 5: The detailed signalling on extend CBW location and width which pending on decision of extended UE CBW for requirements as fixed value (on/off) or variable values and guard-band decision.

3		Conclusion

In this controbution, views on remaining open issues are provided.

Observation 1: Extended CBW approach adopted to serve following purpose

- Specify MPR applicability from outer RB allocation to inter RB allocation

- Specify requirement applicability between IBE and OOBE

Proposal 1: RAN4 shall decide confined fixed value(s) for extended CBW (fixed ratio to UE CHBW) with associated MPR and requirement applicability rules. RAN4 further evaluate follow candidate values on the feasibility

- Candidate value (NRB/2 (default value), NRB/3, NRB/4)

Proposal 2: CBW extension can be applied to one side only or both sides

- if CBW extension is applicable to both sides, the extend CBW value on each side shall be same.

Proposal 3: The guard-band on extended CBW can be decided by extend CHBW ratio and extended RBs ratio which indicated by network.

Proposal 4: Support case 1 for IBE requirements.

Observation 2: Actual BS channel bandwidth/aggregated bandwidth and deployment scenarios can be transparent to UE.

Proposal 5: The detailed signalling on extend CBW location and width which pending on decision of extended UE CBW for requirements as fixed value set and guard-band decision

Reference

[1] R4-2420527, WF on Power domain enhancement, Huawei

[2] R4-2503013	WF on power domain enhancement, Huawei

[3] R4-2505099	WF on power domain enhancements, Huawei