3GPP TSG-RAN WG4 Meeting #115																		R4-2506522

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.21.2

Source: 	OPPO

Title: 		Discussion on general aspects on AI/ML for NR air interface

Document for:	Approval

- Discussion

Delay Requirements:

The following issues were proposed in the previous meeting:

- whether to establish delay requirements for reporting/monitoring,

- whether to establish delay requirements for functionality activation/switching/fallback,

- whether to establish delay requirements for model switching/fallback.

According to our understanding, the delay requirement for reporting should be considered. However, for some AI/ML based predictive use cases (e.g., time-domain CSI prediction, time-domain beam prediction), the reporting delay requirements might just ensure that the predicted information is available by the target time(e.g., time to use predicted CSI, time to use predicted beam). For example, if a UE uses input data (e.g., reference signals) at time T1 to predict target information (e.g., channel state or beam information) that is to be used at time T2, the delay for reporting such information (or its processed results) can be relaxed. The UE does not need to complete reporting immediately, it only needs to meet the T2 deadline.

Proposal 1: Delay requirements for reporting are necessary, but for AI/ML-based predictive use cases, the reporting delay can be relaxed, i.e., when the prediction time span long, UE does not need to complete reporting immediately.

For delay requirements for monitoring, RAN4 agreed to address delay requirements for UE performance monitoring reporting (e.g., Type 1 Option 2, UE-assisted monitoring for BM) on a case-by-case basis. However, the monitoring details for each use case in the WID should first be defined by other working groups(e.g., RAN1/RAN2) to clarify how performance monitoring will be implemented. RAN4 will define monitoring requirements once RAN1 and RAN2 have more detailed conclusions.

Observation 1: The monitoring details for each use case in the WID should first be defined by other working groups to clarify how performance monitoring will be implemented. RAN4 will define monitoring requirements once RAN1 and RAN2 have more detailed conclusions.

Regarding delay requirements for activation/switching/fallback, RAN4 should consider such delays to prevent performance degradation caused by overly slow activation, switching, or fallback of AI/ML functionalities/models. As for delay requirements for deactivation, we think that RAN4 does not need to define them. UE can autonomously execute deactivation of AI/ML functionalities/models when deemed necessary(or be indicated by NW).  Additionally, from our understanding, the fallback delay is different from deactivation delay, as the former includes the activation of the fallback solution, i.e., essentially a special case of switching delay (from an AI/ML solution to a fallback solution).

Proposal 2: Delay requirements for activation/switching/fallback should be considered in RAN4.

Proposal 3: RAN4 does not need to define delay requirements for deactivation.

Observation 2: The fallback delay is different from deactivation delay, as the fallback delay includes the activation of the fallback solution, i.e., essentially a special case of switching delay (from an AI/ML solution to a fallback solution).

Performance monitoring:

For performance monitoring, stability of the performance monitoring and decision-making mechanism should be considered in RAN4, to mitigate the impact of random effects on monitoring outcomes, includes: (1) obtaining a consistent monitoring result by considering multiple evaluating samples within an evaluation window, (2) assessing whether model monitoring should be handled at the UE level or the cell level.

When it comes to a UE-level monitoring, each user must evaluate and make independent monitoring decisions regarding the utilization, updating, switching, and fallback of AI/ML models. This approach offers flexibility to users, but at the cost of preventing the sharing of evaluation and monitoring results among different users. On the other hand, cell-level monitoring assesses whether the current AI/ML model can be utilized, updated, switched, or fallback on a broader scale. The advantage of this approach is that when a significant number of users within a cell found that the current AI/ML model is no longer suitable, the evaluation and decision outcomes can be applied to other users in the same cell. This streamlines the process, saving monitoring time and effort for users who no longer need to individually identify and address potential performance issues with their respective models.

Proposal 4: Stability of performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes.

- Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: Delay requirements for reporting are necessary, but for AI/ML-based predictive use cases, the reporting delay can be relaxed, i.e., when the prediction time span long, UE does not need to complete reporting immediately.

Observation 1: The monitoring details for each use case in the WID should first be defined by other working groups to clarify how performance monitoring will be implemented. RAN4 will define monitoring requirements once RAN1 and RAN2 have more detailed conclusions.

Proposal 2: Delay requirements for activation/switching/fallback should be considered in RAN4.

Proposal 3: RAN4 does not need to define delay requirements for deactivation.

Observation 2: The fallback delay is different from deactivation delay, as the fallback delay includes the activation of the fallback solution, i.e., essentially a special case of switching delay (from an AI/ML solution to a fallback solution).

Proposal 4: Stability of performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes.

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface