3GPP TSG-RAN WG4 Meeting #115																		R4-2506526

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.21.6

Source: 	OPPO

Title: 	RRM performance requirements for beam management

Document for:	Approval

- Discussion

KPI:

In RAN1#116 meeting, 4 options regarding the content in the BM report of inference results have been agreed, as listed below,

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

In RAN4 #114bis meeting, following agreements have been made for the metrics/KPIs for beam ID prediction

Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

The relationship between K and N in current agreement requires further clarification. Our understanding is that RAN4 only needs to consider “The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB”. For example, RAN4 evaluates whether UE’s predicted top-K beams (e.g., K = 1, 2, 3, 4) contain good beams. The necessity of introducing an additional parameter N remains unclear, and discussions during RAN4#114bis seem also indicated inconsistent interpretations of “N” across companies. Therefore, the definition of top-N needs further clarification to determine whether additional top-N evaluation is required.

Proposal 1: The definition of top-N needs further clarification to determine whether additional top-N evaluation is required.

If both RSRP and beam ID are reported, without including RSRP values for all beams in BM-set A, we prefer both Beam ID prediction accuracy and RSRP accuracy of the reported beams should be considered in RAN4 tests. Because,

- If only beam ID prediction accuracy is evaluated, the RSRP accuracy of reported beams might be extremely strong or poor, undermining the significance of RSRP reporting.

- Conversely, if only RSRP accuracy is evaluated, it may fail to reflect beam prediction effectiveness. For instance, in a top-1 beam prediction scenario with RSRP reporting, if beam i is not the best choice but an AI/ML model designates it as the optimal beam, despite achieving a seemingly perfect absolute RSRP accuracy score by accurately predicting the RSRP of beam i, this could lead to sub-optimal outcomes that deviate from expectations.

However, if both RSRP and beam ID are reported with RSRP values for all beams in BM-set A, we think only evaluating the RSRP accuracy of all beams suffices.

Proposal 2: If both RSRP and beam ID are reported, without including RSRP values for all beams in BM-set A, both Beam ID prediction accuracy and RSRP accuracy of the reported beams should be considered in RAN4 tests.

Proposal 3: If both RSRP and beam ID are reported, with RSRP values for all beams in BM-set A, RAN4 could only evaluate the RSRP accuracy of all beams.

Absolute RSRP accuracy and Relative RSRP accuracy:

In RAN4 #112bis meeting, following agreements have been made for absolute RSRP accuracy

Agreement: If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

In RAN4 #114bis meeting, following agreements have been made for Relative RSRP accuracy

Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Following discussions in the RAN4 #114bis meeting on whether the absolute RSRP accuracy requirement applies to all predicted beams or only a subset, it was agreed that the absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least, with further study needed on its applicability to other beams.

Regarding the reporting of BM inference results, existing RAN1 agreements support differential RSRP reporting and in RAN1 #120 meeting, RAN1 agreed that

Agreement For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size.

Based on the above background agreements, our understanding is: 1) absolute RSRP accuracy requirement applies to Top-1 of predicted beams, and 2) relative RSRP accuracy requirement could be applied to other predicted beams with differential RSRP reporting in RAN4 tests.

Proposal 4: Relative RSRP accuracy requirement could be applied to other predicted beams with differential RSRP reporting in RAN4 tests.

Impact of measurement error on BM performance evaluation:

A simulation assumption for beam prediction was agreed as a starting point for the evaluation in RAN4#112bis meeting. Currently, the assumption is to create a measurement error modeling through link-level simulations. And then hope to apply this link-level measurement error model to system-level simulation assumptions, ultimately completing the evaluation of the impact of measurement error on BM performance requirements.

The current discussion in RAN4 and preliminary simulation assumptions that made in precious meetings are beneficial to the progress of research, but still some issues need further clarification and discussion. For example, 1) considering that the testing environment assumes CDL channel modeling, it needs to be clarified whether the impact of measurement error obtained from the SLS UMA channel (measuring error impact added from a CDL channel evaluation) can be directly applied to the final OTA testing environment. Besides, 2) differences in implementation (e.g., BS antenna configurations, BS antenna element radiation pattern, Beamforming characteristic of the BS pattern) among companies may result in inconsistent evaluation results. How to align the evaluation needs to be considered.

Observation 1: Regarding the impact of measurement error on BM performance evaluation, the following aspects should be further considered

- Whether the impact of measurement error obtained from the SLS UMA channel (measuring error added from a CDL channel evaluation) can be directly applied to the OTA testing environment

- Differences in implementation (e.g., BS antenna configurations, BS antenna element radiation pattern, Beamforming characteristic of the BS pattern) among companies may result in inconsistent evaluation results

- Conclusions

In this contribution, we have discussed the RRM core requirement and testing framework for beam management, and got the following proposals:

Proposal 1: The definition of top-N needs further clarification to determine whether additional top-N evaluation is required.

Proposal 2: If both RSRP and beam ID are reported, without including RSRP values for all beams in BM-set A, both Beam ID prediction accuracy and RSRP accuracy of the reported beams should be considered in RAN4 tests.

Proposal 3: If both RSRP and beam ID are reported, with RSRP values for all beams in BM-set A, RAN4 could only evaluate the RSRP accuracy of all beams.

Proposal 4: Relative RSRP accuracy requirement could be applied to other predicted beams with differential RSRP reporting in RAN4 tests.

Observation 1: Regarding the impact of measurement error on BM performance evaluation, the following aspects should be further considered

- Whether the impact of measurement error obtained from the SLS UMA channel (measuring error added from a CDL channel evaluation) can be directly applied to the OTA testing environment

- Differences in implementation (e.g., BS antenna configurations, BS antenna element radiation pattern, Beamforming characteristic of the BS pattern) among companies may result in inconsistent evaluation results

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface