# 3GPP TSG-RAN WG4 Meeting #115	R4-2506531

# Wuhan, Hubei, China, 19th-23rd May, 2025

Title:	Views on UE and BS demodulation requirements for SBFD

Source:	Qualcomm Incorporated

Agenda item:	7.25.6

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. The objective of the performance part of the Rel-19 work item is as follows:

- Specify necessary BS and/or UE demodulation performance requirements to support SBFD operation.

In this contribution we share our views on the demodulation requirements for both UE and base station components for the Rel-19 SBFD WI within RAN4.

# UE demodulation requirements for SBFD-aware UE

To discuss the UE demodulation requirements, it is important to recall what assumptions are considered in Rel-19 WI. From the WID [1], we have the following assumed:

Followings are assumed based on TR 38.858: - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators

As we can see, from UE demodulation point of view, we are interested in SBFD-aware UE, that is only doing half-duplex operation, while being served by an SBFD-capable gNB. The main objective for the UE demodulation requirements is to discuss and agree on the necessity of introducing (new) demodulation requirements for SBFD-aware UEs. In this section, provide our views on such question and propose a way forward for RAN4 to consider.

Introducing new UE demodulation requirements for SBFD-aware UEs should be motivated by the fact that existing demodulation requirements for legacy TDD UEs will not be sufficient to ensure proper demodulation and operation of the SBFD-aware UEs. From the SBFD-aware UE point of view, the major difference it entails when compared to legacy TDD UEs, is the probable additional inter-UE cross-link interference (CLI) that the SBFD-aware UE might experience, that is non-existent (and not considered when the demodulation requirements were defined) for the legacy TDD UEs. Thus, the question of whether to introduce new demodulation requirements for SBFD-aware UEs should be discussed only after RAN4 agrees if the foreseeable inter-UE CLI requires new demodulation requirements that couldn’t be addressed by legacy TDD UE demodulation requirements.

Regarding the impact of inter-UE CLI, it is important to refer to the extensive work that was carried in Rel-18 study item. Section 11 investigated the adjacent channel coexistence considering different deployment scenarios (e.g., Urban Macro, Urban hotspot, and indoor) and network configurations (SBFD Vs. TDD considering each as victim or aggressor) for both FR1 and FR2-1 deployments. Although the simulations referred here are for adjacent channel coexistence, similar trends regarding impact of inter-UE CLI can be drawn for co-channel coexistence case.

Since Rel-19 SBFD WI is focused on the transmission and reception behaviours on SBFD subband configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon [1], our main scenarios of interest are Case 3: aggressor NR TDD DL victim SBFD DU DL and Case 4: aggressor NR TDD UL victim SBFD DU. From [2], we have the following conclusion for such scenarios:

Table 11.3.3-1: Case 3: aggressor NR TDD DL victim SBFD DU DL Deployment Scenario (Aggressor -> Victim); Frequency range; Co-existence conclusion. Urban Macro -> Urban Macro Scenario 1 and Scenario 6; FR1 and FR2-1; No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput for different BS Tx powers, ranging (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and different SBFD BS antenna configurations.. Urban Hotspot -> Urban Hotspot (N/A for FR2-1) Scenario 2. Indoor -> Indoor Scenario 3 and Scenario 9. Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8. Table 11.3.4-1: Case 4: aggressor NR TDD UL victim SBFD DU Deployment Scenario (Aggressor -> Victim); Frequency range; Co-existence conclusion. Urban Macro -> Urban Macro Scenario 1 and Scenario 6; FR1 and FR2-1; No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput respectively for different BS Tx powers (ranging from 46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and SBFD BS antenna configurations.. Indoor -> Indoor Scenario 3 and Scenario 9. Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8. Urban Hotspot -> Urban Hotspot Scenario 2; FR1; Some companies’ results show SBFD DL throughput degradation is observed only for cell edge throughput due to inter-UE CLI for different grid-shifts (5% to 100%), BS Tx powers (46dBm to 53 dBm) and for all antenna configurations. However more companies show that there is no observed degradation for cell edge throughput and cell average throughput for 100% grid-shift, 49dBm BS Tx power and SBFD BS antenna configuration 2..

For case 3, no degradation due to inter-UE CLI is observed for all deployment scenarios considered for both FR1 and FR2-1. For case 4, it can be observed that there is no inter-UE CLI impacting the DL throughout for Urban macro, indoor and urban Micro/ dense deployments for both FR1 and FR2-1, whereas, when we consider an urban hotspot scenario, some companies observed that only the cell edge UEs might experience DL degradation due to inter-UE CLI.

Observation 1: From Rel-18 study item RAN4 findings, inter-UE CLI had no impact of the DL throughput for urban macro, indoor, and urban micro/dense deployments for both FR1 and FR2-1. Only in the urban hotspot deployments, some companies observed that the cell edge UEs were impacted by inter-UE CLI.

It is important to better understand the characteristics of the Urban hotspot scenario that was considered in Rel-18. Based on Annex E of [2], The Urban Hotspot scenario reused the assumptions of Urban Macro, except Urban Hotspot assumes cluster-based UE dropping, where UEs were dropped within a cluster of radii 25m are indoor and UEs dropped outside a cluster are outdoor (note that RAN4 assumed 80% indoor while 20% outdoor). With the high clustered percentage UEs, the probability of UEs transmitting with their maximum UE is higher, thus, causing more interference on close-by UEs receiving in the DL.

Based on the findings of the adjacent channel coexistence and the feasibility study carried out for the SBFD-aware UE in Rel-18, RAN4 has concluded the following [2]

Based on the study, existing UE RF requirements has been applied as default assumptions for study phase conclusion, since no issues related to existing UE RF requirements has been identified in the co-existence study.

Observation 2: In Rel-18 study item, RAN4 has concluded the following in TR 38.858, section 10 regarding impact on UE RF requirements

Based on the study, existing UE RF requirements has been applied as default assumptions for study phase conclusion, since no issues related to existing UE RF requirements has been identified in the co-existence study.

On the other hand, RAN1 also investigated the impact of inter-UE CLI in [2] and how would inter-UE CLI handling schemes address possible degradation by inter-UE CLI. In this regard, we refer to Section 2 in R1-2401442, where we showed that a simple CLI-aware scheduling scheme shows up to 90% of DL performance degradation can be recovered as shown in the figure below. For more info on the investigated inter-UE CLI handling schemes, the reader is kindly referred to Section 8.4 in TR 38.858.

Figure 1 Recovery of DL Throughput loss with simple CLI-aware scheduler [R1 2401442]

Observation 3: From our RAN1 studies, simple CLI-aware scheduler resulted in up to 90% recovery of DL performance degradation from inter-UE CLI.

Owing to the potential gains of inter-UE handling schemes, an important pillar of the Rel-19 SBFD WI, is the ongoing work in RAN1, RAN2, and RAN3 to specify inter-UE CLI handling techniques. The main focus of such techniques is to address the co-channel inter-subband inter-UE CLI.

Observation 4: Ongoing work in RAN1, RAN2, and RAN3 to develop inter-UE CLI handling techniques to address the co-channel inter-subband inter-UE CLI.

Based on the above, we can draw the following conclusions; 1) inter-UE CLI is only apparent when UEs are geographically close to one another which statistically occurs sporadically in urban macro scenarios, 2) Even when inter-UE CLI generated from the SBFD-aware UE is degrading the DL operation of another UE, inter-UE CLI handling techniques will be leveraged to ensure proper handling of this and avoid DL performance degradation at the victim UE, and 3) No impact due to inter-UE CLI has been flagged during the Rel-18 study item when studying the impact on UE RF requirements. Accordingly, our views that there is no need to define additional UE demodulation requirements for Rel-19 SBFD-aware UEs.

Proposal 1: Based on the findings of Rel-18 study item related to inter-UE CLI impact and the ongoing work in RAN1, RAN2, and RAN3 to develop inter-UE CLI handling techniques, RAN4 agrees to not introduce new UE demodulation requirements for Rel-19 SBFD aware UEs.

# Base station demodulation requirements for SBFD-capable BS

Moving to the base station side, similar to what we considered for the UE, we would like to recall some observations/ conclusions from the Rel-19 study item. From Section 11 in [2], the impact of inter-subband inter-gNB CLI and adjacent interference at the SBFD-capable gNB is summarized as follows:

Table 11.3.3-2: Case 3 SBFD UL co-existence conclusions Deployment Scenario (Aggressor -> Victim); Frequency range; Co-existence conclusion. Urban Macro -> Urban Macro Scenario 1 and Scenario 6; FR1; Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput, and minor degradation but acceptable to some companies for average throughput. With other assumptions (higher BS Tx power and lower grid-shifts), the degradation is increased for cell edge throughput and average throughput.. FR2-1; Under baseline assumptions, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher BS Tx power and lower grid-shifts.. Urban Hotspot -> Urban Hotspot Scenario 2; FR1; Under baseline assumptions, SBFD UL throughput degradation is observed at cell edge throughput and average throughput. With higher BS Tx power and lower grid-shifts, the degradation is increased.. Indoor -> Indoor Scenario 3 and Scenario 9; FR1 and FR2-1; No SBFD UL throughput degradation for both average throughput and cell edge throughput is observed.. Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8; FR1; Under FR1 Urban micro 38dBm Tx power assumption, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher BS Tx power (46dBm) and lower grid shifts.. FR2-1; Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput and no degradation is observed for average throughput..

It can be observed that the impact of inter-gNB CLI is dependent on the underlying system and network parameters (E.g., gNB output power, grid shift, antenna configuration, etc.). That being said, it can be observed that the legacy BS demodulation requirements might not be sufficient to address the inter-gNB CLI. Similar to the RF ongoing work in Rel-19, performance work should aim to provide new demodulation requirements for the SBFD-capable gNB. 

Observation 5: Existing BS demodulation requirements might not be sufficient to address the impact of inter-subband inter-gNB CLI.

Although inter-gNB CLI techniques are currently under investigation in different RAN WGs, the impact of the inter-gNB CLI is more dominant when compared to the inter-UE CLI.

One of the main aspects of defining proper demodulation requirements for the SBFD-capable gNB is proper modeling of the inter-subband inter-gNB CLI. During Rel-18 SI, legacy ACLR and ACS values were adopted and a flat leakage model was utilized to study the adjacent channel coexistence. Such assumption might be simplistic and proper interference modeling might be needed for the BS demodulation work.

Proposal 2: RAN4 to discuss proper interference modeling to account for the inter-subband inter-gNB CLI when specifying BS demodulation requirements for SBFD-capable gNB.

# Conclusion

In this contribution, we provided our views on UE and BS demodulation requirements with the following observations and proposals:

Observation 1: From Rel-18 study item RAN4 findings, inter-UE CLI had no impact of the DL throughput for urban macro, indoor, and urban micro/dense deployments for both FR1 and FR2-1. Only in the urban hotspot deployments, some companies observed that the cell edge UEs were impacted by inter-UE CLI.

Observation 2: In Rel-18 study item, RAN4 has concluded the following in TR 38.858, section 10 regarding impact on UE RF requirements

Based on the study, existing UE RF requirements has been applied as default assumptions for study phase conclusion, since no issues related to existing UE RF requirements has been identified in the co-existence study.

Observation 3: From our RAN1 studies, simple CLI-aware scheduler resulted in up to 90% recovery of DL performance degradation from inter-UE CLI.

Observation 4: Ongoing work in RAN1, RAN2, and RAN3 to develop inter-UE CLI handling techniques to address the co-channel inter-subband inter-UE CLI.

Proposal 1: Based on  Rel-18 study item related to inter-UE CLI impact and the ongoing work in RAN1, RAN2, and RAN3 to develop inter-UE CLI handling techniques, RAN4 agrees to not introduce new UE demodulation requirements for Rel-19 SBFD aware UEs.

Observation 5: Existing BS demodulation requirements might not be sufficient to address the impact of inter-subband inter-gNB CLI.

Proposal 2: RAN4 to discuss proper interference modeling to account for the inter-subband inter-gNB CLI when specifying BS demodulation requirements for SBFD-capable gNB.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- TR 38.858, “Technical Specification Group Radio Access Network; Study on Evolution of NR Duplex Operation (Release 18)”, 3GPP.