3GPP TSG RAN WG4 Meeting #115	R4-2506548

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.23.3.1

Source:	vivo

Title:	Discussion on RAN4 impacts for RRM measurement prediction

Document for:	Discussion

- Introduction

In RAN#105 meeting, the revised SID [1] was approved and RAN4 work is copied as

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this contribution, we further provide our views on the RAN4 impacts for RRM measurement prediction based on the latest WF[2].

- Discussion

  - Relative RSRP prediction accuracy

In the RAN4#114bis meeting, the issues on KPI definition on relative RSRP prediction accuracy was discussed. The outcomes are shown as follows.

Issue 2-1-1: Relative RSRP accuracy definition Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed. - For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2]. - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition. - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. - FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

The above conclusions leave three FFS to be further discussed. We will analyze the first two FFS:

For the first highlighted FFS—"on whether accuracy of predicted relative L3-RSRP from beam level for intra-frequency L3-RSRP will be needed"—the terms "beam 1" and "beam 2" can be understood as either L1 beams or L3 beams.

When the beam here refers to L1 beams, it can be considered as corresponding to the scenario of predicting L3 (at point A or A1) from L1. In the original legacy definition of Relative SS-RSRP Accuracy for FR2, it includes "between any two SS-RSRP levels measured on the same cell in FR2", and the SS-RSRP metrics were originally defined at point A1. Therefore, "predicted relative L3-RSRP from L1 beam level for intra-frequency L3-RSRP" has a corresponding scenario and is already defined in the current spec. This definition can be taken into consideration.

- 10.1.3.1.2 Relative SS-RSRP Accuracy The relative SS-RSRP accuracy is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR2.

When the beam here refers to L3 beams, it can be considered as corresponding to the scenario of predicting RSRP at point E. According to the latest RAN2 meeting discussions, L3 beam-level prediction has been confirmed as one of the standardized scenarios for spec impact study. Therefore, the corresponding definition for this scenario should also be reflected in RAN4.

RAN2#129bis meeting Scenarios/Sub-UseCases/Domains: - L3 Beam-level prediction specification impact will be studied - Noted

Prospoal 1: For intra-frequency L3-RSRP, regarding the definition- ‘Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.’, If interpreted in the following manner, the definitions under both approaches can be considered.

- When the beam here refers to L1 beams, it corresponds to the scenario of predicting L3 (at point A or A1) from L1

- When the beam here refers to L3 beams, it corresponds to the scenario of predicting RSRP at point E.

Regarding the discussion on the second highlighted FFS, as clarified in the first sub-bullet, it heavily depends on how the ground truth is defined in the equation.

If the reported RSRP of Cell2 is measured RSRP, then in the predicted relative RSRP accuracy equation, the terms "reported RSRP of Cell2" and "ground truth RSRP of Cell2/Beam2" would cancel each other out. The equation would reduce to the difference between "reported predicted RSRP of Cell1" and "ground truth RSRP of Cell1"—which is exactly the definition of Absolute RSRP Accuracy.

Here, we will first assume that the ground truth is based on the transmitted or received power (this value is ideal and free from any errors) for further analysis.

When the reported RSRP of Cell2 is predicted, this definition involves taking the difference between the predicted value and the actual ground truth for each of the two cells, and then subtracting these two differences. It primarily measures the prediction deviation for two different cells within the same prediction instance. Unlike the first definition in inter-frequency prediction, as clarified in the brackets, the scenario here corresponds to: Cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on Cell 2. In our understanding, the possible scenario may be: Cell 2 is the serving cell, and its RSRP is predicted in the time domain (reported predicted RSRP for future time instance T1 of Cell 2). Cell 1’s RSRP is predicted in the time domain based on past and current measurements of Cell 2 (reported predicted RSRP for future time instance T1 of Cell 1). However, based on the current scenario discussion, inter-frequency prediction is purely frequency-domain prediction and has not been extended to a combined scenario involving both time and frequency domains. Therefore, we believe that before discussing this definition, the actual scenario needs to be clarified.

When the reported RSRP of Cell2 is measured, this definition refers to the difference between the prediction error of RSRP for Cell 1 and the measurement error of RSRP for Cell 2. This accuracy ensures that the prediction error is at least within a certain range compared to the legacy non-AI measurement error. It can be considered as a candidate definition for further discussion in the WI to determine whether this metric needs to be specified as a requirement.

When the reported RSRP of Cell2 is a combination of both predicted and measured samples, based on our understanding, this definition corresponds to a scenario where the L1 predicted RSRP of Cell 2 itself is based on Case B (where L1 predicts L1, and then L3 is calculated). In this use case, the L3 RSRP would be a combination of both predicted and measured L1 samples. This can also be considered as a candidate definition for further discussion.

Proposal 2: For inter-frequency L3-RSRP, regarding the definition

‘accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2].’,

- For the case that the reported RSRP of Cell2 is predicted, it needs to firstly clarify which scenario this actually corresponds to

- For the cases that the reported RSRP of Cell2 is measured and is a combination of both predicted and measured samples, both of these two cases can be considered

  - Inter-frequency prediction scenario

Issue 2-1-8: Inter-frequency prediction scenario Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

At the last RAN4 meeting, a preliminary discussion was held on the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenarios. Next, we will analyze these factors in detail.

During the discussion on simulation assumptions in RAN2, it was agreed that the Pearson correlation coefficient would be used to measure the correlation between two measured and predicted frequency layers, with the input values being the RSRP measurements of the two frequencies. Based on RAN2 simulation observations and final conclusions, it was demonstrated that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy. Therefore, this factor is one of the key elements affecting RSRP prediction accuracy for inter-frequency prediction, and RAN4 needs to consider it when defining metrics.

Additionally, regarding EPRE difference, as well as frequency distance and frequency separation, we believe these are coupled with the correlation coefficient and can be reflected by it. From this perspective, we tend to avoid splitting them into more detailed factors for discussion.

Proposal 3:As the key element affecting RSRP prediction accuracy for inter-frequency prediction, correlation coefficient need to consider it when defining metrics. Regarding EPRE difference, as well as frequency distance and frequency separation, hese are coupled with the correlation coefficient and no need to splitting them into more detailed factors for discussion

As for the impact of the cluster approach, we conducted simulations using measurements from a single cell and measurements from a group of cells. The results of prediction accuracy for different inputs are shown in Table 1.

Table 1 Simulation results of inter-frequency prediction from 2GHz to 4GHz

| Input | Prediction error (dB) |
| --- | --- |
| Single Cell (co-located in the same sector) | 0.55 |
| All Cells (21 Cells) | 0.36 |
| 3 Cells of co-located station | 0.35 |
| 18 Cells (exclude 3 Cells of co-located station) | 1.42 |
| 3 Cells with the highest RSRP | 0.24 |

It can be observed that the performance of the cluster-based approach using all cells is better than that using a single cell as input. Building on this, we further explored the selection methods for a group of cells, which included the following approaches:

- Approach 1: Selecting cells comprising both co-located and non-co-located cells.

- Approach 2: Selecting cells exclusively from co-located stations.

- Approach 3: Selecting cells comprising only non-co-located cells.

- Approach 4: Selecting the 3 cells with the highest RSRP.

Based on the simulation results, the following observations were made:

- The performance of using 3 co-located cells as input is comparable to that of using 21 cells as input.

- When the 3 co-located cells were removed from the 21-cell set, the prediction accuracy dropped significantly. This indicates that cells from co-located stations contribute substantially to prediction accuracy.

- The performance of using the 3 strongest cells (highest RSRP) as input outperformed that of using 21 cells as input. This suggests that a few strong cells play a critical role in improving prediction accuracy.

Based on these simulations, it can be concluded that the criteria for selecting cells in the cluster-based approach—specifically, the choice of cells within a group of measurements—can influence inter-frequency prediction accuracy. This factor could also be considered as a potential aspect in future RAN4 requirement definitions.

Observation 1: The performance of the cluster-based approach using all cells is better than that using a single cell as input.

Proposal 4: For the cluster-based approach, the criteria for selecting cells can influence inter-frequency prediction accuracy. This factor could also be considered as a potential aspect

Additionally, we analyzed the results of prediction accuracy for different combinations of OW length, and the simulation results are shown in the table below.

- Table 2 Simulation results of prediction from 2GHz to 4GHz

| OW length (ms) | 5*40 | 10*40 | 20*40 | 30*40 | 40*40 | 50*40 |
| --- | --- | --- | --- | --- | --- | --- |
| Number of samples within OW | 5 | 10 | 20 | 30 | 40 | 50 |
| Prediction accuracy (30km/h) | 1.746 | 1.483 | 1.153 | 0.932 | 0.823 | 0.721 |
| Prediction accuracy (60km/h) | 2.218 | 1.980 | 1.599 | 1.364 | 1.215 | 1.099 |
| Prediction accuracy (90km/h) | 2.419 | 2.186 | 1.867 | 1.603 | 1.418 | 1.356 |

As can be seen from the above simulation, the increase in OW length also affects prediction accuracy. The longer the OW length, the more accurate the prediction will be. When defining relevant requirements in subsequent RAN4, OW length can be considered as one of the side conditions to ensure that the input samples are not too short.

Proposal 5: For inter-frequency prediction, the OW length can be considered as one of the side conditions

  - Impacts on beam/cell level measurements

- Background: The following agreement was achieved in RAN4#112bis meeting: Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.. The figure below from TS 38.300 (Figure 9.2.4-1) should be considered as a reference for this discussion. - Proposals and Observations: - Proposal 1 (Ericsson): RAN4 to not consider the impact of L1 beam level measurement prediction at point A1, on RAN4 requirements unless agreed to be considered for AI/ML based mobility by RAN2. - Proposal 2 (Ericsson): RAN4 to consider the impact of L3 beam level measurement prediction at point E, on RAN4 requirements. - Proposal 3 (Huawei, CATT): RAN4 can deprioritize the sub-use cases of L3 filtered beam-level measurement predication (Point E). - Proposal 4 (Huawei): For 3 sub-use cases for cell-level RRM measurement prediction, RAN4 is to define measurement prediction accuracy at point C. - Proposal 5 (ZTE): RAN4 should discuss whether prediction accuracy requirement defined at point C could be used for point A1. - Proposal 6 (ZTE): RAN4 to deprioritize studying the impacts on requirements for L3 beam-level prediction, the definition of prediction accuracy for point E will correspondingly be given low priority. - Proposal 7 (CMCC): except L3 cell level RSRP (Point C) prediction, it is proposed that RAN4 to consider L1 beam-level (Point A1). And it is proposed that L3 beam-level (Point E) can be deprioritized based on RAN2 agreements. - Proposal 8 (MediaTek): For RRM measurement prediction, only use cell level RSRP (Point C) as the performance metric for all the three sub-use cases. - Proposal 9 (Apple): For beam-level L1 requirements and given that L1 reporting is not supported within the L3 mobility framework, bypass L3 filtering and utilize a single Tx beam for testing purposes. (point A1 equivalent to C). - Proposal 10 (OPPO): For all three sub-use cases, RAN4 to define prediction accuracy requirements and testing for L3 cell level measurements at point C. - Proposal 11 (Samsung): - RAN4 to prioritize studying the impacts on requirements for L3 cell level RSRP (Point C) prediction - RAN4 to deprioritize studying the impacts on requirements for L3 beam-level (Point E) prediction, aligning with RAN2 prioritization - Proposal 12 (Samsung): If it is proposed to consider L1 beam level prediction, RAN4 to study the impacts on requirements for L1 beam level RSRP (Point A1) prediction - FFS: Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C - Proposal 13 (Nokia): RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E). - Candidate Options: - Option 1: Do not consider L3 beam-level (Point E) and L1 beam-level (Point A1) measurement prediction - Option 2: Consider L3 beam-level (Point E) measurement prediction as second priority - Option 3: Consider L1 beam-level (Point A1) measurement prediction as second priority - Option 4: RAN4 should consider studying the impacts on requirements for L3 beam-level (Point E) - Option 5: RAN4 should consider studying the impacts on requirements for L1 beam-level (Point A1) - Recommended WF: - Discuss different options listed above. RAN4 agreement in RAN4#114bis Issue 2-1-6: Impacts on beam/cell level measurements Agreement: - For Rel. 19 study: - RAN4 to not consider L1 beam-level (Point A1) measurement prediction.

First, regarding the conclusions reached at RAN4#114bis, we believe further clarification is needed. There can be different interpretations of L1 beam-level (Point A1) measurement prediction:

The first interpretation is that L1 beam-level (Point A1) measurement prediction refers to using the prediction result of A1 as the final reported content, with performance metrics defined based on the output of this point.

The second interpretation, when considering the following three cases, is that L1 beam-level (Point A1) measurement prediction may also include using A1 as an intermediate prediction output (i.e., Case 1) and as a model input (i.e., Case 3). Based on this understanding, the conclusion might be interpreted as RAN4 not considering cases related to Point A1 (Case 1 and Case 3 below).

a)	Case 1: To predict L1 filtered beam level results (point A1), then generate L3 filtered results (point C or point E) based on the predicted L1 beam results.

b)	Case 2: To directly predict L3 filtered beam/cell level results (point E or point C) based on the L3 beam/cell level (point E or point C) measurement results.

c)	Case 3: To directly predict L3 filtered beam/cell level results based on the L1 beam level measurement results (Point A or Point A1).

Based on the discussion process, the first interpretation should actually be the consensus. Moreover, since the results at Point A1 are not the final reported results and cannot be defined with performance metrics, the above conclusion was reached. However, to ensure this conclusion is captured clearly in the final TR, we believe it should be refined.

Additionally, since the latest RAN2 meeting also agreed on the NW-sided model, in this scenario, at least for Case 3, the UE needs to report L1 beam-level measurement results (Point A or Point A1), and RAN4 should consider defining requirements for these reported results.

Based on these two points, we propose that RAN4 refine the conclusion as follows:

Proposal 6: For the following conclusion made in RAN4#114bis, RAN4 to update the wording as:

For Rel-19 study, RAN4 to not consider L1 beam-level (Point A1) measurement prediction for UE sided model when L1 beam-level (Point A1) predicted results as the report content

• For Rel. 19 study, RAN4 to not consider L1 beam-level (Point A1) measurement prediction.

  - Impacts of measurement error on prediction accuracy

- Recommended WF - Option 1：RAN4 to consider the impact of measurement error on accuracy requirements in RAN4 during WI stage similar as AI/ML for NR Air Interface - Option 2：RAN4 should study the measurement error modeling by using the tables below as a baseline - Table RF and BB error modelling principle RF and BB modeling principle; Supporting companies. Both RF and baseband errors follow a Gaussian distribution.; X, Y, Z, U .... Baseband and RF errors follow different distributions.; A, B, C, D …. - Table 2 BB error modeling Baseband error modeling; Supporting companies. Gaussian distribution; A, B …. Link-level simulation results; C, D …. - Table 3 RF error modeling RF error modeling; Supporting companies. Uniform distribution; A …. Gaussian distribution; B …. Truncated Gaussian distribution; C …. Fixed value; D .... - Table 4 RF error application How RF error is applied; Supporting companies. Independent for each Tx beam; A …. same for each Tx beam; B …. Independent for each module/panel; C …. same for each module/panel; D ....

Regarding the impact of measurement errors, we believe the error modeling approach can directly leverage the existing discussions on AI/ML for NR Air Interface as a baseline. Below is the error modeling method considered during AI/ML beam simulation.

For BB error, the Link-level simulation results were selected after discussion. For RF error, a Truncated Gaussian distribution under ±4.5 dB RF error (μ=0, σ²=4.5) is used in AI/ML. The Uniform distribution was initially excluded to better align with simulation results, but in reality, the difference between this model and the Truncated Gaussian distribution in terms of BM prediction outcomes is minimal. Therefore, for AI mobility, we believe both models can be retained for now. The Fixed value model can be excluded first—from the BM simulation results, it can be observed that the prediction accuracy is not impacted by the 4 dB RF offset. A possible reason is that the trend of all datasets remains unchanged when a 4 dB offset is applied to all RSRP elements.

Specifically, for RF error, "Independent for each Tx beam" is adopted.

Based on the above analysis, we propose the following for measurement error modeling for AI mobility.

- For BB error modeling:

  - Link-level simulation approach is considered

- For RF error,

  - For error modeling, the following two modelings can be the candidates.

    - Alt 1: Uniform distribution

    - Alt 2: Truncated Gaussian distribution

  - RF error application, each of Tx beam is independent

During the SI (Study Item) phase, we believe this baseline is sufficient. The table can be captured in the TR (Technical Report) to facilitate further inGround Truth Definition for RSRP accuracy

Proposal 7: On the impact of measurement errors, RAN4 to leverage the outcome of the error modeling approach in AI/ML for NR Air Interface. Specifically, the following error modelings can be as a baseline:

- For BB error modeling:

  - Link-level simulation approach is considered

- For RF error,

  - For error modeling, the following two modelings can be the candidates.

    - Alt 1: Uniform distribution

    - Alt 2: Truncated Gaussian distribution

  - RF error application, each of Tx beam is independent

RAN4#114 Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2. Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list RAN4#114bis Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1 Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

During the previous meeting, RAN4 reached preliminary conclusions regarding the Ground Truth definition for RSRP accuracy. For FR1, we will proceed to conduct separate analyses of Alt 1 and Alt 2 across all the proposed aspects mentioned above.

Proposal 8: Regarding the Ground Truth definition for RSRP accuracy, the analyses of Alt 1 and Alt 2 are shown as below

|  | Issues | Impact to Alternative 1 | Impact to Alternative 2 |
| --- | --- | --- | --- |
| 1. | The information at TE side, on the timing UE performs and/ or finishes the measurement and/or prediction | No with AWGN channel is assumed: If the AWGN channel is considered, it is assumed that the interval duration between time instances in both the observation and prediction windows are aligned, and their corresponding starting points are also aligned. The feasibility lies in the fact that both the measurement and prediction configurations can be adjusted. The timing information can be known to the TE by controlling both the measurement configuration and prediction timing. YES with fading channel is assumed: Under a fading channel, due to dynamic changes in the channel environment, obtaining ground truth may involve significant inaccuracies. | No with certain conditions: This issue emphasizes whether timing information can be obtained on the TE (Transmission Entity) side. Although the exact execution and completion time points of measurements are known on the UE side, if UE reporting is considered, the ways for the TE to be aware of the timing information include: Same as the analysis for Alt 1 (AWGN case) on the left The measurement results reported by the UE include timestamp information indicating when the measurements were performed |
| 2. | Impact of L3 filtering | YES, in certain scenarios: The filtering coefficient can be network-configured and thus known to the TE (at least for the case where L1 predicts L1 and then calculates L3). However, for the case where L1 directly predicts L3, the L3 filter is embedded in the model, and the consolidation/filtering process is treated as a black box—meaning the TE cannot know the filtering details. For L1 predicts L1 → then calculates L3: The filtering coefficient is a specific value configured by the network and can be known to the TE. For L1 directly predicts L3: The L3 filter is integrated into the model, and the consolidation/filtering process is black box, so the TE has no visibility into it. | YES, in certain scenarios: Similar to Alt 1, for the case where L1 directly predicts L3, the L3 filter is embedded in the model, and the consolidation/filtering process is treated as a black box. During UE measurements, the filtering coefficient may be based on network configuration. However, during prediction, the process may depend on UE implementation, leading to potential inconsistencies between: The filtering coefficient and the specific process used for the obtained measured value, And the actual model implementation (which remains opaque to the TE). This discrepancy could impact the alignment between measurement and prediction behaviors. |
| 3. | Number of samples of L1 filtering | L1 filtering and the number of samples for filtering are not known by the TE and entirely depend on UE implementation. | NO |
| 4. | Timing window, if it exists, where UE can’t measure the ground truth in Alt2 | NO | In Temporal Case B, ground truth may be missing if measurements are skipped. How to obtain the true measurement values at the corresponding moments requires further analysis. |
| 5. | Channel condition | Under fading channels, due to the dynamic changes in the channel environment, the acquisition of ground truth may be highly inaccurate. | NO |
| 6. | Avoid UE cheat in the test | NO | No feasible solution has been found yet, even in the discussion of the R19 AI interface beam management test. |
| 7. | S(I)NR variations in the test at different time instances (for Alt2) | NO | No with certain conditions the signal environment corresponding to the measured value is the signal environment at the moment of the predicted time instance, and there is no influence caused by changes in channel conditions or excessively low channel conditions |

- Summary

Prospoal 1: For intra-frequency L3-RSRP, regarding the definition- ‘Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.’, If interpreted in the following manner, the definitions under both approaches can be considered.

- When the beam here refers to L1 beams, it corresponds to the scenario of predicting L3 (at point A or A1) from L1

- When the beam here refers to L3 beams, it corresponds to the scenario of predicting RSRP at point E.

Proposal 2: For inter-frequency L3-RSRP, regarding the definition

‘accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2].’,

- For the case that the reported RSRP of Cell2 is predicted, it needs to firstly clarify which scenario this actually corresponds to

- For the cases that the reported RSRP of Cell2 is measured and is a combination of both predicted and measured samples, both of these two cases can be considered

Proposal 3:As the key element affecting RSRP prediction accuracy for inter-frequency prediction, correlation coefficient need to consider it when defining metrics. Regarding EPRE difference, as well as frequency distance and frequency separation, hese are coupled with the correlation coefficient and no need to splitting them into more detailed factors for discussion

Observation 1: The performance of the cluster-based approach using all cells is better than that using a single cell as input.

Proposal 4: For the cluster-based approach, the criteria for selecting cells can influence inter-frequency prediction accuracy. This factor could also be considered as a potential aspect

Proposal 5: For inter-frequency prediction, the OW length can be considered as one of the side conditions

Proposal 6: For the following conclusion made in RAN4#114bis, RAN4 to update the wording as:

For Rel-19 study, RAN4 to not consider L1 beam-level (Point A1) measurement prediction for UE sided model when L1 beam-level (Point A1) predicted results as the report content

Proposal 7: On the impact of measurement errors, RAN4 to leverage the outcome of the error modeling approach in AI/ML for NR Air Interface. Specifically, the following error modelings can be as a baseline:

- For BB error modeling:

  - Link-level simulation approach is considered

- For RF error,

  - For error modeling, the following two modelings can be the candidates.

    - Alt 1: Uniform distribution

    - Alt 2: Truncated Gaussian distribution

  - RF error application, each of Tx beam is independent

Proposal 8: Regarding the Ground Truth definition for RSRP accuracy, the analyses of Alt 1 and Alt 2 are shown as below

|  | Issues | Impact to Alternative 1 | Impact to Alternative 2 |
| --- | --- | --- | --- |
| 1. | The information at TE side, on the timing UE performs and/ or finishes the measurement and/or prediction | No with AWGN channel is assumed: If the AWGN channel is considered, it is assumed that the interval duration between time instances in both the observation and prediction windows are aligned, and their corresponding starting points are also aligned. The feasibility lies in the fact that both the measurement and prediction configurations can be adjusted. The timing information can be known to the TE by controlling both the measurement configuration and prediction timing. YES with fading channel is assumed: Under a fading channel, due to dynamic changes in the channel environment, obtaining ground truth may involve significant inaccuracies. | No with certain conditions: This issue emphasizes whether timing information can be obtained on the TE (Transmission Entity) side. Although the exact execution and completion time points of measurements are known on the UE side, if UE reporting is considered, the ways for the TE to be aware of the timing information include: Same as the analysis for Alt 1 (AWGN case) on the left The measurement results reported by the UE include timestamp information indicating when the measurements were performed |
| 2. | Impact of L3 filtering | YES, in certain scenarios: The filtering coefficient can be network-configured and thus known to the TE (at least for the case where L1 predicts L1 and then calculates L3). However, for the case where L1 directly predicts L3, the L3 filter is embedded in the model, and the consolidation/filtering process is treated as a black box—meaning the TE cannot know the filtering details. For L1 predicts L1 → then calculates L3: The filtering coefficient is a specific value configured by the network and can be known to the TE. For L1 directly predicts L3: The L3 filter is integrated into the model, and the consolidation/filtering process is black box, so the TE has no visibility into it. | YES, in certain scenarios: Similar to Alt 1, for the case where L1 directly predicts L3, the L3 filter is embedded in the model, and the consolidation/filtering process is treated as a black box. During UE measurements, the filtering coefficient may be based on network configuration. However, during prediction, the process may depend on UE implementation, leading to potential inconsistencies between: The filtering coefficient and the specific process used for the obtained measured value, And the actual model implementation (which remains opaque to the TE). This discrepancy could impact the alignment between measurement and prediction behaviors. |
| 3. | Number of samples of L1 filtering | L1 filtering and the number of samples for filtering are not known by the TE and entirely depend on UE implementation. | NO |
| 4. | Timing window, if it exists, where UE can’t measure the ground truth in Alt2 | NO | In Temporal Case B, ground truth may be missing if measurements are skipped. How to obtain the true measurement values at the corresponding moments requires further analysis. |
| 5. | Channel condition | Under fading channels, due to the dynamic changes in the channel environment, the acquisition of ground truth may be highly inaccurate. | NO |
| 6. | Avoid UE cheat in the test | NO | No feasible solution has been found yet, even in the discussion of the R19 AI interface beam management test. |
| 7. | S(I)NR variations in the test at different time instances (for Alt2) | NO | No with certain conditions the signal environment corresponding to the measured value is the signal environment at the moment of the predicted time instance, and there is no influence caused by changes in channel conditions or excessively low channel conditions |

- References

- RP-242393 Revised SID on AIML for mobility in NR, OPPO

- R4-2504910 WF on RRM requirements for FS_NR_AIML_Mob, Nokia