3GPP TSG RAN WG4 Meeting #115	R4-2506549

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.23.3.2

Source:	vivo

Title:	Discussion on RAN4 impacts for measurement event prediction

Document for:	Discussion

- Introduction

In the last meeting, the discussion on potential RRM impacts for measurement event prediction was initially discussed. The considerations from companies and the latest progress on RAN4 impacts for measurement event prediction were summarized in WF [1]. In this contribution, we would like to provide our further analysis on aspects on RRM requirements impacts for measurement event prediction.

- Discussion

Issue 3-1-1: Prioritization of cases for event prediction Agreement: - RAN4 should discuss both indirect and direct cases. If any prioritization will be agreed in RAN2, RAN4 to follow RAN2 prioritization Issue 3-1-2: Performance metric for event prediction use case Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs

The last RAN4 meeting concluded with the above findings regarding measurement event prediction. For the "Potential RRM requirement impact for event prediction for indirect case," it includes two aspects: one is the "delay requirement for predicted event triggered reporting," and the other is the "metric for event prediction."

Regarding the "delay requirement for predicted event triggered reporting," let’s first analyze the existing "requirement for Event Triggered Reporting." The "measurement reporting delay" is defined from the moment an event that will trigger a measurement report actually occurs until the UE transmits the measurement report. In legacy, the time when the event condition is met must always precede the UE’s transmission of the measurement report, ensuring that the UE can confirm the event condition is satisfied during measurement.

However, for measurement event prediction, the process is as follows: The UE inputs past/current measurement results into a model. → Based on measurements and/or prediction results, it determines whether an event will occur at a future time. → If the UE predicts that a measurement event will occur at a future time, it reports the predicted RSRP of serving/neighboring cells as an intermediate output, and the expected occurrence time of a certain measurement event as the final output.

Thus, the expected occurrence time of the measurement event corresponds to a future moment, which will necessarily be later than the UE’s transmission of the prediction report. In this case, the delay can not follow the existing logic of the "measurement reporting delay" definition. Given this issue, we tend not to define the related delay requirement.

Observation 1: In legacy, the time when the event condition is met must always precede the UE’s transmission of the measurement report, ensuring that the UE can confirm the event condition is satisfied during measurement. While for the measurement event prediction, the expected occurrence time of the measurement event corresponds to a future moment. It will be later than the UE’s transmission of the prediction report

Proposal 1: RAN4 not to study the delay requirement for the predicted event triggered reporting

#### 9.2.4.3 Event Triggered Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled. The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

For the accuracy requirement for measurement event prediction, according to the description of the following captures in TR38.744, the F1 score serves as the performance metric for evaluating the accuracy of measurement event prediction. For indirect prediction, the counters n1, n2, and n3 in the calculation formula are related to the length of the maximum ETD. When the ETD (i.e., the timing difference between the ground-truth event and the predicted event) is larger, the actual time of the event occurrence is more likely to fall within the time range of [the expected occurrence time of the event ± ETD], resulting in a better displayed F1 score. Therefore, when evaluating the accuracy of event prediction, it should be defined as the F1 score under certain fixed ETD conditions.

TR 38.744 ### Evaluation methodology, metrics and assumptions The performance metric F1 score is defined as following: F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3) For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD] The ETD i.e. timing difference between ground-truth event and predicted event is illustrated in Figure 5.3.1-1: Figure 5.3.1-1: illustration of ETD As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected. For direct prediction, the counter n1, n2 and n3 in the formula are defined as following: Counter n3 (true event prediction): it increases by 1 when a ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n1 (false event detection): it increases by 1 when no ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n2 (missed event detection): it increases by 1 when a ground-truth event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold For direct prediction, the counter n1, n2 and n3 in the formula are defined as following: Counter n3 (true event prediction): it increases by 1 when a ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n1 (false event detection): it increases by 1 when no ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n2 (missed event detection): it increases by 1 when a ground-truth event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold For direct prediction method, a measurement event could be predicted within an occurrence window starting from current time instance i.e. t0 and future time instance t1 with a probability as illustrated in Figure 5.3.1-1. Figure 5.3.1-2: occurrence window of direct prediction method For measurement event prediction based on intra-frequency temporal domain case A or case B, the simulation assumptions for intra-frequency temporal domain case A or case B in Table 5.2.1-1, Table 5.2.1-2, Table 5.2.1-3 and Table 5.2.1-4 are reused respectively. On top of that, following additional simulation assumptions are used for measurement event prediction based on intra-frequency temporal domain case A in Table 5.3.1-1 and temporal domain case B in Table 5.3.1-2 respectively:

Proposal 2: F1 score under certain fixed ETD conditions can be served as the performance metric for evaluating the accuracy of measurement event prediction.

Regarding whether to define requirements for the predicted RSRPs when the report includes the predicted RSRPs corresponding to the predicted event occurrence time, we believe it is necessary to define corresponding RSRP accuracy requirements. First, the predicted RSRP will be reported to the network as part of the uplink content, and its RSRP accuracy must be ensured. Second, for the indirect case, as we understand it, one of the benefits of introducing the predicted RSRP as an intermediate output to be reported to the network is that when the network detects an error in the predicted measurement time, it can use the predicted RSRP to verify whether the issue lies with the UE's calculation of the predicted measurement event from the correct predicted RSRP or with the model's prediction of the RSRP itself.

Proposal 3: If the report includes the predicted RSRPs corresponding to the predicted event occurrence time, RAN4 will define requirement for the predicted RSRPs

- Summary

Observation 1: In legacy, the time when the event condition is met must always precede the UE’s transmission of the measurement report, ensuring that the UE can confirm the event condition is satisfied during measurement. While for the measurement event prediction, the expected occurrence time of the measurement event corresponds to a future moment. It will be later than the UE’s transmission of the prediction report

Proposal 1: RAN4 not to study the delay requirement for the predicted event triggered reporting

Proposal 2: F1 score under certain fixed ETD conditions can be served as the performance metric for evaluating the accuracy of measurement event prediction.

Proposal 3: If the report includes the predicted RSRPs corresponding to the predicted event occurrence time, RAN4 will define requirement for the predicted RSRPs

- References

- R4-2504901, WF on RRM requirements for FS_NR_AIML_Mob, Nokia