3GPP TSG RAN WG4 Meeting #115	R4-2506552

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.31.5.2

Source:	vivo

Title:	Discussion on RRM impacts on DL coverage for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 further analyzed the potential RRM impacts on the objectives of downlink coverage enhancements. The considerations from companies and the latest progress were summarized in WF [1]. In this contribution, we would like to provide further analysis of the potential RRM impacts on DL coverage.

- 2.	Discussion

  - RRM requirements

- RRC re-establishment:

## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms) Agreement (online): - In RRC re-establishment requirements, for the case SSB Ês/Iot < -8dB and unknown NR cell, - Make the following updates - Remove Note 1 in Table 6.2C.1.2.1-1 and Table 6.2C.1.2.1-2 for the applicable scenarios, and the value of TSMTC and TSMTC,i is FFS - Tidentify_intra_NR: MAX (800 ms, [22] x TSMTC) for intra-frequency carrier - Tidentify_inter_NR,i: MAX (800 ms, [22] x TSMTC,i) for inter-frequency carrier

For the requirement defined as Ês/Iot < -8dB and SSB periodicity >20 ms, the exact delay requirements were left in square brackets for further confirmation during the last meeting. Our concern regarding this value is that, based on the Re-establishment timer, the configurable duration is as follows. When SMTC is 160 ms, the delay corresponding to [22] x TSMTC is very long (e.g., Tidentify_intra_NR + Tidentify_inter_NR = 7040 ms, considering Nfreq = 2). In this case, the Re-establishment timer may have already been expired if the timer length is configured to be relatively short.

To ensure the UE can select a suitable cell through reconstruction as much as possible, the network needs to configure a relatively long Re-establishment timer length accordingly. Choosing such a long Re-establishment delay—compared to leaving it up to UE implementation to determine the Re-establishment duration (i.e., no requirement in legacy) and the timing of entering idle mode for cell selection—may depend on the scenario to determine which approach is better.

However, considering that it can also be resolved by the network configuring an appropriate timer length, we can also accept this method and the corresponding value.

RLF-TimersAndConstants ::=          SEQUENCE {

t310                                ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000, ms4000, ms6000},

n310                                ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20},

n311                                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10},

...,

[[

t311                                ENUMERATED {ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000}

]]

}

Observation 1: When SMTC is 160ms, the delay corresponding to [22] x TSMTC is very long. The Re-establishment timer may have already been expired if the timer length is configured to be relatively short.

- Paging:

#### 4.2C.2.5 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms. Where, If the target cell belongs to the same satellite as the current one, and if the target cell is known, then K = 2. If the target cell belongs to a different satellite than the current one and the target cell’s satellite is GEO, and if the target cell is known, then K = 2. If the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO, then K = 5 if the target cell is known.

- For Maximum interruption in paging reception, some companies propose that when the SSB cycle adopts to 160ms, this will result in a long interruption time (may exceed 1 second for LEO scenario), for which we think:

- Before discussing the enhancement of supporting extended default SSB periodicity to 160ms during initial access, we would like to point out that with the following configuration, the configurable values of SSB periodicity have already included 160ms (not in case initial access).

ssb-PeriodicityServingCell ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160},

- Correspondingly, for interruption in paging reception, the existing requirements do not exclude the case where the SSB cycle is configured to 160ms. Therefore, as we understand it, the issue already exists for legacy requirements, not as a new scenario adaptation issue. From this perspective, our preference is to keep the original requirement unchanged.

- Proposal 1: For paging interruption in RRC Idle/Inactive, no adjustments will be conducted for the requirements when SSB periods =160 ms in NGSO scenarios

  - Reply LS on SMTC enhancements

- LS on SMTC enhancements from RAN2(R2-2503068) - Overall description For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe. - Actions To RAN4: ACTION: RAN2 kindly asks RAN4 to provide feedback on the feasibility to support those agreements in Rel-19 timeframe.

At the last RAN2 meeting, an LS [1] regarding SMTC enhancements was agreed, with the main content as shown above. This LS mainly includes support for two enhancements:

Enhancement 1: In the current NTN design for SMTC, each SMTC offset is provided by each SSB-MTC4 configuration, and the corresponding offset values can differ. However, the duration parameter and periodicity must remain consistent with the smtc1 configuration, meaning that different SMTCs share the same periodicity. Therefore, the first enhancement to support configuring different periodicities for different SMTCs.

- In legacy, SSB-MTC2 already supports configuring different periodicities for each SMTC. Now, in the NTN scenario, since UEs optionally support configuring up to 4 SMTCs, this enhancement extends the scenario to support configuring different periodicities for each of these four SMTCs. This enhancement is relatively straightforward and provides greater flexibility for SMTC configuration, which we believe is acceptable.

- TS38.331 If smtc4list is present, for cells indicated in the pci-List parameter in each SSB-MTC4 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received offset parameter in each SSB-MTC4 configuration and use the duration parameter and periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

- Proposal 2: On configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements, it is feasible and can be supported

Enhancement 2: For Enhancement 2, we first want to confirm how to understand "support configuring more than 4 SMTCs per frequency (i.e., 6) for idle/inactive UEs." There are two potential interpretations:

Interpretation 1: The existing parallelSMTC-r17 will be modified, enhancing UE support from optionally supporting a maximum of 4 SMTCs to a maximum of 6 SMTCs.

This approach would significantly increase UE implementation complexity, requiring UEs to have very strong processing capabilities to handle up to 6 SMTCs simultaneously. We do not favor this type of enhancement.

Interpretation 2: This enhancement does not modify the existing parallelSMTC-r17. The enhancement lies in allowing the network to configure more SMTCs, with RAN2’s corresponding modification involving expanding the following IEs (e.g., extending SIZE (1..3) to SIZE(1..6)) (based on our understanding of "at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases," meaning the UE still supports a maximum of 4 SMTCs). This is our understanding of the second enhancement in the RAN2 LS.

SSB-MTC4List-r17::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

For the enhancement under Interpretation 1, RAN4 discussed a similar scenario during R17. The scenario was "when the number of configured SMTCs per frequency layer exceeds UE capability." Based on the discussion at that time, RAN4's final conclusion, as reflected in the spec, is highlighted in the following section:

The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected.

The included principles are as follows:

- If the number of SMTCs broadcasted by the NW exceeds the UE's capability, the UE may select SMTCs sequentially until all SMTCs can be measured.

- A longer measurement delay is expected if the UE is configured with more SMTCs than its capability.

Regarding RAN2's agreement to increase the number of SMTCs configurable by the network to 6, we understand that this has already been covered by the existing RAN4 spec. From a feasibility perspective, it can be confirmed that such a configuration is permissible, though a longer measurement delay is expected in this case.

From the perspective of RAN4 requirements, since R17 has already discussed whether and how to define corresponding delay requirements, we prefer to maintain the existing RAN4 spec without further discussion on whether additional definition or enhancement of requirements is needed for this scenario.

Proposal 3: Firstly, RAN4 to align the interpretation on the enhancement on ‘supporting configuring more than 4 SMTCs per frequency (i.e.,6) for idle/inactive UEs’ from the following alternatives:

- Interpretation 1: The existing parallelSMTC-r17 will be modified, enhancing UE support from optionally supporting a maximum of 4 SMTCs to a maximum of 6 SMTCs.

- Interpretation 2: This enhancement does not modify the existing parallelSMTC-r17. It lies in allowing the network to configure more SMTCs, with RAN2’s corresponding modification involving expanding the following IEs (e.g., extending SIZE (1..3) to SIZE(1..6))

Proposal 4: If the Interpretation 2 is the correct one, it can be cover by the RAN4-specified case ‘when the number of configured SMTCs per frequency layer exceeds UE capability.’ in Rel-17. RAN4 to follow the legacy following principle to handle this case

- Summary

Observation 1: When SMTC is 160ms, the delay corresponding to [22] x TSMTC is very long. The Re-establishment timer may have already been expired if the timer length is configured to be relatively short.

- Proposal 1: For paging interruption in RRC Idle/Inactive, no adjustments will be conducted for the requirements when SSB periods =160ms in NGSO scenarios

- Proposal 2: On configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements, it is feasible and can be supported

Proposal 3: Firstly, RAN4 to align the interpretation on the enhancement on ‘supporting configuring more than 4 SMTCs per frequency (i.e.,6) for idle/inactive UEs’ from the following alternatives:

- Interpretation 1: The existing parallelSMTC-r17 will be modified, enhancing UE support from optionally supporting a maximum of 4 SMTCs to a maximum of 6 SMTCs.

- Interpretation 2: This enhancement does not modify the existing parallelSMTC-r17. It lies in allowing the network to configure more SMTCs, with RAN2’s corresponding modification involving expanding the following IEs (e.g., extending SIZE (1..3) to SIZE(1..6))

Proposal 4: If the Interpretation 2 is the correct one, it can be cover by the RAN4-specified case ‘when the number of configured SMTCs per frequency layer exceeds UE capability.’ in Rel-17. RAN4 to follow the legacy principle to handle this case

- References

- R4-2504913, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm Incorporated