3GPP TSG-RAN WG4 Meeting # 115                                R4-2506563

# Malta, MT, 19th – 23rd May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion impacts on requirements of AI/ML mobility

Agenda Item:	7.23.3.1

Document for:	Approval

- Introduction

In RAN plenary #105 meeting, a revised SID[1] for Rel-19 was approved to study the AI/ML for mobility in NR. The objectives for RAN4 of this SID are listed as follows.

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

In this document, we will provide some views on requirements impacts based on RAN2 assumptions of AI/ML for mobility from the RRM perspective.

- Discussion

In previous meeting, we discussed the priorities of different use cases and have reached consensus that RAN4 to start the discussion with RRM measurement prediction use-case. On RRM measurement prediction, we provide our views on RRM impact on RRM measurement prediction use-case in the follow.

It’s noted that both cell level measurement prediction and beam level measurement prediction are within the scope of RRM measurement prediction. We can observe that there are three sub use cases need to be considered for cell level measurement. For sub use case 1, beam level results are predicted first and then cell level results are derived. For sub use case 2 and 3, cell level results are predicted directly. For beam level measurement prediction, under the discussion in RAN2, L3 filtered beam level prediction cases are lower priority and there are three cases as well.

=> Cell level measurement prediction model For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results => Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results.

In the pervious meeting, we have reached agreement that define requirement for L3 cell level RSRP prediction, and further dicussion is needed for L1 beam-level and L3 beam-level. In the last meeting, we have reached consensus that L1 beam-level (point A1) measurement prediction is not needed to consider in R19. therefore, we only need to discuss L3 cell level and L3 beam level level RSRP prediction requirements.

Agreement on RAN4#112bis Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress. Agreement on RAN4#114bis Issue 2-1-6: Impacts on beam/cell level measurements Agreement: - For Rel. 19 study: - RAN4 to not consider L1 beam-level (Point A1) measurement prediction.

As to point E, which is related to beam level measurement prediction, we should discuss whether to support beam level measurement prediction in RAN4 first. According to RAN2’s agreement, L3 filtered beam level prediction cases are lower priority and companies should focus on FR2-to-FR2 intra-frequency temporal domain prediction caseA (scenario 4). Therefore, RAN4 should deprioritize the beam level measurement prediction. In our understanding, we should discuss the impacts on requirements for L3 cell-level prediction first and the prediction accuracy for point E will be given low priority.

Observation1: Both cell level measurement prediction and beam level measurement prediction are within the scope of RRM measurement prediction. And according to RAN2’s agreements, L3 filtered beam level prediction cases are lower priority and companies should focus on FR2-to-FR2 intra-frequency temporal domain prediction caseA (scenario 4).

Proposal 1：RAN4 should study the impacts on requirements for L3 cell-level prediction first and the prediction accuracy for L3 beam-level prediction can use the conclusion of L3 cell level as baseline.

In addition, as mentioned by other companies, RAN4 should consider whether to define separate prediction accuracy for each sub use case or define one prediction accuracy requirement to cover all cell level sub use cases.

Proposal 2: RAN4 should discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases.

For measurement prediction accuracy of cell-level measurement prediction, metrics for measurement prediction accuracy defined in RAN2 can be used as start point. In the last meeting, we have discuss this issue and reached consensus blew.

Issue 2-1-1: Relative RSRP accuracy definition Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed. - For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2]. - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition. - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. - FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

In this meeting, we should discuss the remaining issues from the last meeting.

For the first issue, whether the definition of predicted relative L3-RSRP accuracy is needed when beam1 and beam2 are from the same cell. In our understanding, for the case that beam1 and beam2 from the same cell doesn’t belong to temproal domain prediction for cell level prediction and it’s spatial domain prediction (case 6 in table) or L3 beam level prediction.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain caseA | 2ndgoal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain caseB | 1stgoal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1stgoal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2ndgoal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1stgoal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1stgoal | Intra-cell |

Proposal 3: For temproal domain cell level prediction, no need to consider predicted relative L3-RSRP accuracy when beam1 and beam2 are from the same cell.

Proposal 4: If RAN4 consider the relative accuracy for spatial domian or L3 beam level prediction, predicted relative L3-RSRP accuracy when beam1 and beam2 are from the same cell is needed.

For the second issue, whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. For the definition of relative RSRP, it involves two RSRP measurements as references. The legacy relative L3-RSRP accuracy requirement is defined as the delta between the measured L3-RSRP of the serving cell and the measured L3-RSRP of the neighboring cell. Following the same definition, the relative prediction accuracy would be expressed as the delta between the predicted RSRP of the cell 1 and the predicted RSRP of the cell 2. However, we should consider the case that cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2. For this issue, there are two cells considered in the definition and we should discuss case by case.

If both cell1 and cell2 are the target cells for AI/ML based L3-RSRP inference, reported results are predicted results. If cell1 is target cell and UE has the actual measurement results of cell2, which means cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, the reported RSRP of cell2 can be measured. In our view, if we just consider frequency domain prediction, the reported RSRP of cell2 should be measured result. If we combine the measurement prediction on temporal domain and frequency domain, the reported RSRP of cell2 can be predicted or measured. In this phase, we prefer to study the simpler case and not to consider the combined prediction across multiple domains.

Proposal 5: For the case that cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, the reported RSRP of cell2 should be measured results.

Based on the RAN2’s agreement, we should define the prediction accuray of the cell-level measurement prediction, and RSRP difference to the actual measurement should be defined. Meanwhile, measurement reduction rate is also be considered as one KPI and it can be depotitized compared to measurement prediction accuracy.

RAN2#126 => Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases => Definition of measurement reduction for intra-frequency scenario is defined as:(20/20) - Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: - MRRT= skipped measurement time instances / total measurement time instances - Measurement reduction rate in spatial domain (MRRS): - MRRS = skipped beams to be measured/ total beams to be measured

RAN2 has defined measurement reduction for intra-frequency scenario, including MRRT and MRRS. Measurement reduction rate in temporal domain (MRRT) is the ratio between skipped measurement time instances and total measurement time instances, and measurement reduction rate in spatial domain (MRRS) is tha ratio between skipped beams to be measured and total beams to be measured. In our understanding, these two indicators can be used to evaluate the benefits brought by AI models in mobilioty.

Proposal 6: RAN4 should study the RRM impact of MRRT and MRRS, which involve the number of skipped measurement instances and total measurement instances.

For RRM measurement prediction, we should discuss scenarios 2, 3, 4 for cell level measurement prediction. For intra-frequency temporal domain prediction, both caseA and caseB, it can observed that UE will skip some measurement occasions, which means that only subset of measurement occasions are measured so that SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy.

Intra-frequency temporal domain case A: In case A, continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding. Example of temporal domain case A Intra-frequency temporal domain case B-To be discussed in Phase 2: In case B, measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) after measurement result(s) in previous prediction window is/are skipped. Example 1 of intra-frequency temporal domain case B Example 2 of intra-frequency temporal domain case B

For inter-frequency inter-cell (frequency domain), not all configured MOs need to be measurement, just need to consider the actually measurement frequency layers when we define measurement delay requirements. Thus, the CSSF assumptions need to be updated.

Proposal 7: Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are different, which will impact measurement requirements. And RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML monility:

Observation1: Both cell level measurement prediction and beam level measurement prediction are within the scope of RRM measurement prediction. And according to RAN2’s agreements, L3 filtered beam level prediction cases are lower priority and companies should focus on FR2-to-FR2 intra-frequency temporal domain prediction caseA (scenario 4).

Proposal 1：RAN4 should study the impacts on requirements for L3 cell-level prediction first and the prediction accuracy for L3 beam-level prediction can use the conclusion of L3 cell level as baseline.

Proposal 2: RAN4 should discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases.

Proposal 3: For temproal domain cell level prediction, no need to consider predicted relative L3-RSRP accuracy when beam1 and beam2 are from the same cell.

Proposal 4: If RAN4 consider the relative accuracy for spatial domian or L3 beam level prediction, predicted relative L3-RSRP accuracy when beam1 and beam2 are from the same cell is needed.

Proposal 5: For the case that cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, the reported RSRP of cell2 should be measured results.

Proposal 6: RAN4 should study the RRM impact of MRRT and MRRS, which involve the number of skipped measurement instances and total measurement instances.

Proposal 7: Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are different, which will impact measurement requirements. And RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR , OPPO (rapporteur)

- R4-2504901, WF on RRM requirements for FS_NR_AIML_Mob, Nokia