3GPP TSG-RAN WG4 Meeting # 115                                    R4-2506568

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.29.3.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on CSI-RS based L1-RSRP measurement

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to L3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we will propose our views on RRM requirement for CSI-RS based L1-RSRP measurement.

Discussion

In previous meeting, we have discussed the RRM scope of CSI-RS based L1-RSRP measurement and reached consensus that RAN4 should discuss at least measurement delay, measurement reporting requirements, measurement restrictions and scheduling availability for CSI-RS based L1-RSRP measurement on LTM candidate cells, which is considered in high level and same as the procedure of study on SSB based L1-RSRP measurement on candidate cells.

- Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 Agreement: - Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting. - Further discuss whether step 2 can be skipped for serving cell or not.

In the last meeting, we discussed the measurement procedure for CSI-RS based L1 measurement and reached consensus that step2 can be skipped for neighbor based on new UE capability reporting. If step 2 is not executed and step 3 is directly executed, the Rx beam used for CSI-RS measurement may be different from the Rx beam measured for SSB measurement. Correspondingly, the best Rx beam corresponding to the Tx beam will be selected, and the received and transmitted beam pair will be more matched. However, the lost of the finer measurements is long measurement delay, N=8 times maybe extended for worst case.

In this meeting, we should discuss whether step2 can be skipped for serving cell or not. In legacy for CSI-RS L1-RSRP measurement requirements, the CSI-RS resource for L1 measurement shall be QCL-D with SSB index for L1 measurement. Similar as neighboring cell, if step2 can be skipped, longer measurement period should be considered for serving cell, and we are not sure if the gain obtained by skipping step 2 is worth extending the delay. Meanwhile, we have a question about whether the modification of the measurement procedure for serving cells within the scope of this WI.

Proposal 1: Slightly prefer to keep step2 about SSB-based L1 measurements and reports on serving cell.

In the last meeting, we have discussed the definiation of intra-f and inter-f measurement for CSI-RS based L1 RSRP measurement. And there are two options can be chosen.

- Issue 4-4: CSI-RS based intra-frequency/inter-frequency measurement definition Candidate options: - Option 1: Apple, CMCC A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: MediaTek, ZTE, Ericsson, Nokia, Qualcomm A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP. - Option 3: Huawei If L1 CSI-RS RSRP measurement with gap is not supported, either option of intra-frequency CSI-RS based L1 RSRP measurement definition has no big difference. If CSI-RS measurement with gap is supported, to define measurement requirements, the “CSI-RS frequency layer” concept will be touched. Then the intra-frequency definition based on center frequency is more preferred. - Option 4: vivo - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

For option1, it is samilar as the definition of intra-frequency/inter-frequency CSI-RS based L3 measurement, which is aglin with the previous discussion for measurement. Of course, there are some differences with CSI-RS based L3 measurement. The main reason is the different configuration methods of CSI-RS, one is to follow CSI-RS configuration for MO (MeasObjectNR), the other one is to follow NZP-CSI-RS-Resource structure. In order to address the issue that different CSI-RS resources on the serving cell may have different SCS and CP types, active DL BWP is used instead of "serving cell" in legacy L3 measurement. Meanwhile, regarding center frequency, any of the CSI-RS resources of the serving cell indicated for L1 measurement is used for judgment. In our opinion, such modifications can solve the problem of L1 and L3 mismatch based on the definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement.

For option2, the different from option1 is focus on bullet3. Regardless of whether the center frequency is aligned, only focus on whether the CSI-RS resources are included in the active DL BWP. In our understanding, it is because the difference between intra-f measurement and inter-f measurement mainly lies in whether a gap is needed during measurement. If the measurement signal is fully included in the active DL BWP, gap is not required for measurement. Conversely, if it is not included, the use of gap needs to be considered.

As to how to define CSI-RS resource is included within the active DL BWP, option2 mentions that at least 48 RBs of the CSI-RS resource need to be confined within the active DL BWP. Based on the previous agreement, define the measurement period and accuracy requirements based on Density=3 with (≥) 48 PRBs for CSI-RS based L1-RSRP measurement on neighbor cell. Therefore, the 48RB CSI-RS is the minimum value we define for requirements and require at least 48 PRBs to be included in the active BWP. Based on the analysis above, we can compromise to option2 which not consider center frequency but within active BWP.

Proposal 2: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- It is applied for SCS = 60KHz

- the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

- Issue 4-8: Restriction on CSI-RS resource configuration Candidate options: - Option 1: Ericsson - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms.

- This issue is about the restriction on CSI-RS resource configuration and some condition is added base on the agreement we have in RAN4#114 meeting [2].

- Agreement on RAN4#114

Issue 4-10: Restriction on CSI-RS resource configuration

Agreement:

- In FR1, when CSI-RS based L1 measurement does not cause scheduling restriction, no need to consider restriction on measurement timing configuration.

- In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction:

  - All CSI-RS resources on [one intra-frequency layer] are configured within up to two separate windows where each window is up to [5 ms]

- Compared to the initial agreement, peoponent added “whithin a 40ms window” here. In our understanding, we have the description that All CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms for CSI-RS based L3 measurement, and the periodicities and offsets of CSI-RS based L3-RSRP and CSI-RS for L1-RSRP are different. As mentioned by other company, the maximum periodicity for L3-RSRP is 40 ms and the maximum periodicity for L1-RSRP is 640 slots.

- That is to say, in L3 measurement, for periods and offsets that do not exceed 40ms, there can be up to two 5ms windows. So if we want to use the same restriction for L1 measurement, we understand that this 40ms constraint is necessary because 640 slots are not always less than or equal to 40ms.

- Proposal 3: In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms

Conclusions

In this contribution, we put forward the following proposals on R19 L1/L2 triggered mobility.

Proposal 1: Slightly prefer to keep step2 about SSB-based L1 measurements and reports on serving cell.

Proposal 2: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- It is applied for SCS = 60KHz

- the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

Proposal 3: In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple