3GPP TSG-RAN WG4 Meeting # 115                                   R4-2506569

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.29.3.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Event triggered L1 measurement reporting

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to layer3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we will propose our views on RRM requirement for event triggered L1 reporting.

Discussion

According to the WID, RAN4 should discuss the impact on RRM requirements depends on the RAN1/2 specification introduced for Event triggered L1 measurement reporting. In the pervious meetings, we have reached consensus on Event-triggered L1 reporting delay requirement and accuracy requirement from high level and detail should be discussed.

The RRM impact due to event triggered L1 measurement reporting could include the followings:

- Capabilities for Support of Event Triggering and Reporting Criteria

- Other potential impact of event-triggered L1 reporting

Below, we will express our views on the above issues.

- Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Yes (Xiaomi, CMCC, ZTE, CTC, vivo) - Option 2: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple) - Issue 2-4: whether to define a separate capability for supported number of events triggered L1 measurement reporting Candidate options: - Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. (Xiaomi, CMCC, ZTE, E///, CTC) - Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///) - Option 2: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO)

For the first bullet, some companies proposed that requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell in pervious meeting. In our understanding, it is necessary to introduce or update the requirements of Capabilities for Support of Event Triggering and Reporting Criteria specified in TS38.133, which is similar as legacy event triggered L3 measurement reporting. It’s commom understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. Event triggering L1 reporting for LTM maybe not support if there is no requirements for reporting criteria per measurement category. In addition, the number of events to be supported by UE for LTM shall be discussed in RAN1 first.

Proposal 1: The requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS 38.133 need to be updated to cover event triggered L1 measurement reporting.

As for how to define relevant capabilities to support of event triggering L1 measurement reporting, there are two intuitive methods. As mentioned in last meeting by some companies, compare to leagcy capability for events triggered measurement reporting, same or separate capability can be considered for supported number of events triggered L1 measurement reporting. If using same capability as legacy, Ecat. should be updated to cove L3 and L1 events due to the number of events is too limited. In addition, the number of events to be supported by UE for LTM shall be discussed in RAN1 first. To avoid affecting L3 measurement reports, we prefer to define a separate capability for the number of L1 measurement reports triggered by supported events.

proposal 2: RAN4 should define a separate capability for supported number of events triggered L1 measurement reporting.

- Issue 2-7: Impact on other LTM requirements Agreement: - For FR1: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, - Case 1: leaving condition of the event is configured and corresponding condition is not met, - the related cell switch delay and TCI state activation requirements are applicable - Case 2: leaving condition of the event is NOT configured, FFS - For FR2: FFS

In this issue, event-triggered L1 report is considered to reduce the cell switch delay or TCI state activation delay. It’s benefical to extend the cell switch delay and TCI state activation requirements to more reasonable scenarios. For FR1, UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, if leaving condition of the event is configured and corresponding condition is not met, it means the measurement is not stopped utill receiving command and the reported TCI state is known. Meanwhile, if the leaving condition of the event is NOT configured, there maybe no measurement after event-triggered L1 report. However, according to the known conditions for TCI state, the TCI state is considered as known during 1280ms after event-triggered L1 report.

For FR2, some company mentioned in the last meeting that UE will prioritize L1-RSRP measurement on those TCI state(s) and may never measure the beam which is not activated if NW activates some TCI state(s). So UE may stop measuring the beam if NW activates some other TCI state(s) after event-triggered L1 report. In this case, the report TCI state maybe unknown and the related cell switch delay and TCI state activation requirements can not applicable. Therefore, same as the leaving condition of event is NOT configured in FR1, the TCI state is considered as known during 1280ms after event-triggered L1 report.

Proposal 3: For both FR1 and FR2, the TCI state is considered as known during 1280ms after event-triggered L1 report. And the related cell switch delay and TCI state activation requirements are applicable.

Conclusions

In this contribution, we put forward the following proposals on R19 L1/L2 triggered mobility.

Proposal 1: The requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS38.133 need to be updated to cover event triggered L1 measurement reporting.

proposal 2: RAN4 should define a separate capability for supported number of events triggered L1 measurement reporting.

Proposal 3: For both FR1 and FR2, the TCI state is considered as known during 1280ms after event-triggered L1 report. And the related cell switch delay and TCI state activation requirements are applicable.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple