3GPP TSG-RAN WG4 Meeting # 115                                   R4-2506570

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.29.4

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on performance requirements for LTM

Document for:	Approval

Introduction

The RRM core requirement on Mobility work item has been summarized, which include

- Event triggered L1 measurement reporting

- CSI-RS measurements for LTM procedures

- (subsequent) conditional LTM

This contribution provides our view on performance requirements for R19 mobility enhancement.

Discussion

RAN4#115 is the first meeting for performance requirements on mobility and we need to discuss relative issues on performance part. Based on the current status of each objective, it is possible to evaluate the test case need at least on high level. In this contribution, we provide our view based on core part.

2.1.L1-RSRP accuracy

For L1-RSRP measurement requirements, RAN4 should discuss in performance part for event triggered L1 measurement reporting and CSI-RS based L1 measurement. In previous meeting, we have an agreement on event triggered L1 measurement reporting:

RAN4#112bis

- Agreement:

  - for SSB based L1 RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R18 LTM L1-RSRP accuracy requirements

  - for CSI-RS based L1-RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R19 CSI-RS based L1-RSRP accuracy requirements for LTM, which will be discussed under CSI-RS based L1-RSRP measurement agenda.

We can observed from above that LTM L1-RSRP accuracy requirements are reused for L1 RSRP event triggered reporting, including SSB based and CSI-RS based. For SSB based, we have discussed L1-RSRP accuracy requirements in R18 and can be reused directly. However, for CSI-RS based, there is no conclusion on L1-RSRP accuracy for neighbor cell, we should discuss accuracy and side condition for CSI-RS based L1-RSRP measurement first.

As to the accuracy requirement for CSI-RS based L1-RSRP measurement, we prefer to reuse the requirements of the serving cell. That is to say, it is proposed to use legacy side condition of SINR=-3dB for CSI-RS based L1 measurement for neighbor cell.

Proposal 1: CSI-RS based L1-RSRP accuracy of the serving cell can be applied for CSI-RS based L1-RSRP of the candidate neighbour cells.

both intra-frequency measurement and inter-frequency measurement are in scope. However, the R17 existing accuracy for L1-RSRP measurements are defined for the intra-frequency L1-RSRP measurement.

In the last meeting, we have reached consensus that no need to define requirement for L1 measurement with gap. Therefore, there is no need to define accuracy requirements for inter-frequency L1-RSRP measurement with gap. Meanwhile, the LTM L1-RSRP reporting scheme reuses the differential scheme which is similar as legacy L1-RSRP reporting. So the absolute and relative accuracy for intra-frequency is needed.

Proposal 2: Define both absolute and relative accuracy requirements for LTM intra-frequency L1-RSRP accuracy requirements.

2.2 Test coverage

For LTM, there are a lot of core requirements defined and there are various scenarios that may have different requirements.

It’s fine for us to define a test case to cover multiple scenarios and avoiding duplicated testing on similar functions. First, we should discuss which cases can be specified. In our understanding, as per current progress, the following test cases can be specified:

2.2.1 Event triggered L1 measurement reporting

For this part, we can use the test case for L3 event triggered reporting as baseline and further discuss whether it is needed or not for L1 event triggered reporting. In current spec, there are many test cases for intra-f in clause A6.6 and inter-f in clause A6.7. We have summaried in follow table.

| A.6.6.1.1 | SA event triggered reporting tests without gap under non-DRX |
| --- | --- |
| A.6.6.1.2 | SA event triggered reporting tests without gap under DRX |
| A.6.6.1.3 | SA event triggered reporting tests with per-UE gaps under non-DRX |
| A.6.6.1.4 | SA event triggered reporting tests with per-UE gaps under DRX |
| A.6.6.1.5 | SA event triggered reporting tests without gap under non-DRX with SSB index reading |
| A.6.6.1.6 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading |
|  | ... |
| A.6.6.2.1 | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used |
| A.6.6.2.2 | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used |
| A.6.6.2.5 | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used |
| A.6.6.2.6 | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used |
| A.6.6.2.10 | SA event triggered reporting tests for FR1 when DRX is used |
| A.6.6.2.11 | SA event triggered reporting tests for FR1 without gap when DRX is not used |
|  | ... |

In our inderstanding, for SSB based L1-RSRP measurement, we considered three scenarios that intra-f measurement without gap, inter-f measurement with type1 gap and inter-f measuremnt without gap. Meanwhile, with or without SSB index reading also considered. Therefore, we should at least consider follow secnarios when define test cases:

- With gap &without gap, intra-f & inter-f

- non-DRX & DRX

- With SSB index reading & without SSB index reading

Proposal 3: define test cases for L1 RSRP event triggered reporting and follow aspects should be considered:

- With gap &without gap, intra-f & inter-f

- non-DRX & DRX

- With SSB index reading & without SSB index reading

- Other aspects are not precluded.

2.2.2 CSI-RS based measurement

For SSB based L1-RSRP measurement, we defined several test cases, including intra-frequency measurement, inter-frequency measurement with gap and inter-frequency without gap for both FR1 and FR2. In addition, test cases for inter-frequency L1-RSRP accyracy requirements. For CSI-RS based L1-RSRP measurement, based on agreements in core part, we don’t need to consider the test cases for measurement with gap. Meanwhile, the definiation of intra-frequency is still inder discussion, whether the test case for accuracy is needed or not should wait the conclusion on this issue.

| A.6.6.26.1 | Intra-frequency SSB based L1-RSRP measurement in FR1 |
| --- | --- |
| A.6.6.27.1 | Inter-frequency SSB based L1-RSRP measurement with measurement gap |
| A.6.6.28.1 | Inter-frequency SSB based L1-RSRP measurement without measurement gap |
| A.6.7.17.1 | Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1 |
| A.7.6.20.1 | Intra-frequency SSB based L1-RSRP measurement in FR2 |
| A.7.6.21.1 | Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM |
| A.7.6.22.1 | Inter-frequency SSB based L1-RSRP measurement without measurement gap in FR2 |
| A.7.7.15.1 | Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1 |

Proposal 4: define test cases for Intra-frequency SSB based L1-RSRP measurement in FR1 and FR2.

2.2.3 (subsequent) conditional LTM

- For (subsequent) conditional LTM, we should define test cases based on R18 LTM. In R18 LTM,we defined the test cases for RACH-based/RACH-less Intra-frequency/inter-frequency PCell/PSCell switch from FR1 to FR1 or FR2 to FR2. during Release 18, we have done the down selection when designed the test cases. So, we can use the test list in R18 as baseline when we discuss the tst case for conditional LTM.

| A.6.3.4.1 | RACH-based Intra-frequency PCell switch from FR1 to FR1 |
| --- | --- |
| A.6.3.4.2 | RACH based Inter-frequency LTM PCell switch from FR1 to FR1 |
| A.6.3.4.3 | RACH-less Intra-frequency PCell switch from FR1 to FR1 |
| A.6.3.4.4 | RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement |
| A.6.3.5.1 | RACH-based intra-frequency LTM PSCell switch from FR1 to FR1 |
| A.7.3.4.1 | RACH based Intra-frequency cell switch from FR2 to FR2 |
| A.7.3.4.2 | RACH-less Intra-frequency PCell switch from FR2 to FR2 |
| A.7.3.4.3 | RACH-less Inter-frequency LTM PCell switch from FR2 to FR2 |
| A.7.3.5.1 | Intra-frequency RACH-based LTM PSCell switch from FR2 to FR2 |
| A.6.3.2.4 | LTM PDCCH-order Random Access |
| A.7.3.2.4 | LTM PDCCH-order Random Access |

Proposal 5: Define test cases for (subsequent) conditional LTM and test cases defined for R18 LTM cell switch can be used as baseline. Follow aspects should be considered:

- RACH-based & RACH-less

- FR1 to FR1& FR2 to FR2

- Intra-f & inter-f

- PCell switch & PScell switch

- without L1-RSRP measurement for FR1

Other aspects are not precluded.

- For PDCCH-order random access, we have an agreement in RAN4#114 meeting that no need to change the existing TCI state activation delay requirements based on the current RAN2 agreements, which means the same procedure is used for LTM and CLTM. Therefore, there is no need for repeated testing.

- Proposal 6: For (subsequent) CLTM, there is no need to define test cases for PDCCH-order Random Access.

Conclusions

In this contribution, we put forward the following proposal on performance part for mobility enhancements.

Proposal 1: CSI-RS based L1-RSRP accuracy of the serving cell can be applied for CSI-RS based L1-RSRP of the candidate neighbour cells.

Proposal 2: Define both absolute and relative accuracy requirements for LTM intra-frequency L1-RSRP accuracy requirements.

Proposal 3: define test cases for L1 RSRP event triggered reporting and follow aspects should be considered:

- With gap &without gap, intra-f & inter-f

- non-DRX & DRX

- With SSB index reading & without SSB index reading

- Other aspects are not precluded.

Proposal 4: define test cases for Intra-frequency SSB based L1-RSRP measurement in FR1 and FR2.

Proposal 5: Define test cases for (subsequent) conditional LTM and test cases defined for R18 LTM cell switch can be used as baseline. Follow aspects should be considered:

- RACH-based & RACH-less

- FR1 to FR1& FR2 to FR2

- Intra-f & inter-f

- PCell switch & PScell switch

- without L1-RSRP measurement for FR1

Other aspects are not precluded.

- Proposal 6: For (subsequent) CLTM, there is no need to define test cases for PDCCH-order Random Access.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom