3GPP TSG-RAN4 Meeting #115	R4-2506575

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5531 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on test cases for NR channel BW less than 5MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW_Ph2-Perf |  |  |  |  |  | Date: |  |  | 2025-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | correction on test cases for NR channel BW less than 5MHz |  |  |  |  |  |  |  |  |
| Summary of change: |  | correction on test cases for NR channel BW less than 5MHz based on endorsed bigCR R4-2504974. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements maybe incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.3.1.X, A.6.5.3.X, A6.5.11.X, A.6.5.12.X1, A.6.5.12.X2, A.6.6.9.X.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change1>>

#### A.6.3.1.X	Handover with PSCell change delay where target PSCell is with 12PRB SSB bandwidth

##### A.6.3.1.X.1	Test Purpose and Environment

The purpose of this test is to verify the handover delay requirements and PSCell change delay requirements in HO with PSCell from NR-DC (FR1-FR1) to NR-DC (FR1-FR1) defined in clauses 6.1.5.4. The requirements are applicable to NR FR1-FR1 intra-frequency PCell handover and NR FR1-FR1 intra-frequency PSCell change. The requirements are only applicable when the target PSCell is configured with 12PRB SSB bandwidth.

##### A.6.3.1.X.2	Test Parameters

The supported test configurations are given in table A.6.3.1.X.2-1. Only 15kHz FDD cases are considered. The test scenario comprises 3 NR cells, source PCell(Cell 1), target PCell(Cell 2), target PSCell(Cell 3).

Cell 1 and Cell 2 are on radio channel 1 in FR1. Cell 3 is on radio channel 2 in FR1. Test parameters are given in Tables A.6.3.1.X.2-2, and A.6.3.1.X.2-3 below. Note that for Cell 3 the SSB configuration refers to SSB pattern 13 in FR1: SSB allocation for SSB SCS=15kHz in 3 MHz. In the test, the SSB is configured with 12PRB bandwidth.

The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of T1, the UE shall be connected to Cell 1 on radio channel 1. UE is not aware of Cell 2 and Cell 3. Starting T2, Cell 2 and Cell 3 becomes detectable to the UE and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

The UE is required to successfully handover from Cell 1 to Cell 2 and at the same time in time period T2 add Cell 3 as PSCell attached to Cell 2. This test case verifies the delay starting from the start of T2 to time points where the UE finishes handover to Cell 2 and where the UE finishes addition of Cell 3. Test cases delay requirements are specified.

Table A.6.3.1.X.2-1: Supported test configurations for HO with PSCell with 12PRB SSB bandwidth

| Config | Description |
| --- | --- |
| 1 | Source PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target PSCell: FR1 NR 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |

Table A.6.3.1.X.2-2: Cell specific test parameters for Intra-frequency handover

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 15 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 15 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -98 |  |  |  |
|  |  |  | dB | 8 | 8 | -Infinity | 12 |
|  |  |  | dB | 8 | 8 | -Infinity | 12 |
| SSB_RP | Config 1 |  | dBm/SCS | -90 | -90 | - Infinity | -93 |
| IoNote3 | Config 1 |  | dBm/ 9.36 MHz | -61.41 | -61.41 | - Infinity | -57.78 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.6.3.1.X.2-3: Cell specific test parameters for Cell 3

| Parameter |  |  | Unit | Cell 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| NR RF Channel Number |  |  |  | 2 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
| BWchannel |  | Config 1 | MHz | 3: NPRB,c = 15 |  |
| BWP BW |  | Config 1 | MHz | 3: NPRB,c = 15 |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |
| DRX Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.2 FDD |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.2 FDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  | Config 1 |  | SSB.13 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 15 kHz |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 15 kHz |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1 |  | dBm/SCS | -98 |  |
|  |  |  | dB | -Infinity | 12 |
|  |  |  | dB | -Infinity | 12 |
| SSB_RP | Config 1 |  | dBm/SCS | -Infinity | -86 |
| IoNote3 | Config 1 |  | dBm/ SCS MHz | -Infinity | -85.7 |
| Propagation condition |  |  | - | AWGN | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

##### A.6.3.1.X.3	Test Requirements

Handover to PCell delay requirements

In this test, the UE shall start to transmit the PRACH to target PCell (Cell 2) less than [78] ms from the beginning of time period T2.

The handover delay is defined as

DHOwithPSCell_PCell = RRC procedure delay + Tinterrupt, where

Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin

The UE is required to follow the below parameters according to derivation from the test setup specified:

- RRC procedure delay = 16 ms,

- Tsearch = 20 ms,

- TIU = 20 ms where PRACH association period should be configured as 1,

- Tprocessing = 20 ms,

- Tmargin = 2ms.

That sums up to 78ms exactly.

PSCell change/addition delay requirements

The UE shall transmit the PRACH to target PSCell (Cell 3) no later than [178] ms from the beginning of time period T2.

PSCell change delay during handover is defined as

DHOwithPSCell_PSCell = TRRC_delay + Tprocessing + Tsearch_PCell + Tsearch_PSCell + T∆ + TPSCell_ DU + 2 ms.

In this test the UE is required to follow the below parameters according to derivation from the test setup specified:

- TRRC_delay = 16ms,

- Tprocessing = 20 ms,

- Tsearch_PCell = 0,

- Tsearch_PSCell = 60 ms,

- T∆ = 60 ms,

- TPSCell_ DU = 1*10+10 = 20 ms where PRACH association period should be configured as 1.

That sums up to 178ms exactly.

The test is considered successful only when both requirements are correctly verified, and the success rate during repeated tests shall be at least 90 %.

<<End of Change1>>

<<Start of Change2>>

### A.6.5.3	SCell Activation and Deactivation Delay

#### A.6.5.3.X	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle with less than 5MHz BW

##### A.6.5.3.X.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation, and the SCell operate with a bandwidth of less than 5 MHz. Supported test configurations are specified in Table A.6.5.3.X.1-1 below. General test parameters for known Scell activation case is specified in Table A.6.5.3.1.1-2. Supported Cell specific test parameters for NR PCell specified in Table A.6.5.3.1.1-3. Supported Cell specific test parameters for NR SCell specified in Table A.6.5.3.1.1-4 apply except those specified in Table A.6.5.3.X.1-2.

The test procedure specified in A.6.5.3.1 applies to this test.

Table A.6.5.3.X.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | PCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode SCell: NR 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |

Table A.6.5.3.X.1-2: Cell specific test parameters for NR SCell

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| BWchannel | ConfigSCell 1 | MHz | 3 MHz |  |  |
| BWoccupied | ConfigSCell 1 | RB | 12 |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | SR.1.2 FDD |  |  |
| SSB Configuration | ConfigSCell 1 |  | SSB.13 FR1 |  |  |
| Io Note | ConfigSCell 1 | dBm/ 2.7 MHz | -60.31 | -58.58 | -58.58 |
| NOTE : Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

##### A.6.5.3.X.2	Test Requirements

Test requirements specified in Clause A.6.5.3.1.2 apply to this test except Tactivation_time will be replaced with TFirstSSB+ T∆ + 5ms.

<<End of Change2>>

<<Start of Change3>>

### A.6.5.11	PSCell addition and release delay

#### A.6.5.11.X	Addition and Release Delay of unknown NR FR1 PSCell with less than 5 MHz

##### A.6.5.11.X.1	Test purpose and environment

The purpose of this test is to verify that the NR PSCell addition and release delay under NR-DC with a bandwidth of less than 5 MHz meets the requirements defined in Clauses 8.9.2 and 8.9.3, where the PSCell is unknown to the UE at the time of addition.

Supported test configurations are shown in Table A.6.5.11.X.1-1. The test scenario comprises two NR cells, Cell 1 and Cell 2, on radio channel 1 and radio channel 2 in FR1, respectively. Test parameters are given in Table A.6.5.11.1.1-2. The cell-specific parameters as specified in Table A.6.5.11.1-3 with config 1 apply except those specified in Table A.6.5.11.X.1-2.

The test procedure specified in A.6.5.11.1 applies to this test.

Table A.6.5.11.X.1-1: Supported test configurations for FR1 PSCell Addition and Release with less than 5 MHz

| Configuration | Description |
| --- | --- |
| 1 | NR SCS 15 kHz, BW 3 MHz (12 PRB), FDD |

Table A.6.5.11.X.1-2: Cell Specific Parameters for FR1 PSCell Addition and Release with less than 5 MHz

| Parameter | Unit | Config | Cell 1 | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T3 | T4 |
| BWchannel | MHz | 1 |  | 3: NPRB,c = 12 |  |  |  |  |
| Data PRBs allocated |  | 1 |  | 12 |  |  |  |  |
| PDSCH Reference Measurement Channel |  | 1 |  | SR.1.2 FDD |  |  |  |  |
| RMSI CORESET parameters |  | 1 |  | CR.1.2 FDD |  |  |  |  |
| Dedicated CORESET parameters |  | 1 |  | CCR.1.6 FDD |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1 |  | 15 |  |  |  |  |
| SSB configuration |  | 1 |  | SSB.13 FR1 |  |  |  |  |
| IoNote | dBm/2.7MHz | 1 |  | N/A |  | –63.4 dBm |  |  |
| NOTE : Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.5.11.X.2	Test Requirements

The UE shall transmit the PRACH preamble to PSCell no later than 212ms  with 12 PRB SSB bandwidth. Otherwise 172 msNote1 from the start of T2.

The UE shall send at least one CSI report for PSCell with non-zero CQI index during T3.

The UE shall periodically send CSI reports for PSCell after the UE has sent first CQI report with non-zero CQI index during T3.

The UE shall stop sending CSI reports for PSCell no later than 20 ms from the start of T4.

All the above test requirements shall be fulfilled in order for the observed PSCell addition delay and PSCell release delay to be counted as correct. The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90 %.

Note1:	The PSCell addition delay can be expressed as follows as specified in clause 8.9.2:

Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + 3* T∆ + TPSCell_ DU + 2 ms with 12 PRB SSB bandwidth.

Otherwise, Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms

Where:

TRRC_delay = 50 ms

Tprocessing = 20 ms

Tsearch 	= 60 ms

T∆ = 20 ms

TPSCell_ DU = 1*10+10 = 20 ms

<<End of Change3>>

<<Start of Change4>>

#### A.6.5.12.X1	Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth

##### A.6.5.12.X1.1	Test purpose and environment

The purpose of this test is to verify that the subsequent conditional NR PSCell change with 12 PRB SSB bandwidth under NR-DC is within the requirements stated in clause 8.11E.2.

For UE supporting subsequent conditional PSCell addition/change, UE only needs to pass one of addition and release delay of PSCell with 12 PRB SSB bandwidth defined in clause A.6.5.11.X and  intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth defined in clause A.6.5.12.X1 and inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth defined in clause A.6.5.12.X2 and handover with PSCell delay defined in clause A.6.3.1.X3.

##### A.6.5.12.X1.2	Test Parameters

Supported test configurations are shown in table A.6.5.12.X1.2-1. The test scenario comprises three NR cells, Cell 1, Cell 2 and Cell 3. Cell1 is on radio channel 1 in FR1. Cell 2 and 3 are on radio channel 2 in FR1. General test parameters as specified in table A.6.5.12.1.2-2 with config 1 apply except those specified in table A.6.5.12.X1.2-2. Cell specific test parameters as specified in table A.6.5.12.1.2-3 with config 1 apply except those specified in table A.6.5.12.X1.2-3.

The test procedure specified in clause A.6.5.12.1.2 applies to this test. The Cell 3 is the target cell operating with 12 PRB SSB bandwidth.

Table A.6.5.12.X1.2-1: Supported test configurations for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode. |

Table A.6.5.12.X1.2-2: General Test Parameters for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2, Cell 3 | Neighbour cells with 12 PRB SSB bandwidth on RF channel number 2. |
| Final | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Condition | active PSCell |  | Cell 3 | PSCell with 12 PRB SSB bandwidth on RF channel number 2. |
|  | Neighbour cell |  | Cell2 | Neighbour cell with 12 PRB SSB bandwidth on RF channel number 2. |

Table A.6.5.12.X1.2-3: Cell Specific Parameters for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter | Unit | Test configuration | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode |  | Config 1 | FDD |  |  |  |  |  |
| BWchannel | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |  |  |
| BWP BW | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 | SR.1.2 FDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 | CR.1.2 FDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 | CCR.1.6 FDD |  |  |  |  |  |
| SSB parameters |  | Config 1 | SSB.13 FR1 |  |  |  |  |  |
| IoNote1 | dBm/3 MHz | Config 1 | -75.45 | -66.81 | -62.46 | -75.45 | -66.81 | -62.46 |
| Propagation Condition |  | Config 1 | AWGN |  |  | AWGN |  |  |
| NOTE 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.5.12.X1.3	Test Requirements

TRRC_delay + TEvent_DU for PSCell addition (Cell 2) occurs during T1 as the addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 920+10+102 ms=1032 ms from the start of T2.

The UE shall start to transmit the PRACH to Cell 3 less than TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 0+920+10+102 ms=1032 ms from the start of T3.

All of the above test requirements shall be fulfilled in order for the observed conditional PSCell addition and release delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90 %.

<end of the change1>

<start of the change2>

#### A.6.5.12.X2	Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth

##### A.6.5.12.X2.1	Test purpose and environment

The purpose of this test is to verify that the subsequent conditional NR PSCell addition with 12 PRB SSB bandwidth under NR-DC is within the requirements stated in clause 8.9C.2.

For UE supporting subsequent conditional PSCell addition/change, UE only needs to pass one of addition and release delay of PSCell with 12 PRB SSB bandwidth defined in clause A.6.5.11.X and intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth in clause A.6.5.12.X1 and inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth defined in clause A.6.5.12.X2 and handover with PSCell delay defined in clause A.6.3.1.X3.

##### A.6.5.12.X2.2	Test Parameters

Supported test configurations are shown in table A.6.5.12.X2.2-1. The test scenario comprises three NR cells, Cell 1, Cell 2 and Cell 3. Cell 1 is on radio channel 1 in FR1. Cell 2 is on radio channel 2 in FR1. Cell 3 is on radio channel 3 in FR1.

General test parameters as specified in table A.6.5.12.2.2-2 with config 1 apply except those specified in table A.6.5.12.X2.2-2. Cell specific test parameters as specified in table A.6.5.12.2.2-3 with config 1 apply except those specified in table A.6.5.12.X2.2-3.

The test procedure specified in clause A.6.5.12.2.2 applies to this test. The Cell 3 is the target cell operating with 12 PRB SSB bandwidth.

Table A.6.5.12.X2.2-1: Supported test configurations for Inter-frequency Subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode. |

Table A.6.5.12.X2.2-2: General Test Parameters for Subsequent Conditional PSCell Addition and Release

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2 | Neighbour cell with 12 PRB SSB bandwidth on RF channel number 2. |
|  |  |  | Cell 3 | Neighbour cell with 12 PRB SSB bandwidth on RF channel number 3. |
| Final | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Condition | active PSCell |  | Cell 3 | PSCell with 12 PRB SSB bandwidth on RF channel number 3. |
|  | Neighbour Cell |  | Cell 2 | Neighbour cell with 12 PRB SSB bandwidth on RF channel number 2 |

Table A.6.5.12.X2.2-3: Cell Specific Parameters for Subsequent Conditional PSCell Addition and Release

| Parameter | Unit | Test configuration | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| Duplex mode |  | Config 1 | FDD |  |  |  |  |  |  |  |
| BWchannel | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |  |  |  |  |
| BWP BW | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 | SR.1.2 FDD |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 | CR.1.2 FDD |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 | CCR.1.6 FDD |  |  |  |  |  |  |  |
| SSB parameters |  | Config 1 | SSB.13 FR1 |  |  |  |  |  |  |  |
| IoNote1 | dBm/3 MHz | Config 1 | -62.4 | -59.4 | -62.4 | -62.4 | -62.4 | -62.4 | -62.4 | -59.4 |
| Propagation Condition |  | Config 1 | AWGN |  |  |  | AWGN |  |  |  |
| NOTE 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

##### A.6.5.12.X2.3	Test Requirements

TRRC_delay + TEvent_DU for PSCell addition (Cell 2) occurs during T1 as the addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 1040+10+102 ms=1152 ms from the start of T2.

The UE shall start to transmit the PRACH to Cell 3 less than TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 0+1040+10+102 ms=1152 ms from the start of T4.

All of the above test requirements shall be fulfilled in order for the observed conditional PSCell addition and release delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90 %.

<<End of Change4>>

<<Start of Change5>>

#### A.6.6.1.X	Deactivated PSCell measurement test with 12 PRB SSB bandwidth in FR1

##### A.6.6.1.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of deactivated PSCell measurement results when the PSCell is configured with 12 PRB SSB bandwidth. This test verifies the deactivated PSCell measurement period requirements specified in clause 9.2.5.1.

##### A.6.6.1.X.2	Test Parameters

The supported test configurations are given in table A.6.6.1.X.2-1. Only 15kHz FDD cases are considered. The test scenario comprises 2 NR cells – PCell (Cell 1) and target PSCell (Cell 2).

Cell 1 is on radio channel 1 in FR1. Cell 2 is on radio channel 2 in FR1. Test parameters are given in Tables A.6.6.1.X.2-2, A.6.6.1.X.2-3 below. Note that for Cell 2 the SSB configuration refers to SSB pattern 13 in FR1: SSB allocation for SSB SCS=15kHz in 3 MHz. In the test, the SSB is configured with 12PRB bandwidth.

The test consists of two successive time periods, with time duration of T1, and T2 respectively.

At the start of T1, the UE is in connected mode to Cell 1. During time duration T1, the UE is configured to add Cell 2 as PSCell so that Cell 1 and Cell 2 serve UE under NR-DC operation. Meanwhile the UE is configured to measure on both Cell 1 and Cell 2 serving carrier frequencies. The UE is configured to read the SSB index of the target deactivated PSCell.

At the end of T1, the UE deactivates Cell 2. T2 starts exactly when the UE starts carrying out deactivated PSCell measurements on Cell 2. During T2, Cell 2 remains detectable to the UE.

Measurement gap or DRX is not configured.

The UE is required to make correct and timely reporting during T2. This test case verifies that the UE reporting is within the delay requirement and measurement accuracy is tested in other test cases.

Table A.6.6.1.X.2-1: Supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, PCell with 10 MHz bandwidth and PSCell with 3 MHz bandwidth, both FDD duplex mode |

Table A.6.6.1.X.2-2: General test parameters for deactivated PSCell measurements when PSCell is configured with 12 PRB SSB bandwidth

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR1 NR carrier frequencies are |
| PCell |  | Config 1 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Deactivated PSCell |  | Config 1 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| measCyclePSCell | ms | Config 1 | 160 | Same as SMTC and SSB periodicity |
| Gap Pattern Id |  | Config 1 | Not configured |  |
| Measurement gap offset |  | Config 1 | Not configured |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 for Cell 1 SSB.13 FR1 for Cell 2 | As specified in clause A.3.10.1 |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 8 |  |

Table A.6.6.1.X.2-3: Cell specific test parameters for deactivated PSCell measurement with 12 PRB SSB bandwidth

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
| BWchannel |  | MHz | Config 1 | 10: NPRB,c = 52 |  | 3: NPRB,c = 15 |  |
| BWP BW |  | MHz | Config 1 | 10: NPRB,c = 52 |  | 3: NPRB,c = 15 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | DLBWP.0.1 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | SR.1.2FDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | CR.1.2 FDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | CCR1.6 FDD |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.13 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 15 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1 | -98 |  | -98 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1 | -94 |  | -91 |  |
|  |  | dB | Config 1 | 4 |  | 7 |  |
|  |  | dB | Config 1 | 4 |  | 7 |  |
| IoNote3 |  | dBm/SCS MHz | Config 1 | -92.6 |  | -90.2 |  |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.1.X.3	Test Requirements

The UE shall send one measurement report for the deactivated PSCell, with a measurement reporting delay less than 5440 ms from the beginning of time period T2.

The UE is required to successfully detect PSS/SSS for Cell 2 within

Ceil(5 x Kp) x measCyclePSCell x CSSFintra;

The UE is required successfully read the SSB index for Cell 2 within

Ceil(7 x Kp) x measCyclePSCell x CSSFintra;

The UE is required to successfully measure on Cell 2 within

Ceil(5 x Kp) x measCyclePSCell x CSSFintra;

Where

- measCyclePSCell = 160 ms,

- Kp = 1,

- CSSFintra = 2.

This sums up as 5440 ms in total, consisting of 1600 ms of PSS/SSS detection, 2240 ms of SSB index detection and 1600 ms of measurement time.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<<End of Change5>>

<<Start of Change6>>

#### A.6.6.9.X	SA Idle mode CA/DC measurement for FR1 with 12RB SSB

##### A.6.6.9.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE performs the required measurements on the serving cell and the configured inter-frequency carrier for idle mode measurement reporting after the UE has entered Idle mode, with 12 RB SSB on the target carrier. This test will partly verify the Idle mode CA/DC measurements requirements in clause 4.4.

The test environment in clause A.6.6.9.1.1 apply to this test, except that beam level reporting for early measurements is configured. Supported test configuration is given in table A.6.6.9.X.1-1. General test parameters as specified in table A.6.6.9.1.1-2 with config 1 apply to this test. Cell specific test parameters as specified in table A.6.6.9.1.1-3 with config 1 apply to this test, except those specified in table A.6.6.9.X.1-2.

Table A.6.6.9.X.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |
| NOTE 1: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.9.X.1-2: Cell specific test parameters for connected mode for SA Idle mode CA/DC measurement for FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |  |  |
| BWP BW | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 | SR.1.2 FDD |  |  | N/A |  |  |
| CORESET Reference Channel |  | Config 1 | CR.1.3 FDD |  |  | N/A |  |  |
| SSB parameters |  | Config 1 | SSB.1 FR1 |  |  | SSB.13 FR1 |  |  |
| Io | dBm/2.7 MHz | Config 1 | -67.66 | -67.66 | -67.66 | -75.45 | -72.44 | -72.44 |

##### A.6.6.9.X.2	Test Requirements

The test requirements in clause A.6.6.9.1.2 apply to this test.

<<End of Change6>>