# 3GPP TSG-RAN WG4 Meeting #115                              R4-2506589

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.29.3.1

Source: 	CMCC

Title: 			 Discussion on event triggered L1 measurement reporting

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #107 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last meeting, there was discussion on measurements related enhancements for LTM, and a WF was agreed [3]. This contribution provides discussion on event triggered L1 measurement reporting.

- Discussion

- Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Reuse the framework from L3 event triggered reporting. Discuss the details further.

According to the previous discussion, one of the issue is how to handle the case that measurement period of the cells corresponding to the event are different, i.e. RS periodicity for candidate cell and serving cell are different. In this case, it is possible that the report is triggered by old measurement results of the cell which have longer RS period. This issue also exists in legacy L3 measurement report, no explicit clarification are made. One way is to follow L3 approach and do not have further clarification. The other way is to make it clear. We do not have strong view on which way to go. If it is agreed to make it clear in L1 measurement report, how to make the clarification need to be discussed. According to the discussion in last meeting, there are two options.

- Option 1: the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

- Option 2: If the event is triggered by the serving cell measurement, the event triggered measurement delay is TL1_measurement periods_serving cell. If the event is triggered by the candidate cell measurement, the event triggered measurement delay is TL1_measurement periods_candidate cell.

Our preference is option 2. The reason is that for option 1 with maximum of the L1 measurement periods to measure both current beam and new beams, the potential issue is that option 1 may delay the report. Even with shorter RS periodicity, UE may evaluate the event only during the longest measurement intervals. Based on above consideration, option 2 is preferred.

Proposal 1: If the event is triggered by the serving cell measurement, the event triggered measurement reporting delay shall be no larger than TL1_measurement periods_serving cell. If the event is triggered by the candidate cell measurement, the event triggered measurement reporting delay shall be no larger than TL1_measurement periods_candidate cell.

- Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Yes (Xiaomi, CMCC, ZTE, CTC, vivo) - Option 2: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple) - Issue 2-4: whether to define a separate capability for supported number of events triggered L1 measurement reporting Candidate options: - Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. (Xiaomi, CMCC, ZTE, E///, CTC) - Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///) - Option 2: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO) - Issue 2-5: whether and how to coordinate with R19 MIMO Agreement: - RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

For legacy event triggered L3 measurement reporting, the capabilities for support of event triggering reporting criteria are defined (section 9.1.4, TS38.133). Similarly, for Rel-19 event triggered L1 measurement reporting, it is necessary to define the capability. However, according to the discussion in last meeting, some companies mentioned that the number of events to be supported by UE shall be discussed in RAN1 first. We do not have strong view. But at least according to our knowledge, RAN1 have not discussed this. LS to RAN1 can be considered if necessary.

If it is agreed to define capability for supported number of events triggered L1 measurement reporting in RAN4, the related issue is how to define the capabilities for support of event triggering L1 measurement reporting, whether to update L3 capability or to define a seperate capability for L1 measurement reporting. In our view, a seperate capability is prefered. It is not a good way to replace or update legacy capability for event triggered L3 measurement reporting. For Rel-18 LTM measurement, the requirements are applied with the condition that the cell is known, which means that L3 measurement need to be configured. And also, network may adjust LTM configuration based on L3 measurement report.

Proposal 2: If it is agreed to define capability for supported number of events triggered L1 measurement reporting in RAN4, it is proposed to define a separate capability for supported number of events triggered L1 measurement reporting, to avoid the impact on L3 measurement reporting.

- Conclusion

This contribution provides discussion on event triggered L1 measurement reporting. The proposals are:

Proposal 1: If the event is triggered by the serving cell measurement, the event triggered measurement reporting delay shall be no larger than TL1_measurement periods_serving cell. If the event is triggered by the candidate cell measurement, the event triggered measurement reporting delay shall be no larger than TL1_measurement periods_candidate cell.

Proposal 2: If it is agreed to define capability for supported number of events triggered L1 measurement reporting in RAN4, it is proposed to define a separate capability for supported number of events triggered L1 measurement reporting, to avoid the impact on L3 measurement reporting.

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4

- R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2414060, WF on R19 Further NR mobility enhancement, Apple

- RP-233970, Revised WID on Further NR Mobility Enhancements

- R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4