# 3GPP TSG-RAN WG4 Meeting #115                              R4-2506592

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.29.4

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements for NR mobility enhancements

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #107 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

This is the first meeting for RAN4 to discuss RRM performance requirements for NR mobility enhancements. This contribution provides intial discussion on this topic.

- Discussion

For inter-CU Layer1/Layer 2 Triggered Mobility (LTM), from RAN4 perspective, following agreements are reached.

RAN4 agreements at #112 meeting (R4-2414060) Issue 2-1-1: RAN4 scope of inter-CU LTM Agreement: - Existing R18 LTM related RRM requirements also apply to same procedures in inter-CU scenario. - No RRM impact is foreseen to support inter-CU LTM. RAN4 can revisit this if any issue identified. RAN4 agreements at #113 meeting (R4-2420101) Issue 1-1: R19 inter-CU subsequent LTM <Agreement> - The Rel-18 cell switch delay requirements can be reused for Rel-19 inter-CU subsequent LTM. - Note: this can be revisited if any critical issue identified.

It can be seen that legacy Rel-18 LTM related RRM requirements are applied to inter-CU LTM and inter-CU subsequent LTM. Compared with intra-CU LTM, from cell switch procedure perspective, UE behaviour is similar, i.e. UE need to process the configuration, and perform DL and UL synchronization. From this point of view, it seems that there is no need to introduce new test cases. But we are open for further discussion.

Proposal 1: for inter-CU LTM and inter-CU subsequent LTM, it is proposed to discuss whether to define test cases.

For event triggered L1 measurement reporting, RAN4 agreed to define requirements for SSB based and CSI-RS based measurements. In addition to intra-frequency measurement and inter-frequency measurement without gap, support gap-based inter-frequency measurement for SSB-based L1 measurement. For CSI-RS based measurement, only measurement without gap are considered. Both FR1 and FR2-1 are in scope of RRM requirements for Event triggered L1 measurement reporting. To guarantee the performance, it is proposed to define define test cases for SSB based and CSI-RS based event triggered L1 measurement reporting. From test coverage point of view, with gap and withou gap, FR1 and FR2-1 need to be covered.

Proposal 2: for event triggered L1 measurement reporting, it is proposed to define test cases for SSB based measurement and CSI-RS based measurements.

- Both FR1 and FR2-1 are considered.

- For SSB based event triggered L1 measurement reporting, intra-frequency measurement, inter-frequency measurement without gap, gap-based inter-frequency measurement need to be considered.

- For CSI-RS based event triggered L1 measurement reporting, measurement without gap need to be considered.

Following LTM events based on beam specific quality of serving cell and candidate cells are supported as the L1 LTM measurement events.

- Event LTM2: Beam of serving cell becomes worse than absolute threshold

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell

- Event LTM4: Beam of candidate cell becomes better than absolute threshold

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

From test coverage point of view, it is preferred to define different test cases cover different LTM event. However, the test burden also need to be considered. To reduce the test number, selecting some of these LTM events are also fine. In our view, LTM event 3 and LTM event 5 are preferred. Since relative accuracy can be verified by Event LTM3, and absolte accuracy can be verified by Event LTM5.

Proposal 3: it is proposed to define test cases for LTM event 2/3/4/5 .

According to previous agreements, both FR1 and FR2-1 are in scope of RRM requirements for CSI-RS based L1-RSRP measurement on LTM candidate cell(s). CSI-RS based L1 measurement without gap is supported. From RAN4 perspective, not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell.

- The agreements on measurement procedure for CSI-RS based LTM candidate cell measurements are summarized as following:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

    - Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting.

- For a UE not supporting the capability, Step 2 is necessary.

  - The CSI-RS resources in Step 3 should be at least Type-D QCL’ed with the SSB in Step 2.

  - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

- For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped:

  - The CSI-RS resources in Step 3 should be Type-D QCL’ed with the SSB in Step 1.

  - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

  - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor.

    - Note: Step 2 is removed for FR1.

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

    - Depending on whether step 2 is skipped, the requirements in step 3 may be different.

It can be seen that whether step 2 is removed or not has impact on CSI-RS based L1-RSRP measurement period. Without step 2, UE Rx beam sweeping factor is included in the CSI-RS based L1-RSRP measurement period. With step 2, UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period. Since UE behaviour is different, it is necessary to define test cases to cover both cases.

Proposal 4: for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested.

The legacy CSI-RS based L1-RSRP measurement are for serving cell, both absolute accuracy and relative accuracy are specified. The leagcy CSI-RS based L1-RSRP measurement accuracy are defined based on the assumption that the bandwidth of CSI-RS is 48 PRBs and the density is 3. The side condition is -3dB.

For CSI-RS based L1-RSRP measurement on neighbor cell, it was agreed to define the measurement period and accuracy requirements based on Density=3 with (≥) 48 PRBs. As for side condition for CSI-RS based L1 measurement for neighbor cell, it was agreed to use legacy side condition of SINR=-3dB. It can be seen that the BW, density, and side condition are same are same as that for legacy CSI-RS based L1-RSRP measurement are for serving cell. And in last meeting, it was agreed not to define RAN4 requirements for RTD > CP. Taking above into account, the legacy accuracy requirements can be used as baseline to define accuracy requirements for CSI-RS based LTM candidate cell measurements. Whether further update is needed can be futher discuss.

Proposal 5: for accuracy requirements for CSI-RS based LTM candidate cell measurements, including absolute accuracy requirements and relative accuracy requirements, it is proposed to take legacy CSI-RS based L1-RSRP measurement accuracy requirements as baseline.

Both CLTM and subsequent CLTM are considered. CLTM cell switch delay requirements and subsequent CLTM  cell switch delay requirements will be defined. It is necessary to define test cases for the two cases.

Proposal 6: it is proposed to define test cases for CLTM cell switch delay and subsequent CLTM cell switch delay.

In previous meeting, it was agreed that Rel-19 CLTM requirements are applicable for NR SA scenarios. In detail, following cases are agreed.

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

From test coverage point of view, it is necessary to define test cases to cover these cases.

Proposal 7: for CLTM and subsequent CLTM, it is proposed to define the tese cases for following scenarios:

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

- Conclusion

This contribution provides discussion on RRM performance requirements for NR mobility enhancements. The proposals are:

Proposal 1: for inter-CU LTM and inter-CU subsequent LTM, it is proposed to discuss whether to define test cases.

Proposal 2: for event triggered L1 measurement reporting, it is proposed to define test cases for SSB based measurement and CSI-RS based measurements.

- Both FR1 and FR2-1 are considered.

- For SSB based event triggered L1 measurement reporting, intra-frequency measurement, inter-frequency measurement without gap, gap-based inter-frequency measurement need to be considered.

- For CSI-RS based event triggered L1 measurement reporting, measurement without gap need to be considered.

Proposal 3: it is proposed to define test cases for LTM event 2/3/4/5 .

Proposal 4: for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested.

Proposal 5: for accuracy requirements for CSI-RS based LTM candidate cell measurements, including absolute accuracy requirements and relative accuracy requirements, it is proposed to take legacy CSI-RS based L1-RSRP measurement accuracy requirements as baseline.

Proposal 6: it is proposed to define test cases for CLTM cell switch delay and subsequent CLTM cell switch delay.

Proposal 7: for CLTM and subsequent CLTM, it is proposed to define the tese cases for following scenarios:

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4