# 3GPP TSG-RAN WG4 Meeting #115                              R4-2506593

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.30.3

Source: 	CMCC

Title: 			 Discussion on XR (eXtended Reality) for NR Phase 3

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on XR (eXtended Reality) for NR Phase 3 was approved [1]. According to the WID, one of the objectives having RAN4 impact is to enable transmission/reception in gaps/restrictions that are caused by RRM measurements, the details are duplicated as following.

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In last meeting, there was discussion on XR (eXtended Reality) for NR Phase 3, and a WF was agreed [2]. This contribution provides discussion on this topic.

- Discussion

Issue 2-1: Types of measurement gaps In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG. <Agreement>: - Gaps which shall not be cancelled - MUSIM - Positioning - For other gap types: - The companies bring the issues and solutions on the RAN4#115. If the solution is not agreeable in RAN4#115, the gap type is not part of Rel-19 - The discussion in RAN4#115 starts with Type 2 MG, then other gap types based on incoming contributions if time allows.

In previous meetings, it was agreed that for the type of measurement gap, as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other measurement gap types are deprioritized, but the discussion on other measurement gap types will not impact the completion of the WI [3]. According to the WID, the objective is to specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements [1]. It can be seen that no specific measurement gap is the target, which means that all the measurement gaps introduced so far need to be considered. Type 1 MG, Type 2 MG, NCSG, Pre-MG, concurrent gaps, MUSIM gaps, positioning gaps are introduced in RAN4. considering that positioning gaps are introduced for positioning, and MUSIM gaps are introduced for the measurement of another network, whether positioning gaps and MUSIM gaps can be cancelled for XR need further discussion. All other measurement gap types are for measurement for the mobility purpose. Since Type-1 MG is considered, we do not see issues to preclude other measurement gap types.

In detail, for NSCG, the measurement gap skipping is applied to VIL1 and VIL2 of a NCSG pattern. The time distance between DCI indicating gap skipping and the start of VIL1 is no less than the time offset. For Pre-MG, the measurement gap skipping is applied to the activated Pre-MG, i.e apply to the first activated Pre-MG which satisfy the time offset. For concurrent gap, the measurement gap skipping is applied to the kept gap after applying dropping rules based on priority.

Proposal 1: for types of measurement gaps, it is proposed that Type 1 MG, Type 2 MG, NCSG, Pre-MG, concurrent gaps are considered.

- For NSCG, the measurement gap skipping is applied to VIL1 and VIL2 of a NCSG pattern

- For Pre-MG, the measurement gap skipping is applied to the first activated Pre-MG which satisfy the time offset

- For concurrent gap, the measurement gap skipping is applied to the kept gap after applying dropping rules based on priority.

Issue 3-2: Formulas for measurement delay extension <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - FFS: CSSFinter

It was agreed to follow NR-U-like approach for designing the formulas for measurement delay extension. Following is the duplication of legacy measurement period for NR intra-frequency measurements with CCA. It can be seen that Lmeas,gaps is used to reflect the number of SMTC occasions not available at the UE due to LBT. For XR, measurement gap occasions indicated to be skipped are not available for measurement. It can be seen that the situation are similar between NR-U and XR. The way to define delay requirements for CCA can be reused to calculate measurement delay extension due to MG skipping for XR.

Table 9.2A.6.2-1: Measurement period for intra-frequency measurements with gaps (FR1)

| Condition | TSSB_measurement_period_intra_CCA |
| --- | --- |
| No DRX | max(200 ms, (5+Lmeas,gaps) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x (5+Lmeas,gaps)) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | (5+Lmeas,gaps) x (MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, Lmeas,gaps is the number of SMTC occasions not available at the UE during TSSB_time_index_intra_CCA for measurement where Lmeas,gaps < Lmeas,gaps,max. When DRX is configured, Lmeas,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_time_index_intra_CCA for measurement where Lmeas,gaps < Lmeas,gaps,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 2: Lmeas,gaps,max = 7 for Max(DRX cycle, SMTC period, MGRP)≤40ms where DRX cycle is 0 for non-DRX, Lmeas,gaps,max = 5 for 40ms<Max(DRX cycle, SMTC period, MGRP)≤320ms, Lmeas,gaps,max = 3 for DRX cycle>320ms. NOTE 3: Upon exceeding Lmeas,gaps,max over the TSSB_measurement_period_intra_CCA period of time, the UE has to restart the measurement procedure. |  |

Proposal 2: taking FR1 inter-frequency measurements with gap as an example, for the case of no DRX, the measurement delay extension due to measurement skipping is proposed as following:

- Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

Issue 6-1: Semi-static schemes <Way forward>: - Further discussion based on recommended options - Option 1: Do not consider semi-static solutions - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution. - Option 2a: RRC pattern-based - Option 2b: MAC CE bitmap for skipping indication - Option 2c: RRC configuration patterns with MAC CE activation - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

We see the benifit to introduce semi-static schemes additionally. On one hand, semi-static scheduling mechanisms, i.e. CG and SPS, are used for XR service. RRC configuraion is more suitable for semi-static scheduling mechanisms. On the other hand, it is more efficient to indicate the skipped measurement occasions by RRC configuration, which not only reduces signaling overhead, but also simplifies UE implementation.

Proposal 3: for solutions to enable transmission/reception in gaps/restrictions, it is proposed to introduce semi-static schemes.

- Conclusion

This contribution provides discussion on XR (eXtended Reality) for NR Phase 3. The observations and proposals are:

Proposal 1: for types of measurement gaps, it is proposed that Type 1 MG, Type 2 MG, NCSG, Pre-MG, concurrent gaps are considered.

- For NSCG, the measurement gap skipping is applied to VIL1 and VIL2 of a NCSG pattern

- For Pre-MG, the measurement gap skipping is applied to the first activated Pre-MG which satisfy the time offset

- For concurrent gap, the measurement gap skipping is applied to the kept gap after applying dropping rules based on priority.

Proposal 2: taking FR1 inter-frequency measurements with gap as an example, for the case of no DRX, the measurement delay extension due to measurement skipping is proposed as following:

- Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

Proposal 3: for solutions to enable transmission/reception in gaps/restrictions, it is proposed to introduce semi-static schemes.

- Reference

- RP-234080, New WID: XR (eXtended Reality) for NR Phase 3

- R4-2504911, WF on RRM requirements for XR_Ph3

- R4-2502700, WF on RRM requirements for XR_Ph3

- R4-2420103, WF on RRM requirements for XR_Ph3, Nokia

- R4-2416864, WF on RRM requirements for XR_Ph3, Nokia

- R1-2209536, XR-specific capacity enhancements, Nokia, Nokia Shanghai Bell

- R4-2414043, WF on RRM requirements for XR_Ph3, Nokia

- R1-2405736, LS on UE assistance information

- R1-2407561, LS on gaps/restrictions that are caused by RRM measurements