# 3GPP TSG-RAN WG4 Meeting #115                             R4-2506597

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.18.3.1

Source: 	CMCC

Title: 			 Discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN #103 meeting, the new WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is FR2-1 SSB based L3 measurement delay reduction for connected mode, the details are duplicated as following.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

There is discussion on FR2-1 SSB based L3 measurement delay reduction for connected mode in last meeting, and a WF was approved [2]. This contribution provides discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor.

- Discussion

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” - Proposals for scenario 1/2/3/4 - Option 1: UE autonomous quitting. - Option 1a (vivo, LGE, HW, Intel, Samsung, Ericsson): - UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results. - (vivo): If there is no consensus, it is preferred to only consider threshold conditions as the exit conditions. - Option 1b (LGE): RAN4 to leave it to UE implementation whether to keep or re-active FR2 FBS when serving cell’s signal quality is lower than the threshold configured by network and after quit conditions being discussed have been met. - Option 1c (HW): - UE is allowed to fall back to normal measurement when UE has performed fast measurement for at least T1. - After quitting fast measurement, UE shall re-activate fast measurement no longer than T2 from the quitting time. - Option 1d (Intel): - The reduced beam sweeping factor state continues to apply until the UE reports the measurement results to the network. And the UE makes another exam on the criteria that are configured for the serving cell quality. If the criteria are met, the UE continues to use reduced beam sweeping factor while the UE exits the state when the quality criteria are no longer met. - Option 1e (Samsung): - Define the maximum period of time of UE to continue BSF measuring after it has reported the qualified measurement result as a timer. - Option 1f (Ericsson): - UE may quit FBS if the UE has completed at least one measurement in the FBS way and the UE identifies that there isn’t any urgent task to continue FBS, e.g., the UE remains a stationary state or no qualified neighbour cells are available for switching. - Option 2: UE indication to the NW is used to deactivate FBS based L3 measurements. - Option 2a (OPPO): The detailed signaling can leave to RAN2 decision. - Option 2b (LGE): Reuse Rel-18 UAI ‘multiRx-PreferenceFR2’. - Option 2c (CMCC): Introduce UAI similar as “multiRx-PreferenceFR2” to inform network there is power consumption issue. - Option 2d (CT, Samsung, MTK): UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction. - Option 2e (Apple): To consider UE power consumption or overheating, UAI mechanism should be used. Either the existing UAI IE “multiRx-PreferenceFR2-r18” can be re-purposed to indicate not only L1 multi-RX operation, but also L3 measurement delay reduction, or a new IE such as “FastL3Measurement-r19” can be designed. - To address the power consumption issues in the two cases (Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving; Case 2: UE is still in the cell centre and the triggering conditions are met due to block.), it is proposed to consider the following two options: - Option 1: No specific action is taken, assuming UE can use UAI to inform the network that it needs to fall back to Non-FBS mode (i.e., deactivate FBS operation) if power consumption/overheating becomes an issue. - Option 2: After the UE sends a qualified measurement report, the UE can start a timer, and when the timer expires, the UE exits from FBS operation. - Option 3: Timer-based solution to deactivate Rx BSF reduction. - Option 3a (CATT) - UE deactivates Rx BSF reduction feature based on timer-solution, i.e., when either of the conditions are met: - Serving cell quality threshold condition is still met, but timer expires. - While timer is still running, but serving cell quality threshold condition is no long met. - Introduce another timer, and UE re-evaluates the serving cell quality threshold condition after it expires. This timer starts to run when UE deactivates Rx BSF reduction feature. - Option 3b: (Ericsson) - Instead of TTT, RAN4 to introduce ‘time to deactivate’ parameter, which limits the minimal FBS measurement time after activated/enabled. - UE may quit FBS in certain case with or without other conditions, but at least after the minimal time period ‘time to deactivate’. - Proposals for scenario 5/7 - Option 4 (OPPO): - For scenario 5/7, UE deactivate FBS based L3 measurements after the delay for handover or RRC Connection Release with Redirection.

According to the previous discussion, scenario 1/2/3/4/5/7 are considered as first priority. For scenario 1/2/3/4, (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. For scenario 5/7, for UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch) and RRC re-establishment.

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 5: Handover

- Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

The intention to consider quit mechanism is from the perspective of power consumption. According to previous discussion, some companies propose that UE is allowed to fall back to normal measurement mode (i.e. deactivated multiple panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification. We have different views, HO and SCell related configuration is up to network implementation. If the conditions to trigger UE to activate multiple panel reception is met, it is expected that UE can continue in this mode until reception of network command, e.g. HO command. The reason is that if UE fall back to normal measurement mode after UE has reported qualified measurement results, there may be mobility issue since the reported measurement results are outdated. It is not preferred to allow UE quit multi-Rx for L3 measurement autonomously.

We understand UE’s concern on power consumption. Similar issue exist in Rel-18 multi-RX reception. And UAI “multiRx-PreferenceFR2” is introduced. Similar approach can be considerd for L3 measurement delay reduction by optimizing Rx BSF. UAI similar as “multiRx-PreferenceFR2” can be introduced to inform network there is overheating issue. Then netwok can enable UE to deactivate multi-Rx for L3 measurement by reconfiguring the threashold.

Proposal 1: for scenario 1/2/3/4, it is proposed to introduce UAI similar as “multiRx-PreferenceFR2” to inform network there is power consumption issue.

For scenario 5/7, there is proposal that UE deactivate FBS based L3 measurements after the delay for handover or RRC Connection Release with Redirection. We do not think this is necesary. In RAN4#112 meeting, it was agreed that for UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch), and faster beam sweeping always applies during RRC re-establishment. In our view, the proposal that UE deactivate FBS based L3 measurements after the delay for handover or RRC Connection Release with Redirection is confusing. After the delay for handover or RRC Connection Release with Redirection seems not part of scenario 5/7. It is proposed to follow previous agreements.

Proposal 2: for scenario 5/7, it is proposed to follow previous agreements that:

- For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch).

- For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

- Conclusion

This contribution provides discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor. The proposals are:

Proposal 1: for scenario 1/2/3/4, it is proposed to introduce UAI similar as “multiRx-PreferenceFR2” to inform network there is power consumption issue.

Proposal 2: for scenario 5/7, it is proposed to follow previous agreements that:

- For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch).

- For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2504900, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2502593, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2416853, WF for NR_RRM_Ph5_Part1, Apple

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple

- RP-240817, Moderator's summary for discussion on RRM enhancements