# 3GPP TSG-RAN WG4 Meeting #115                             R4-2506605

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.21.7

Source: 	CMCC

Title: 			 Discussion on demodulation for CSI prediction

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion core requirements for CSI prediction.

- Discussion

Issue 1-1: Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Issue 1-2: Requirement baseline Agreement: - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed Issue 1-4: Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML [a given SNR] using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML [the same SNR] with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

According to the revised WID [2], CSI prediction (one-sided model) is one of the objectives for WI, and RAN4 need to specify necessary RAN4 requirements. In previous meeting, it was agreed to use relative throughput as requirement metric for CSI prediction. Reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook.

For CSI prediction, it was agreed that AI/ML based performance requirement should not be worse than the legacy, and FFS which legacy requirement/feature are relevant.

Agreements in RAN4#112bis Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

In last meeting, it was agreed to use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions. According to TS38.101-4, the legacy PMI requirements for transmission scheme 1 with higher  layer parameter codebookType set to 'typeII' or 'typeII-r16' or 'typeII-doppler-r18' or 'typeII-CJT-r18' are specified in terms of the ratio:

In the definition of γ, for 16TX PMI requirements, is 90 %  of the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured at with random precoding. In our view, for CSI prediction, AI/ML based performance requirement should not be worse than the legacy requirements for eType II Doppler codebook.

Proposal 1: for CSI prediction, it is proposed that AI/ML based performance requirement should not be worse than the legacy requirements for eType II Doppler codebook.

For testing conditions, it was agreed to use static conditions as baseline, FFS on whether to have non-static condition. Static condition is similar as we have for legacy test. Non-static condition is more suitable to verify the generalization and is benificial in reducing the number of tests. It is good if we could have tests for both static and non-static condition.

However, considering the limited timeline and there are so many issues to be solved, it seems impossible to have parallel discussion on both cases. Especially, non-static conditions are new for RAN4 which will take a long time to discuss the detail, for example, how the channel model and SNR settings are changed over the test. It is preferred to firstly focus on static condition, and non-static condition can be further discussed if time allowed.

With static conditions, differenttest scenarios, e.g. antenna configuration, UE speed, nee to be considered. In legacy PMI test, the test cases are defined without considering diffferent senarios for a certain precoder type, since it is assumed same PMI reporting algorithm is used. However, AI/ML performance is impacted by training dataset, which is related with scenarios. According to RAN1 evaluation, CSI prediction performance is impacted by the observation window length, prediction window length, and UE speed. These factors which have impact on performance need to be considered in define requirements. To guarantee the performance, it is necessary to define test cases with different test configuration.

Proposal 2: for CSI prediction test with static condition, it is proposed to define test cases with different configuration/parameters, e.g. antenna configuration, observation window length, prediction window length, and UE speed .

For channel model for CSI prediction tests, it was agreed that TDL channel models will be used, FFS on whether CDL channel model is needed for generalization test. It is beneficial to use CDL channel model. However, legacy PMI tests are not based on CDL. And the feasibility for demodulation performance requirements with CDL channel model is under discussion in Rel-19 SI. Considering current situation, it is not feasible to consider CDL channel model in Rel-19. Using CDL channel model for CSI prediction test can be further dicsussed in later release based on the outcome of Rel-19 SI on SCM.

Proposal 3: For channel model for CSI prediction tests, it is proposed to focus on TDL channel models in Rel-19. Using CDL channel model for CSI prediction test can be further dicsussed in later release based on the progress on SCM.

In last meeting, the baseline simulation assumptions for AIML based CSI prediction was approved [11]. We provide simulation results for step 1 to check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE).

Table 1 Agreed simulation assumptions (R4-2505229)

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| gNB TX antennas | Option1: 32 (N1,N2)=(4,4) as defined in Rel-15 type1 Option2: 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Delay spread | 30ns |
| Channel estimation | Start with Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Further consider practical channel estimation in the next step - Use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| Rank per UE | Rank 1 and 2 |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step2: take CSI feedback into account (wait more related agreements in RAN4) - Option 1: Use Rel-18 eTypeII-Doppler - Companies are encouraged to check RAN1 agreements |

In our simulation, 4GHz for TDD is in use. gNB TX antennas is 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements. Doppler spread is 50Hz. Rank1 is assumed. Table 2 is our simulation results.

Table 2 SGCS for CSI prediction

|  | SGCS |
| --- | --- |
| ConvLSTM | 0.9249 |

Proposal 4: for CSI prediction, the evaluation results for step 1 are shown as Table 2.

- Conclusion

This contribution provides discussion on CSI prediction. The proposals are:

Proposal 1: for CSI prediction, it is proposed that AI/ML based performance requirement should not be worse than the legacy requirements for eType II Doppler codebook.

Proposal 2: for CSI prediction test with static condition, it is proposed to define test cases with different configuration/parameters, e.g. antenna configuration, observation window length, prediction window length, and UE speed .

Proposal 3: For channel model for CSI prediction tests, it is proposed to focus on TDL channel models in Rel-19. Using CDL channel model for CSI prediction test can be further dicsussed in later release based on the progress on SCM.

Proposal 4: for CSI prediction, the evaluation results for step 1 are shown as Table 2.

Table 2 SGCS for CSI prediction

|  | SGCS |
| --- | --- |
| ConvLSTM | 0.9249 |

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2502970, Baseline Simulation assumptions for AIML based CSI prediction

- R4-2505229, Updated simulation assumptions for AIML CSI prediction