# 3GPP TSG-RAN WG4 Meeting #115                             R4-2506606

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.21.2

Source: 	CMCC

Title: 			 Discussion on general aspects for AI/ML for NR air interface

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on general aspects.

- Discussion

In last meeting, it was agreed that AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference. In our view, there are two cases.

- Case 1 is from non-AI to AI, i.e. from the status that no any AI funtionality is activated to the status that at least one AI funtionality is activated.

- Case 2 is switch among different AI funtionalities, e.g. from AI funtionality 1 switch to AI funtionality 2, which means AI funtionality 2 is activated and AI funtionality 1 is deactivated.

If the delay reuqirements for funtionality activation is defined, we are not sure whether same delay requirements are applied for both cases. Our view is that same requirements are applied, but we are not sure from UE implementation point of view, is there any different between the two cases which result in different activation delay.

Proposal 1: if AI/ML functionality activation delay requirements are defined, it is proposed to discuss whether same activation delay requirements are applied to following two cases:

- Case 1 is from non-AI to AI, i.e. from the status that no any AI/ML funtionality is activated to the status that at least one AI/ML funtionality is activated.

- Case 2 is switch among different AI funtionalities, e.g. from AI/ML funtionality 1 switch to AI/ML funtionality 2, which means AI/ML funtionality 2 is activated and AI/ML funtionality 1 is deactivated.

Similar issue exists for AI/ML functionality deactivation. There are two cases:

- Case 1 is from AI to non-AI, i.e. from the status that at least one AI/ML funtionality is activated to the status that no any AI/ML funtionality is activated.

- Case 2 is switch among different AI/ML funtionalities, e.g. from AI/ML funtionality 1 switch to AI/ML funtionality/ 2, which means AII/ML funtionality 2 is activated and AII/ML funtionality 1 is deactivated.

Our view is that same deactivation delay requirements are applied for the two cases, but we are not sure from UE implementation point of view, is there any different between the two cases which result in different activation delay.

Proposal 2: if AI/ML functionality deactivation delay requirements are defined, it is proposed to discuss whether same deactivation delay requirements are applied to following two cases:

- Case 1 is from AI to non-AI, i.e. from the status that at least one AI/ML funtionality is activated to the status that no any AI/ML funtionality is activated.

- Case 2 is switch among different AI/ML funtionalities, e.g. from AI/ML funtionality 1 switch to AI/ML funtionality/ 2, which means AII/ML funtionality 2 is activated and AII/ML funtionality 1 is deactivated.

Agreements in RAN4#112bis Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

For the target for AI/ML based beam management, our view is that AI/ML based performance requirement should not be worse than the legacy. According to the discussion in last meeting, some companies mentioned that the AI/ML based performance will be worse than the legacy, since the gain come from overhead reduction. From operator point of view, we do not think that this is a good way to go. If the predicted results are worse than the legacy measured results, it will have impact on the network deployment, which is not preferred.

Proposal 3: for AI/ML based beam management, AI/ML based performance requirement should not be worse than the legacy measurement requirements.

- Conclusion

This contribution provides discussion on general aspects. The proposals are:

Proposal 1: if AI/ML functionality activation delay requirements are defined, it is proposed to discuss whether same activation delay requirements are applied to following two cases:

- Case 1 is from non-AI to AI, i.e. from the status that no any AI/ML funtionality is activated to the status that at least one AI/ML funtionality is activated.

- Case 2 is switch among different AI funtionalities, e.g. from AI/ML funtionality 1 switch to AI/ML funtionality 2, which means AI/ML funtionality 2 is activated and AI/ML funtionality 1 is deactivated.

Proposal 2: if AI/ML functionality deactivation delay requirements are defined, it is proposed to discuss whether same deactivation delay requirements are applied to following two cases:

- Case 1 is from AI to non-AI, i.e. from the status that at least one AI/ML funtionality is activated to the status that no any AI/ML funtionality is activated.

- Case 2 is switch among different AI/ML funtionalities, e.g. from AI/ML funtionality 1 switch to AI/ML funtionality/ 2, which means AII/ML funtionality 2 is activated and AII/ML funtionality 1 is deactivated.

Proposal 3: for AI/ML based beam management, AI/ML based performance requirement should not be worse than the legacy measurement requirements.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420336, WF on AI/ML air interface part 2, Ericsson

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R5-247647, Discussion on handling of AI/ML in RAN5, China Mobile, AT&T, CAICT, Ericsson, Huawei, MediaTek, Nokia, ZTE

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface