# 3GPP TSG-RAN WG4 Meeting #115                              R4-2506608

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.22.2

Source: 	CMCC

Title: 			 Discussion on general aspects for study on AI/ML for NR air interface Phase 2

Document for:	Discussion

- Introduction

In RAN #107 meeting, the revised SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are duplicated as following.

- Testability and interoperability [RAN4]: - Further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [2]. This contribution provides discussion on general aspects.

- Discussion

Issue 1-1-1: How many requirement sets to define - Proposals - Option 1: Define requirement sets for at least every feature and applicable subset (Apple proposal 1/12, Nokia proposal 1) - Option 1a: FFS how to relate subsets and applicable conditions depending on definition of applicable conditions (Apple proposal 2, 13) - Option 1b: A minimum set of configurations/IDs shall be mandatory (Nokia proposal 2) - Option 1c: Do not define requirements with a change of associated ID during testing (Nokia proposal 3) - Option 1d: Ensure consistency between applicable conditions signalling during training and inference (Apple proposal 10) - Option 2: At least a requirement set per functionality, TBC for subsets depending on conclusions on granularity (CMCC proposal 3, CATT proposal 4)

According to RAN1 discussion, for AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. It can be seen that UE will report the supported configurations for a certain model/functionality. The scenarios and/or configurations for requirements/tests can be decided based on the supported configuration reported by UE.

A LS on applicable functionality reporting for beam management UE-sided model is sent from RAN2 to RAN1. One of the questions is what is the granularity of functionality. For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others. According to RAN1 agreements, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same. The concept/terminology “functionality”of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs. The concept/terminology “ functionality”  of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters. The Activated functionalities may be enabled based on CSI framework. The definition is also alligned with RAN2 understanding. Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling). Applicable functionalities refers to functionalities that the UE is ready to apply for inference. Activated functionalities refers to functionalities already enabled for performing inference.

RAN1 agreements in RAN1 #119 - The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “ functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same. RAN2 agreements in RAN2 #127 Agreements on definitions - Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) - Applicable functionalities refers to functionalities that the UE is ready to apply for inference - Activated functionalities refers to functionalities already enabled for performing inference

Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities in step 1. UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. The applicable functionalities can be sent to network in step 4 or updated in UAI. The funtionalily can be activated after step 3 or step 5 pending on whether full inference configuration is signalled in step 3 or step 5. The UE applicable functionalities can be activated autonomously by UE or activated by network indication.

Based on RAN1 and RAN2 agreements, it can be seen that the interaction between network and UE is in the granularity of funtionality. From RAN4 point of view, the requirements/tests are defined at least based on funtionality (funtionality refer to AI/ML-enabled Feature/FG) .

Proposal 1: RAN4 requirements are specified at least based on functionality (funtionality refer to AI/ML-enabled Feature/FG)

Issue 1-1-2: How many requirements to define per requirement set - Proposals - Option 1: To determine how many requirements to define, a study is needed of how AI/ML behaviour changes with different conditions (Apple proposal 3, Nokia proposal 4) - Option 2: Different test configurations/parameters are used for tests, at least for generalization (CMCC proposal 2). - Option 3: It is necessary to ensure that the testing avoids the overfitting problem; if not other solutions may be needed (Korea test laboratory proposals 4 and 5) - Option 4: Create a mixed test-set for generalization testing, based on UE capability signaling configuration report (Apple proposals 7, 8) - Option 5: Scenarios not identified in the capability report are other scenarios for test (ZTE)

In previous meeting, the testing goals are updated as to verify whether the minimum performance of AI/ML functionality/feature can be achieved, but FFS on whether and how many different test configurations/parameters are used for tests. Our view is that it is necessary to consider different test configurations/parameters at least for generalization. According to Rel-18 discussion, the goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations.

Based on RAN1 evaluation, generalization is impacted by many factors. In high level, according to TR 38.843, deployment scenarios, UE distributions, carrier frequency, TxRU mappings are considered for generalization of CSI compression. And CSI payload sizes, Tx port numbers, are considered for scalability evaluations for CSI compression. For beam management, the performance is impacted by deployment scenarios, UE mobility, etc. Similar situation for positioning. These factors which have impact on performance need to be considered in define requirements

Proposal 2: it is proposed that different configurations/parameters are considered for requirements/tests definition.

Delay requirements for activation/switching/fallback Agreement: - RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2: - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference - functionality deactivation refers to the process of network deactivating an active functionality Agreement: - FFS on whether the deactivation requirements will be introduced - The starting point is when the UE receives the signalling to commence deactivation - FFS when the endpoint for deactivation is - FFS whether fallback delay is the same as deactivation delay or has some other components Agreement: - For beam management use case - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistant reporting based on AI/ML - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML - FFS on the definition of first reporting

In our internel check, the understanding that AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference and functionality deactivation refers to the process of network deactivating an active functionality are aligned with RAN1 and RAN2.

As for the activation delay requirements, it can be discussed case by case. As agreed in RAN2, the activation machanism is different from different CSI reporting type.

RAN2 agreements in RAN2#129 Agreements - Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). - The full inference configuration is sent in CSI-ReportConfig. - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig) - Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities FFS option B Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

It can be seen that how applicable functionalities is activated is different for different report type. If option A is configured in Step 3 (inference configuration/parameters can be signalled in step 3), for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). For the case that option A is configured in Step 3 (inference configuration/parameters are signalled in step 3), since it is autonomously activated by reporting applicable functionalities, no need to define activation delay requirements. For the case that option B is in use (inference configuration are not signalled in step 3), UE report the applicable funtionalities in step 4, but these applicable funtionalities are not autonomously activated. After UE receive the inference configuration signalled in step 5, UE start to perform the funtionality activation. In this case, the activation delay can be defined, and the starting point is when UE receive inference configuration, the ending point is when UE report inference results.

Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework. For semi-persistent CSI reporting, the applicable functionality is activated by MAC CE/DCI, for aperiodic CSI reporting, the applicable functionality is activated by DCI. For these two cases, the activation delay can be defined, and the starting point is the point receiving activation indication by MAC CE/DCI, the ending point is the point reporting inference results.

Proposal 3: for funtionality activation, it is proposed that

- For periodic CSI reporting,

- For the case that option A is configured in Step 3 (i.e. inference configuration/parameters are signalled in step 3), no need to define activation delay, since RAN2 agreed that UE autonomously activate the applicable functionalities upon reporting applicable functionalities.

- For the case that option B is configured in Step 3 (i.e. inference configuration/parameters are not signalled in step 3), the starting point of activation delay is the point receiving inference configuration.

- For semi-persistent reporting, the starting point of activation delay is the point receiving activation indication by MAC CE/DCI.

- For aperiodic CSI reporting, the starting point of activation delay is the point receiving activation indication by DCI.

Way forward on post-deployment in RN4#114meeting The following table presents the description of options 1 and 2 and sub-options as already agreed. A new name for “option 1” and “option 2” is proposed to reduce ambiguity when discussing in the future. The descriptions of the options are the same as the already agreed description. The “further potential clarifications” box captures issues that have been raised that could further clarify the options. It does not represent any agreement on the issues, or that all are relevant, but is intended to stimulate input and discussion to future meetings. The “significant issues” line is intended to capture issues that have been raised that should be answered in order to make a decision. Option 1; Option 2. New name; Pre-activation functionality/model update testing; Post deployment management based on LCM. Description; Conduct the validation of a change in AI functionality before its deployment/activation in already deployed UEs • Validation takes into account the UE hardware in which the model is to be deployed/activated.; Using performance monitoring and LCM procedures • Performance monitoring will be designed in other groups • RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.. Sub-options; Possible collection of input data to a model during conformance testing for use later on.. Further potential clarifications (not resolved); The following proposals have been made for option 1 but not discussed. o Whether the testing of a new model is on a device or in a lab. o Whether models that have been updated post conformancetesting should be kept in a device. o Whether parallel operating of one model and (in-device) testing of another can be assumed. o Whether signaling from a UE of changes/updates are needed. - Whether there is any relation to GCF procedures. - Whether it is possible to differentiate “major” and “minor” model changes and only apply option 1 to major changes. - Definition of Fine-tuning and Model update - Model update can have a performance impact. - Fine-tuning might have little performance impact. - Hybrid Validation approach - A hybrid approach integrating both pre-activation functionality/model update testing (Option 1) and post-deployment management based on LCM (Option 2) aims to provide a balanced validation strategy.; The following proposals have been made for option 1 but not discussed. - Whether the metric used for performance testing and the metric used for monitoring can be aligned. - Whether monitoring used for post-deployment testing should be NW sided only.. Significant issues; Whether the option is reasonable or would cause too much complexity and test time burden when introducing updates.; Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.) Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs.. Agreements in RAN4#114bis meeting: - Rename “option 1” as “Post-deployment pre-activation functionality/configuration update testing” - Rename “option 2” as “Post-deployment post-activation functionality testing based on performance monitoring”

For post-deployment, according to the discussion in previous meetings, for option 1 (Post-deployment pre-activation functionality/configuration update testing), one of the further potential clarifications (not resolved) is whether there is any relation to GCF procedures. Our views is provided as following.

- Figure 1: certification procedures for the scenario that the UE already supports AI functionality/features when it is certified, and the certified UE is subsequently modified with updated AI models which enable the certified UE with new functionality/features

- Figure 2: certification procedures for the scenario that the UE does not support AI functionality/features when it is certified, and the certified UE is subsequently modified with addition of AI functionality/features

Proposal 4: for post-deployment, for the clarification about whether there is any relation to GCF procedures, the possible related certification procedures are provided as above Figure 1 and Figure 2.

- Conclusion

This contribution provides discussion on general aspects for study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2. The proposals are:

Proposal 1: RAN4 requirements are specified at least based on functionality (funtionality refer to AI/ML-enabled Feature/FG)

Proposal 2: it is proposed that different configurations/parameters are considered for requirements/tests definition.

Proposal 3: for funtionality activation, it is proposed that

- For periodic CSI reporting,

- For the case that option A is configured in Step 3 (i.e. inference configuration/parameters are signalled in step 3), no need to define activation delay, since RAN2 agreed that UE autonomously activate the applicable functionalities upon reporting applicable functionalities.

- For the case that option B is configured in Step 3 (i.e. inference configuration/parameters are not signalled in step 3), the starting point of activation delay is the point receiving inference configuration.

- For semi-persistent reporting, the starting point of activation delay is the point receiving activation indication by MAC CE/DCI.

- For aperiodic CSI reporting, the starting point of activation delay is the point receiving activation indication by DCI.

Proposal 4: for post-deployment, for the clarification part about whether there is any relation to GCF procedures,  the possible related certification procedures are provided as following Figure 1 and Figure 2.

- Figure 1: certification procedures for post-deployment for the scenario that the UE already supports AI functionality/features when it is certified, and the certified UE is subsequently modified with updated AI models which enable the certified UE with new functionality/features

- Figure 2: certification procedures for post-deployment for the scenario that the UE does not support AI functionality/features when it is certified, and the certified UE is subsequently modified with addition of AI functionality/features

- Reference

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

- R4-2505107, WF on AI/ML air interface study, Ericsson

- R4-2503953, WF on AI/ML air interface study, Ericsson

- R4-2420336, WF on AI/ML air interface part 2, Ericsson

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R5-247647, Discussion on handling of AI/ML in RAN5, China Mobile, AT&T, CAICT, Ericsson, Huawei, MediaTek, Nokia, ZTE

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface