# 3GPP TSG-RAN WG4 Meeting #115                              R4-2506614

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.26.4

Source: 	CMCC

Title: 			 Discussion on RRM requirements for A-IoT

Document for:	Discussion

- Introduction

In RAN #107 meeting, the revised WID on Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. According to the WID, the objectives of RAN4 scope are duplicated as following.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

In last meeting, there is discussion on RRM requirements for A-IoT, and a WF was approved [2]. This contribution provides discussion on this topic.

- Discussion

Issue 1-1: A-IoT paging Agreement: - RAN4 not to define RRM requirements for A-IoT paging Issue 1-2: A-IoT data transmission Agreement: - Excepting the timing issue discussed in Issue 1-4, no other potential RRM requirement impact is identified for “A-IoT data transmission” Issue 1-3: A-IoT random access procedure Agreement: - RAN4 to include RRM requirements for correct A-IoT device behaviour during CBRA and CFRA. FFS the details

RAN2 running CR on introduction of Ambient IoT is under email discussion, but the baseline is stable and email email discussion is used to capture further agreements. According to RAN2 running CR on TS38.300, A-IoT Random Access procedures are as following.

Figure 1	A-IoT CBRA

Figure 2	A-IoT CFRA

In last meeting, it was agreed that RAN4 will include RRM requirements for correct A-IoT device behaviour during CBRA and CFRA. FFS the details. The selectionof random access type is base on RA Type field in the A-IoT Paging message. In high level, as shown in Figure 1, A-IoT CBRA includes A-IoT MSG1 and A-IoT MSG2. For CBRA, the A-IoT device randomly selects one A-IoT MSG1 resource for transmission of the Random ID among A-IoT MSG1 resources provided in A-IoT paging message, and transmits the A-IoT MSG1 on this resource. After A-IoT MSG1 transmission, the device monitors A-IoT MSG2 from the gNB-reader for contention resolution. Upon successful reception of A-IoT MSG2 which contains the same random ID as transmitted in A-IoT MSG1, the A-IoT device considers the contention resolution as successful. Otherwise, the device considers the A-IoT random access procedure as failure. If we follow the legacy way to capture RRM requirements for correct A-IoT device behaviour during CBRA, the correct behaviour when transmitting A-IoT MSG1, the correct behaviour when receiving A-IoT MSG2, and the correct behaviour when not receiving A-IoT MSG2 are need to be specified.

Proposal 1: for RRM requirements on A-IoT CBRA, it is proposed to consider following aspects:

- the correct behaviour when transmitting A-IoT MSG1

- the correct behaviour when receiving A-IoT MSG2

- the correct behaviour when not receiving A-IoT MSG2

In high level, as shown in Figure 2, A-IoT CFRA includes First D2R message. For CFRA, the A-IoT device shall use the dedicated resource provided in A-IoT paging message to send the first D2R message. If we follow the legacy way to capture RRM requirements for correct A-IoT device behaviour during CFRA, the correct behaviour when transmitting First D2R message is needed.

Proposal 2: for RRM requirements on A-IoT CFRA, it is proposed to consider following aspects:

- the correct behaviour when transmitting First D2R message

Issue 1-4: timing Agreement: - Further discuss whether there is RRM impact on transmit timing requirements of device pending on RAN1 conclusion.

RAN1 agreements are duplicated as following. It can be seen that RAN1 defined the starting time for the first Msg1 time domain resource, which is reflect by Toffset1 . FFS detailed value(s) of Toffset1. RAN1 agreed that for any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective, which means that SFO is not considered in Toffset1. However, SFO will impact device own timing, and device tranmit MSG1 may not on the ideal time instance indicated by  Toffset1. There will be some timing error due to SFO. From this point of view, it is proposed to define transmit timing error requirements, which is impacted due to SFO.

RAN1 agreements in RAN1#120bis Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission. Agreement A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device. Conclusion For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective. - Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion. - Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19 - Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2 Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access.

Proposal 3: it is proposed to define device transmit timing error requirements due to SFO.

In addition to define requirements, how to test is also very important. For legacy UE transmit timing test, the transmit timing is verified by the UE transmitting SRS. After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB. Similarly, for A-IoT, the transmit timing is verified by device D2R transmission. Taking RACH as an example, the test equipment will verify that the timing of the device is within  a period, e.g. (Toffset1 ± Te_SFO),  of the end the corresponding R2D transmission triggering random access.

Proposal 4: For RACH, the transmit timing is verified by the device D2R transmission, and the test equipment will verify that the timing of the device is within a period of the end the corresponding R2D transmission triggering random access, where the period is related with Toffset1 and timing error due to SFO.

- Conclusion

This contribution provides discussion on RRM requirements for A-IoT. The proposals are:

Proposal 1: for RRM requirements on A-IoT CBRA, it is proposed to consider following aspects:

- the correct behaviour when transmitting A-IoT MSG1

- the correct behaviour when receiving A-IoT MSG2

- the correct behaviour when not receiving A-IoT MSG2

Proposal 2: for RRM requirements on A-IoT CFRA, it is proposed to consider following aspects:

- the correct behaviour when transmitting First D2R message

Proposal 3: it is proposed to define device transmit timing error requirements due to SFO.

Proposal 4: For RACH, the transmit timing is verified by the device D2R transmission, and the test equipment will verify that the timing of the device is within a period of the end the corresponding R2D transmission triggering random access, where the period is related with Toffset1 and timing error due to SFO.

- Reference

- RP-250796, Revised WID: New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc

- R4-2504905, WF on RRM requirements for Ambient_IoT_Solutions, CMCC