3GPP TSG-RAN4 Meeting #115          		R4-2506616

Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on measurement period for L1-SRS-RSRP measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the work split of Rel-19 SBFD (R4-2504904), this draftCR provides measurement period for L1-SRS-RSRP measurement. The CR is drafted based on following agreements in RAN4#114bis meeting (R4-2504904): - The number of samples (M) for measurement period: - For periodic and semi-persistent SRS resource, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise - For aperiodic SRS resources M=1 - Measurement restriction - Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options - Option 1: Prioritize L1-SRS-RSRP measurement - Option 2: Prioritize L1-CLI-RSSI measurement - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Provide measurement period for L1-SRS-RSRP measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.x.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of 1st change >>

9.x.1.5	L1-SRS-RSRP measurement period

The UE shall be capable of performing L1-SRS-RSRP measurement based on the configured SRS resource, and the UE shall be capable of reporting SRS-RSRP measured over measurement period of TL1_SRS_RSRP_measurement_period for FR1 and FR2.

Table 9.X.1.5-1 Measurement period TL1_SRS_RSRP_measurement_period

| Configuration | TL1_SRS_measurement_period (ms) |
| --- | --- |
| No DRX | Max(60, M X TSRS) |
| DRX cycle ≤ 320 ms | Max(60, Ceil(1.5 X M) X max(TSRS, TDRX)) |
| DRX cycle > 320 ms | M X TDRX |
| NOTE 1: TSRS is SRS measurement periodicity configured SRS-PeriodicityAndOffset, and TDRX is the DRX cycle length. NOTE 2: For periodic and semi-persistent SRS resource,M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. For aperiodic SRS resources, M=1 |  |

If the SRS resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TL1_SRS_RSRP_measurement_period.

[Editor notes: measurement restriction for the case that measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping will be added later if consensus reached]

## << End of 1st change >>