3GPP TSG-RAN4 Meeting #115          		R4-2506617

Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on event-triggered reporting for CSI-RS based L1 measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the work split of Rel-19 Mobility enhancement (R4-2504965), this draftCR provides event-triggered reporting and event-triggered periodic reporting for CSI-RS based L1 measurement. The CR is drafted based on following previous agreements: Agreements in RAN4#113 meeting (R4-2420101) Issue 2-2: starting point in event triggered L1 report requirements <Agreement in RAN4#112bis> - Use the following as starting point. Wording can be discussed in CR. - Starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting. Issue 2-5: Event triggered reporting delay for L1 reporting <Agreement> - Agree on the following high level proposal: - The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period [on the cells corresponding to the event or on the candidate cells]. - FFS reuse L1-RSRP measurement period from the existing spec with Treport = 0. - Additional time TSSB_time_index for SSB index acquisition is needed, unless deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. Issue 2-3: ending point in event triggered L1 report requirements <Agreement> - Ending point in event triggered L1 report requirements is defined as the point when the UE starts to transmit the first UL transmission to report measurement result over the air interface. Agreements in RAN4#114 meeting (R4-2502601) Issue 2-9: Impact of UL resource waiting time. Agreement: - For event triggered L1 measurement reporting, measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting. Agreements in RAN4#114bis meeting (R4-2504965) Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Reuse the framework from L3 event triggered reporting. Discuss the details further. Issue 4-11: Whether to consider CSI-RS based L1-SINR measurement Agreement: - RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Provide event-triggered reporting and event-triggered periodic reporting for CSI-RS based L1 measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.14a.3.x1, 9.14a.3.x2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of 1st change >>

9.14a.3.x1	Event-triggered Periodic Reporting

Reported CSI-RS based L1-RSRP measurements contained in event-triggered periodic measurement reports shall meet the requirements in clauses 10.x1.y1.z1 for FR1 and 10.x2.y2.z2 for FR2.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.14a.3.x2.

9.14a.3.x2	Event Triggered Reporting

Reported CSI-RS based L1-RSRP measurements contained in event triggered measurement reports shall meet the requirements in clauses clauses 10.x1.y1.z1 for FR1 and 10.x2.y2.z2 for FR2.

The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled.

The measurement reporting delay is defined as the time between the point when a condition exists at the UE which will trigger the reporting and the point when the UE starts to transmit the first UL transmission to report measurement result over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

The event triggered measurement reporting delay, shall be no larger than the CSI-RS based L1-RSRP measurement period [on the cells corresponding to the event or on the candidate cells] defined in clause 9.14a.5.

## << End of 1st change >>