3GPP TSG-RAN WG4 Meeting #115	R4-2506626

St Julian’s, Malta, 19th – 23rd May 2025

Source:	Ericsson

Title:	Discussion on Rel-19 MMSE-IRC BS demodulation requirements

Agenda item:	7.17.4

Document for:	Discussion

# Introduction

In previous RAN4#114-bis meeting, companies had agreement on the open issues on interference modelling, test parameters, and manufacturer declarations for Rel-19 NR MMSE-IRC BS requirement [1]. However, the open issue on applicability rules and DM-RS configuration still exists and is discussed in this contribution.

# Discussion

## 2.1	Applicability Rules

Open Issue:

Applicability for different BS classes and test scenarios

- FFS whether to consider additional applicability rule on how the test is performed:

- Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped

- Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

Agreements:

Online agreement:

- Follow the LTE approach – Only specifies that HomNet requirements are not applicable for Local Area BS.

In the last meeting, most companies wanted to follow the approach for applicability rule as done in LTE TS 36.141, where only homogenous network requirements are specified, which are not applicable for Local Area BS. But since heterogenous network requirements are not mentioned, this implies that for Wide and Medium Area BS, both homogenous and heterogenous requirements are applicable. Therefore, Wide and Medium Range BS would need to pass the test for both requirements, which would eventually increase the test time. Hence additional applicability rule is needed.

Observation 1: Without additional applicability rule, Wide and Medium Area BS needs to pass the test for both homogenous (INR set 1) and heterogenous (INR set 2) network requirements, thereby increasing the test time.

Therefore, considering that homogenous network requirements are stricter compared to heterogenous requirements, we propose option 1 from additional applicability rule.

Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped.

Open Issue:

DMRS configuration - Ratio of PUSCH EPRE to DM-RS EPRE

Option 1: Ratio of PUSCH EPRE to DM-RS EPRE: -3dB with Number of DM-RS CDM group(s) without data = 2

Samsung: With opt 1, there will be 3dB mismatch between data and the estimated interference covariance matrix, under the agreed DMRS based MMSE-IRC receiver assumption.

Other options are not precluded

2.2	DM-RS configuration

As mentioned in the email discussion, the concern arises from the typical Rel-15 setup, where 2 CDM groups are assumed, along with a 3dB power boosting on DM-RS is applied to both serving and interfering cells. This results in a power mismatch between DM-RS REs and data REs, potentially causing a misalignment in the interference covariance matrix estimation, which is typically based on DM-RS REs.

Further, in real deployment each BS manages its own transmission independently and thus has no information if the interfering cell has enabled or disabled DM-RS power boosting.

Observation 2: The serving BS has no information on DM-RS power boosting from the interfering cell.

Table 2.2-1: PUSCH MMSE-IRC requirements with DM-RS Power boosting

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | PUSCH Mapping type | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | A | 16 | 11.52 | 15.26 | 3.74 |
|  |  | TDLC300-100 |  | 5.36 | A |  | 11.75 | 13.45 | 1.70 |

Table 2.2-2: PUSCH MMSE-IRC requirements without DM-RS Power boosting

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | PUSCH Mapping type | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | A | 16 | 11.77 | 15.26 | 3.51 |
|  |  | TDLC300-100 |  | 5.36 | A |  | 12.03 | 13.45 | 1.42 |

To have some analysis, we performed simulations with and without DM-RS power boosting on serving and interfering cell, as given in Table 2.2-1 and 2.2-2.

Observation 3: Based on our simulation results, we do not observe significant variation in performance gain between IRC and MRC, with enabled and disabled DM-RS power boosting.

In the typical test setup, to obtain the MMSE-IRC receiver weight matrix, the covariance matrix including the sources of inter-cell interference needs to be estimated. The covariance matrix is estimated at DM-RS REs. Further, also aligning with LTE, it would be better to have the same assumption for MMSE-IRC processing.

Proposal 2: It does not seem necessary to define BS MMSE-IRC requirements by disabling DM-RS power boosting.

# Conclusion

Based on the discussion, we made the following observation:

Observation 1: Without additional applicability rule, Wide and Medium Area BS needs to pass the test for both homogenous (INR set 1) and heterogenous (INR set 2) network requirements, thereby increasing the test time.

Observation 2: The serving BS has no information on DM-RS power boosting from the interfering cell, as there is no defined signaling.

Observation 3: Based on our simulation results, we do not observe significant variation in performance gain between IRC and MRC, with enabled and disabled DM-RS power boosting.

Based on the discussion, we made the following proposal:

Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped.

Proposal 2: It does not seem necessary to define BS MMSE-IRC requirements by disabling DM-RS power boosting.

# References

[1] R4-2504768, “Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS”, China Telecom, RAN4#114-bis, Wuhan, Hubei, China.