3GPP TSG-RAN WG4 Meeting #115	R4-2506628

# St Julian’s, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | DRAFT | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_demod_Ph5-Perf) Draft CR for 38.141-1 on Base Station supporting enhanced performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There have been agreement in RAN4 meeting to introduce BS supporting enhanced performance requirements. Therefore, it is time to prepare the draft CR to capture corresponding requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the following sections: Sec. C.3: Measurement of performance requirements Sec. D.5.x: Connection diagrams for BS type 1-C performance requirements for MMSE-IRC Sec. D.6.x: Connection diagrams for BS type 1-H performance requirements for MMSE-IRC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would be no derivation of test requirements and functional set-up for performance requirements for BS type 1-C and 1-H supporting enhanced performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.3, D.5.x and D.6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

################## Start of Change #1 ######################

# C.3	Measurement of performance requirements

Table C.3-1: Derivation of Test Requirements (Performance tests)

| Test | Minimum Requirement in TS 38.104 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 8.2.1 Performance requirements for PUSCH with transform precoding disabled | SNRs as specified | 0.6 dB for 1Tx cases 0.8 dB for 2Tx cases 1.0 dB for 4Tx cases | Formula: SNR + TT T-put limit unchanged |
| 8.2.2 Performance requirements for PUSCH with transform precoding enabled | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3 Performance requirements for UCI multiplexed on PUSCH | SNRs as specified | 0.6 dB | Formula: SNR + TT BLER limit unchanged |
| 8.2.4 Performance requirements for PUSCH for high speed train | SNRs as specified | 0.3 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.5 Performance requirements for UL timing adjustment | SNRs as specified | 0.3 dB for AWGN | Formula: SNR + TT T-put limit unchanged |
| 8.2.6 Performance requirements for PUSCH with 0.001% BLER | SNRs as specified | 0.3 dB | Formula: SNR + TT + 1dB 1dB is added to the test requirement to facilitate early test pass. The BLER delivered by the device during the test will be lower than the test requirement, which enables compliance to the requirement to be demonstrated with a number of observed block errors lower than a certain threshold. |
| 8.2.7 Performance requirements for PUSCH repetition Type A | SNRs as specified | 0.6 dB | Formula: SNR + TT BLER limit unchanged |
| 8.2.8 Performance requirements for PUSCH Mapping Type B with non-slot transmission | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.12 Performance requirements for PUSCH TB over Multi-Slots | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.13 Performance requirements for PUSCH with DM-RS bundling | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.14 Performance requirements for PUSCH for ATG | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.15 Performance requirements for PUSCH with enhanced DM-RS | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.16 Performance requirements for PUSCH with synchronous interference | SNRs as specified | [0.6 dB] | Formula: SNR + TT T-put limit unchanged |
| 8.3.1 Performance requirements for PUCCH format 0 | SNRs as specified | 0.6 dB | Formula: SNR + TT False ACK limit unchanged Correct ACK limit unchanged |
| 8.3.2 Performance requirements for PUCCH format 1 | SNRs as specified | 0.6 dB | Formula: SNR + TT False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged |
| 8.3.3 Performance requirements for PUCCH format 2 | SNRs as specified | 0.6 dB | Formula: SNR + TT False ACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 8.3.4 Performance requirements for PUCCH format 3 | SNRs as specified | 0.6 dB | Formula: SNR + TT UCI BLER limit unchanged |
| 8.3.5 Performance requirements for PUCCH format 4 | SNRs as specified | 0.6 dB | Formula: SNR + TT UCI BLER limit unchanged |
| 8.3.6 Performance requirements for multi-slot PUCCH | SNRs as specified | 0.6 dB | Formula: SNR + TT False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged |
| 8.3.12 Performance requirements for PUCCH format 1 with DM-RS bundling | SNRs as specified | 0.6 dB | Formula: SNR + TT False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 8.3.13 Performance requirements for PUCCH format 3 with DM-RS bundling | SNRs as specified | 0.6 dB | Formula: SNR + TT UCI BLER limit unchanged |
| 8.4.1 PRACH false alarm probability and missed detection | SNRs as specified | 0.6 dB for fading cases 0.3 dB for AWGN cases | Formula: SNR + TT PRACH false detection limit unchanged PRACH detection limit unchanged |

################## End of Change #1 ######################

################## Start of Change #2 ######################

D.5.x	Performance requirements for PUSCH transmission in multipath fading propagation conditions with synchronous interference

BS tester

Wanted

signal

Interferer 1

BS under tes

t

Rx A

Rx B

ATT 2

Channel

simulator

Channel

simulator

AWGN

generator

AWGN

generator

ATT 1

Channel

simulator

Channel

simulator

HARQ Feedback

Figure D.5.x-1: Functional set-up for enhanced performance requirements of 2Rx PUSCH in multipath fading conditions with synchronous interference (1 interferer and 2 Rx case shown)

BS tester

Wanted

signal

Interferer 1

Interferer 2

ATT 1

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

AWGN

generator

AWGN

AWGN

generator

AWGN

AWGN

generator

AWGN

AWGN

generator

AWGN

BS under test

Rx A

Rx B

Rx C

Rx D

ATT 2

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

ATT 3

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

HARQ Feedback

Figure D.5.x-2: Functional set-up for enhanced performance requirements of 4Rx PUSCH in multipath fading conditions with synchronous interference (2 interferers and 4 Rx case shown)

BS tester

Want

ed signal

Interferer 1

Interferer 2

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

BS under test

Rx A

Rx B

Rx C

Rx D

Rx E

Rx F

Rx G

Rx H

ATT 1

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simu

lator

Channel

simulator

Channel

simulator

Channel

simulator

ATT 2

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

ATT 3

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

HARQ Feedback

Figure D.5.x-3: Functional set-up for enhanced performance requirements of 8Rx PUSCH in multipath fading conditions with synchronous interference (2 interferers and 8 Rx case shown)

################## End of Change #2 ######################

################## Start of Change #3 ######################

D.6.x	Performance requirements for PUSCH transmission in multipath fading propagation conditions with synchronous interference

Figure D.6.x-1: Functional set-up for enhanced performance requirements of 2Rx PUSCH in multipath fading conditions with synchronous interference (1 interferer and 2 Rx case shown)

BS tester

Wanted

signal

Interferer 1

Interferer 2

ATT 1

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

AWGN

AWGN

AWGN

AWGN

AWGN

AWGN

AWGN

AWGN

#1

ATT 2

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

ATT 3

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

HARQ Feedback

transceiver unit array

Load

Transceiver array boundary connector TAB(n)

#2

#3

#4

#K

generator

generator

generator

generator

Figure D.6.x-2: Functional set-up for enhanced performance requirements of 4Rx PUSCH in multipath fading conditions with synchronous interference (2 interferer and 4 Rx case shown)

BS tester

Want

ed signal

Interferer 1

Interferer 2

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

AWGN

geneartor

AWGN

generator

ATT 1

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simu

lator

Channel

simulator

Channel

simulator

Channel

simulator

ATT 2

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

ATT 3

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

Channel

simulator

HARQ Feedback

transceiver unit array

transceiver array boundary

#1

#2

#3

#4

#5

#6

#7

#8

#K

Load

Transceiver array boundary connector TAB(n)

Figure D.6.x-3: Functional set-up for enhanced performance requirements of 8Rx PUSCH in multipath fading conditions with synchronous interference (2 interferer and 8 Rx case shown)

################## End of Change #3 ######################