3GPP TSG-RAN WG4 Meeting #115	R4-2506631

# St Julian’s, Malta, 19th – 23rd May 2025

Source:	Ericsson

Title:	Simulations on NR NTN less than 5MHz SAN demodulation requirements

Agenda item:	7.10.4.6

Document for:	Information

# Introduction

In RAN4#114-bis meeting, the discussions were kicked-off for SAN demodulations requirements for less than 5 MHz channel bandwidth. Based on the WF [1], we realize it would be feasible to perform simulations on initial SAN PUSCH and PUCCH requirements.

# Simulations

# 2.1 Simulation assumptions and results for PUSCH with transform precoding disabled

Table: 2.1-1 Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth for 5Mhz/ 12 PRBs for 3MHz |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

Table 2.1-2: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 3/5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex D) | Fraction of maximum throughput | SNR (dB) for 3 MHz CBW | SNR (dB) for 5 MHz CBW |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | 1.51 | 0.91 |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | -0.65 | -0.69 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | -2.78 | -3.07 |
|  |  | Normal | NTN-TDLC5-200 Low | 70% | -4.01 | -4.09 |

# 2.2 Simulation assumptions and results for PUCCH format 2 UCI BLER performance requirements

Table 2.2-1: Test Parameters

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) (Note 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Note 1: For 3MHz, the first PRB after frequency hopping is 3. |  |

Table 2.2-2: Minimum requirements for PUCCH format 2, 15 kHz SCS and 3/5 MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex D) | SNR (dB) for 3 MHz CBW | SNR (dB) for 5 MHz CBW |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 8.42 | 4.21 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 0.79 | -1.49 |

# Conclusions

The simulations results are delivered for SAN normal PUSCH and PUCCH format 2 UCI BLER requirements.

# References

[1]   R4-2504753, “WF for NR IoT NTN less than 5MHz demod”, Ericsson, RAN4#114-bis, Wuhan, China.