3GPP TSG-RAN WG4 Meeting #115	R4-2506634

St Julian’s, Malta, 19th – 23rd May 2025

Source:	Ericsson

Title:	Discussion on Rel-19 7 MHz NR FR1 channel bandwidth

Agenda item:	7.8.6

Document for:	Discussion

# Introduction

### 4.1	Objective of SI or Core part WI or Testing part WI

The core part of this WI includes:

- Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4]

  - Introduce the channel bandwidth 7MHz

    - Only include 15 kHz SCS

  - Specify the spectrum utilization

- Enable, if feasible, optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4]

  - No impact to 38.307 is expected.

### 4.2	Objective of Performance part WI

- Specify, if necessary, BS demodulation performance requirements (RAN4)

According to WID [1], it is the time to kick-off demodulation discussion for this WI. The corresponding objective for core part and performance part are copied below.

RAN4#114bis endorsed a CR to introduce new channel bandwidth 7MHz in FDD NR bands n5 and n26 [2], according to WID [1]. In the CR, the maximum transmission bandwidth for 7MHz CBW is specified to 35 RBs.

In this contribution, general view for BS and UE demodulation performance requirements are provided.

# BS Demodulation requirements

Given the large investment in spectrum, especially low band spectrum, it is critical that operators have a way to efficiently utilize all their spectrum. While several approaches were initially proposed for this WID, the agreement was to include only native 7 MHz channel bandwidths (CBWs) for n5 and n26 in Rel-19.

2.1 General

Regarding the 7 MHz channel bandwidth in the overlapping frequency bands n5 and n26, there appear to be no modifications to the uplink physical channels. Consequently, the base station (BS) receiver algorithm is not expected to change.

Observation 1: No uplink physical channel modifications are introduced for 7 MHz channel bandwidth (CBW).

From the demodulation requirement perspective, current PUSCH applicability rule for different channel bandwidth is:

8.1.2.1.2 Applicability of requirements for different channel bandwidths

For each subcarrier spacing declared to be supported, the test requirements for a specific channel bandwidth shall apply only if the BS supports it (see D.14 in table 4.6-1).

Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.

Based on the applicability rule, we already have the requirements for appropriate PUSCH channel bandwidth (i.e, 5, 10 and 20 MHz for 15 KHz SCS). Further, since no significant difference in target SNR has been observed between 5 MHz and 10 MHz channel bandwidths [3], a similar outcome is expected for the 7 MHz bandwidth. Therefore, 5 MHz PUSCH requirement could be considered to test 7 MHz channel bandwidth.

Observation 2: No significant performance difference in target SNR is expected between 5 and 7 MHz channel bandwidth.

Proposal 1: Do not introduce new PUSCH demodulation requirements for 7 MHz CBW with 15 kHz SCS, unless performance difference of target SNR from 5 MHz CBW is observed.

Similarly, the PUCCH applicability rule for different channel bandwidth is:

8.1.2.2.3 Applicability of requirements for different channel bandwidths

For each subcarrier spacing declared to be supported by the BS, the test requirements for a specific channel bandwidth shall apply only if the BS supports it (see D.14 in table 4.6-1).

Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.

It is the same as PUSCH applicability rule, where we also have existing channel bandwidths (i.e., 5, 10 and 20 MHz for 15 KHz SCS) for various PUCCH formats. Therefore, with the same reasoning as PUSCH, it may not be needed to define new PUCCH requirement for 7 MHz channel bandwidth, unless a reasonable performance difference is seen.

Proposal 2: Do not introduce new PUCCH demodulation requirements for 7 MHz CBW with 15 kHz SCS, unless performance difference of target SNR from 5 MHz CBW is observed.

Further regarding PRACH, existing BS demodulation requirements are defined for any channel bandwidth per sub-carrier spacing (SCS). Therefore, there is no need to define new requirements for 7 MHz bandwidth.

Proposal 3: Do not define new PRACH demodulation requirements for 7 MHz channel bandwidth.

2.2 Requirement Scope

As per the WID [1], BS demodulation requirements should be defined for bands- n5 and n26. However, since no physical layer enhancements are expected, limited test cases of PUSCH and PUCCH requirements with 7 MHz could be checked, if target SNR is different compared to 5 MHz channel bandwidth.

As a starting point, if companies agree to define requirements, normal PUSCH and PUCCH requirements can be considered for simulations. As for enhanced features, such as URLLC, NR-U, HST, coverage enhancements etc., they are likely not the primary use cases for 7 MHz bandwidth and may be deprioritized in the context of simulation discussions.

Proposal 4: If PUSCH demodulation requirements with 7 MHz channel bandwidth are agreed to be defined, consider following cases for simulations:

- PUSCH with transform precoding disabled

- PUSCH with transform precoding enabled

Proposal 5: If PUCCH demodulation requirements with 7 MHz channel bandwidth are agreed to be defined, consider following cases for simulations: Format 0/1/2/3/4.

2.3 Specification Impact

As currently in TS 38.104 [2], PUSCH and PUCCH requirements are defined for different channel bandwidth per SCS. A similar approach can be followed for 7 MHz channel bandwidth requirements, if they are defined. Further, new fixed reference channels (FRCs) would be needed as well.

Proposal 6: If 7 MHz channel bandwidth requirements are defined for PUSCH or PUCCH, new FRCs would be needed.

Proposal 7: If 7 MHz channel bandwidth requirements are defined for PUSCH or PUCCH, they can be added to TS 38.104 specification as done for other channel bandwidths, i.e., different bandwidths per SCS.

# UE Demodulation requirements

3.1	PDSCH demodulation requirements

The existing single carrier PDSCH demodulation requirements for FR1 FDD SCS=15kHz is defined with 10MHz channel bandwidth because almost all the FR1 FDD bands support 10MHz CBW. In NR_FR1_7MHz_BW WI, RAN4 has added 7MHz channel bandwidth to the FDD bands n5 and n26 [2]. Since both the bands n5 and n26 support 10MHz CBW according to TS 38.101-1 Table 5.3.5-1, We don’t think RAN4 need to add new single carrier PDSCH demodulation requirements for 7MHz CBW.

Proposal 8: RAN4 does not specify single carrier PDSCH demodulation requirements with 7MHz CBW for FDD SCS=15kHz.

RAN4 also defines CA PDSCH demodulation requirements in TS 38.101-4. The CA demodulation requirements are designed to cover the combinations of any component carrier (CC) specified in the UE RF specifications TS38.101-1/2/3. For example, for FR1 FDD SCS=15kHz, RAN4 defines PDSCH demodulation requirements per CC whose CBW is 5, 10, 15, 20, 25, 30, 35, 40, 45, and 50MHz.

Although RAN4 has not yet defined CA/DC band combinations including 7MHz CC in Rel-19, we think RAN4 should define the PDSCH demodulation requirements with 7MHz CC so that not only all generic requirements are complete but also the band combinations can be added in the future basket WIs without the need for demodulation work. Please note that RAN4 also added the PDSCH demodulation requirements with 35MHz/45MHz CC in Rel-17 without CA/DC basket WIs.

Considering the limited TU and RAN4 workload, we propose to define PDSCH demodulation requirements for 7MHz CC for the basic scenario only, that is, TDLA30-10 scenario.

Proposal 9: RAN4 define the 2Rx/4Rx PDSCH demodulation requirements with 7MHz CBW CC for FDD SCS 15kHz in TS 38.101-4 5.2A.2.1.

Based on the existing FRC defined in TS 38.101-4 Table A.3.2.1.1-9, we propose the simulation parameters shown in Table 1.

Table 1	Simulation parameters for PDSCH demodulation for 7MHz CC

| Band width | SCS | MCS | TBS | Propagation condition | Antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 7MHz (35 RBs) | FDD 15kHz | 64QAM, 0.48 | 17424 | TDLA30-10 | 2x2, ULA Low |
| 7MHz (35 RBs) | FDD 15kHz | 64QAM, 0.48 | 17424 | TDLA30-10 | 2x4, ULA Low |

Proposal 10: Use the simulation assumption shown in Table 1, i.e., FDD 15kHz, 7MHz CBW (35 RBs), MCS19 Rank 2, TDLA30-10, 2x2/2x4 ULA Low.

3.2	SDR requirements

In TS 38.101-4 5.5A, the SDR requirements are applicable for CA scenario and are also applicable for any CA combination. This means the SDR requirements are also applicable for CA including 7MHz CC.

Observation 3: The existing SDR requirements specified in TS 38.101-4 5.5.1 is applicable for any channel bandwidth.

According to TS 38.101-4 Table 5.5A-1, the SDR test configures one OFDM symbol for CORESET configuration. Since the maximum transmission bandwidth for 7MHz CBW is 35 RBs [1], the number of RBs for CORESET is 30 (CORESET RBs should be multiple of 6). Considering two PDCCH transmissions on one slot, the maximum PDCCH aggregation level should be set to AL2.

Proposal 11: Set the number of CORESET PRBs to ‘30’ for FDD 7MHz in TS 38.101-4 Table 5.5A-4.

Proposal 12: Set the number of PDCCH candidates and aggregation levels to ‘2/AL2’ for 15kHz/7MHz in TS 38.101-4 Table 5.5A-1.

3.3	PDCCH/PBCH demodulation requirements

Since the NR bands n5 and n26 support 10MHz CBW, the existing FDD PBCH/PDCCH demodulation requirements are sufficient to verify UE supporting 7MHz CBW. RAN4 does not need add new PDCCH/PBCH demodulation requirements

Proposal 13: No new PDCCH/PBCH demodulation requirements in WI NR_FR1_7MHz_BW.

3.4	CSI reporting requirements

Since the NR bands n5 and n26 support 10MHz CBW, the existing CSI reporting requirements for sufficient to verify UE supporting 7MHz CBW.

Proposal 14: No new CSI reporting requirements (CQI, PMI, RI) for WI NR_FR1_7MHz_BW.

TS 38.101-4 also specifies the CQI reporting requirements in static condition for CA scenario. Since the static CA CQI requirements are designed to apply to any CA combination, it is applicable for the CA including 7MHz CC. For example, the measurement channel configuration considers any channel bandwidth as shown in Table 6.2A.3.1.1-1.

Observation 4: The existing CA CQI reporting requirements in static condition specified in TS 38.101-4 6.2A is applicable for any channel bandwidth.

Proposal 15: Existing CA CQI reporting requirements specified in TS 38.101-4 6.2A.3.1.1 is applicable for CA with 7MHz CBW CC.

TS 38.101-4 Table 6.2A.3.1.1-2 does not specify the CQI reporting sub-band size for 7MHz CBW. Since the existing parameter table configures the subband size to 8 for 5MHz and 10MHz CC CBW, we can also configure the subband size to 8 for 7MHz CBW.

Proposal 16: Set CQI subbandSize to ‘8’ for 7MHz CBW in TS 38.101-4 Table 6.2A.3.1.1-2.

# Conclusion

4.1 BS Demodulation requirements

Based on the above discussion, following observation and proposals are made:

Observation 1: No uplink physical channel modifications are introduced for 7 MHz channel bandwidth (CBW).

Observation 2: No significant performance difference in target SNR is expected between 5 and 7 MHz channel bandwidth.

Proposal 1: Do not introduce new PUSCH demodulation requirements for 7 MHz CBW with 15 kHz SCS, unless performance difference of target SNR from 5 MHz CBW is observed.

Proposal 2: Do not introduce new PUCCH demodulation requirements for 7 MHz CBW with 15 kHz SCS, unless performance difference of target SNR from 5 MHz CBW is observed.

Proposal 3: Do not define new PRACH demodulation requirements for 7 MHz channel bandwidth.

Proposal 4: If PUSCH demodulation requirements with 7 MHz channel bandwidth are agreed to be defined, consider following cases for simulations:

- PUSCH with transform precoding disabled

- PUSCH with transform precoding enabled

Proposal 5: If PUCCH demodulation requirements with 7 MHz channel bandwidth are agreed to be defined, consider following cases for simulations: Format 0/1/2/3/4.Proposal 6: If 7 MHz channel bandwidth requirements are defined for PUSCH or PUCCH, new FRCs would be needed.

Proposal 6: If 7 MHz channel bandwidth requirements are defined for PUSCH or PUCCH, new FRCs would be needed.

Proposal 7: If 7 MHz channel bandwidth requirements are defined for PUSCH or PUCCH, they can be added to TS 38.104 specification as done for other channel bandwidths, i.e., different bandwidths per SCS.

4.2 UE Demodulation requirements

Observation 3: The existing SDR requirements specified in TS 38.101-4 5.5.1 is applicable for any channel bandwidth.

Observation 4: The existing CA CQI reporting requirements in static condition specified in TS 38.101-4 6.2A is applicable for any channel bandwidth.

Proposal 8: RAN4 does not specify single carrier PDSCH demodulation requirements with 7MHz CBW for FDD SCS=15kHz.

Proposal 9: RAN4 define the 2Rx/4Rx PDSCH demodulation requirements with 7MHz CBW CC for FDD SCS 15kHz in TS 38.101-4 5.2A.2.1.

Proposal 10: Use the simulation assumption shown in Table 1, i.e., FDD 15kHz, 7MHz CBW (35 RBs), MCS19 Rank 2, TDLA30-10, 2x2/2x4 ULA Low.

Proposal 11: Set the number of CORESET PRBs to ‘30’ for FDD 7MHz in TS 38.101-4 Table 5.5A-4.

Proposal 12: Set the number of PDCCH candidates and aggregation levels to ‘2/AL2’ for 15kHz/7MHz in TS 38.101-4 Table 5.5A-1.

Proposal 13: No new PDCCH/PBCH demodulation requirements in WI NR_FR1_7MHz_BW.

Proposal 14: No new CSI reporting requirements (CQI, PMI, RI) for WI NR_FR1_7MHz_BW.

Proposal 15: Existing CA CQI reporting requirements specified in TS 38.101-4 6.2A.3.1.1 is applicable for CA with 7MHz CBW CC.

Proposal 16: Set CQI subbandSize to ‘8’ for 7MHz CBW in TS 38.101-4 Table 6.2A.3.1.1-2.

# References

- RP-250373, “Revised WID on NR FR1 7 MHz Channel Bandwidth”, T-Mobile USA, China Telecom, RAN#107, March 2025, Incheon, Korea.

- R4-2505136, “Introduction of 7 MHz channel bandwidth”, Nokia.

- 3GPP TS 38.104, “Base Station (BS) radio transmission and reception (Release 18)”