3GPP TSG-RAN WG4 Meeting #115	   R4-2506663

# St.Julian’s, Malta, 19th – 23rd , May , 2025

Agenda Item:	7.18.4

Source: 	Ericsson

Title:	On Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

From RAN #103 meeting the WID for RRM enhancement phase 5 is listed in [1]. The RAN4#114 bis WF is listed in [2].  In this paper we provide our view for the remaining open issues.

Background: WID objective #2

- Fast SCell activation for UE supporting Rel-18 EMR

- Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting

- Apply fast scell activation in FR1 and FR2-1

- Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

Based on the latest agreement for the open issues from RAN4 114 bis meeting:

Issue 2-1-2: Update requirements of unknown PUCCH SCell activation based on EMR Directly treat the CR (R4-2503864) online. No agreement is needed.

The Rel-19 fast Scell activation requirement enhancement is based on the UE supporting Rel-18 EMR enhancement. The scope including the PUCCH Scell activation based on UE supporting the Rel-18 EMR. This can also be reflected based on the endorsed CR R4-2502685.

One thing to notice is that for PUCCH Scell activation, the requirement is being specified based on not only whether target cell is known to the UE or not but also whether the PL-RS is known to the UE or not.

The PL-RS can be the reference signal used for L3 measurement reporting or the L1-RSRP measurement reporting. The PL-RS can also be QCL-ed typed D with the RS used for L3 / L1-RSRP measurement reporting. This is specified as known condition for the PL-RS in clause 8.3.12.

We think the RS used for Rel-18 EMR measurement reporting can serve for the same purpose to provide the side condition for the PL-RS known for both known and unknown target cell. The detail wording can be seen from our CR R4-2506665 for addressing this issue.

- Proposal 1: For PUCCH Scell activation, the PL-RS known condition can be updated based on the RS used for Rel-18 eEMR reporting.

Issue 1-2-4: UE capability Agreement: • Discuss the UE capability by the end of core part.

The purpose of the Rel-19 RRM enhancement for expedited Scell activation, specifically for UEs supporting Rel-18 eEMR, is to reduce Scell activation delays by leveraging the advanced capabilities introduced in Rel-18.

The endorsed draft CR R4-2420132 has documented the scenarios agreed upon for specification to reduce delays. These scenarios are based on the UE capabilities defined in Rel-18 eEMR and Rel-16 EMR, which serve as the foundation for specifying the Scell activation requirements in Rel-19.

- UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or, - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or, - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode,

The first bullet refers to the Rel-18 UE capability for measurement reporting based on the eEMR configuration. The second bullet highlights the Rel-18 UE capability for supporting cell reselection measurement reporting. The third bullet pertains to the Rel-16 UE capability for measurement reporting based on the EMR measurement configuration.

The scenarios and requirement applicability for the Rel-19 enhancements are well-defined and straightforward. Therefore, we do not see any need to introduce new capabilities that could unnecessarily complicate the requirement specification.

- Proposal 2: RAN4 shall refrain from introducing any new UE capability for Rel-19 fast scell activation for UE supporting Rel-18 eEMR.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: For PUCCH Scell activation, the PL-RS known condition can be updated based on the RS used for Rel-18 EMR reporting.

	Proposal 2: RAN4 shall refrain from introducing any new UE capability for Rel-19 fast scell activation for UE supporting Rel-18 eEMR.

# References

- RP-240830 “New WID: WI resulting from discussion on RRM enhancements”, VIVO, March 2024

- R4-2504900 “RRM pha5 part II WF”, CATT, April, 2025