3GPP TSG-RAN WG4 Meeting #115	   R4-2506667

# St. Julian’s, Malta, 12th – 19th , May, 2025

Agenda Item:	7.25.4.2

Source: 	Ericsson

Title:	Discussion on RRM impacts for SBFD operations (SBFD)

Document for:	Discussion

# Introduction

The agreements on the SBFD CLI handling and SBFD operation during RAN4#114 bis meeting are captured in the approved WF [1].

In this paper we provide our view on the listed open issues on RRM core requirement within related WID [2] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

## 2.1 Other RRM core requirements for SBFD operation

### CSI-RS based L1 measurement

#### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Background: - Different cases for DUD - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). - RAN1 agreements related to CSI processing time Agreement [RAN1#118b] For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.. - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other

For the CSI-RS based L1 measurement, due to the puncturing in the frequency domain for the CSI-RS resources, also due to the CSI-RS resource discontinuity in frequency domain, some company claim that the CSI-RS measurement delay requirement need to be updated.

Based on RAN1 agreement, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy, based on TS 38.214 clause 5.4 if we look at the Z2 number of symbols, regardless of the SCS, the time is around 4 ms.

…

When the CSI-RS resource is configured across two DL sub band, the processing time will become 8ms. We think instead of adding another measurement sample which will be bounded by the periodicity of the measurement resource, it is a better solution to add a lower bound of 8ms.

- Proposal 1: If UE indicates that CSI processing time is scaled by 2, the measurement delay requirement can add a lower bound of 8ms.

#### Issue 2-1-1c: CSI-RS L1 measurement – different symbol types - For L1-RSRP and L1-SINR measurement period, further discuss the following options. - Option 1: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach. - Option 2: other - For RLM/BFD/CBD, further discuss the following options - FFS whether to add Note: For the potential power difference between SBFD symbol and non-SBFD symbol, no RAN4 enhancement on power difference handling in Rel-19.

Due to the CSI-RS resources will be available based on different symbol type and the UE will measure based the network indicated valid symbol type, the CSI-RS based L1 measurement requirement will be impacted. The configuration of the SBFD symbols and non-SBFD symbols in the TDD pattern will impact the measurement period, when network indicates one valid symbols type, the missing of the measurement sample on the other symbols will be known from the configuration.

The NR-U approach to define the measurement period is based on counting the unavailable measurement occasions during the measurement period. Even though the TDD pattern and the SBFD configuration will be known to the UE, the extending measurement period by counting the punctured invalid symbol can be used.

- Proposal 2: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type.

We acknowledge the potential technical issue due to the RF antenna panel design for the SBFD, the potential power differences between the SBFD and non-SBFD symbols perhaps will have impact on the performance, however, due to the fact that the core part of SBFD have been almost to the end, it is quite late to start discuss and addressing this issue properly in Rel-19.

- Proposal 3: Not to specify any RAN4 RRM requirement based on the potential power differences between the SBFD and non-SBFD symbols.

- Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs

Our interpretation of the priority rule agreed on RAN1 is based on the exact OFDM symbols that contains SSB. Within one DL slot the typical configuration of the SSB is starting from symbol#2 and the symbol#8 span over 4 OFDM symbols. In this case the symbols that not covered by the SSB within 1 slot are only 2 OFDM symbols. Also considering there will be switching gaps being introduced from gNB point of view based on the SBFD and non-SBFD slot, the scheduling restriction on these 2 symbols is not necessary.

- Proposal 4: Scheduling restriction on symbol level right after the SSB measurement within the SBFD slot is not necessary.

Issue 1-3-2: DL/UL subband configurations support for SBFD Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = 40 as the default value - For band n39, X = 20MHz is used instead.

We analyze the typical 40MHz total BW case to see how to guarantee the CSI-RS BW configuration figure 1. If we add X3’ value from 24PRBs on top of the 48PRBs we can increase the requirement coverage by 30%, even though still 50% of the potential configuration will end up have no requirements.

Figure 1: typical configuration of the DUD of SBFD PRBs numbers within 1 DL subband

We think 2 set requirements can cover the requirement in a better way and the 3 cases can all be covered.

- Case 1: X1 >= 48PRBs and X2 >= 48PRBs, requirement set 1

- Case 2: X1 >= 48PRBs and 24PRBs<X2 < 48 PRBs, can be further discussed

- Case 3: X1 >= 24PRBs and X2 >= 24PRBs, requirement set 2

- Proposal 6: RAN4 further study how the accuracy requirement for both 48PRBs and 24PRBs for CSI-RS based L1-RSRP and L1-SINR.

#### Issue 2-1-2: Requirements for CSI-RS based L3 measurement - Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

For CSI-RS-based L3 measurements, we believe that without signaling for the neighbor cell SBFD configuration, or without knowing the frequency start point and the number of PRBs allocated for the CSI-RS, it is unclear how an SBFD-aware UE can perform measurements unless it operates as a legacy UE.

For DU configuration, the requirement is simpler. If the CSI-RS is punctured in the frequency domain and does not meet the bandwidth conditions, then no requirements will apply.

However, for DUD configuration, the situation is more complex. When considering legacy UE operation in an SBFD-capable base station, CSI-RS-based L3 measurements will be impacted because certain TDD downlink symbols are unavailable to legacy UEs. As a result, measurement performance will degrade, as certain samples must be dropped. However, this performance degradation is not captured within the work item.

For SBFD-capable UEs, referring to the typical bandwidth based on base station declarations in Figure 1, applying the legacy 48 PRBs bandwidth condition would result in more than 55% of declared configurations not meeting the requirements.

L3 measurement reports are intended for mobility decisions. To ensure fair decision-making between SBFD and non-SBFD capable UEs, we believe it is difficult to specify appropriate requirements for DUD configuration. This difficulty arises because the network cannot guarantee requirement coverage for more than half of the configurations, and SBFD-aware UEs cannot perform measurements based on their SBFD capability due to the lack of neighbor cell SBFD configuration.

The proper resolution to this issue is to request that RAN2 support SBFD-aware UEs by providing the neighbor cell SBFD configuration.

- Proposal 7: RAN2 neighbour cell SBFD configuration is necessary for setting up proper CSI-RS based L3 requirements. RAN4 can send LS to RAN2 to ask for supporting the configuration.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: If UE indicates that CSI processing time is scaled by 2, the measurement delay requirement can add a lower bound of 8ms.

	Proposal 2: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type.

	Proposal 3: Not to specify any RAN4 RRM requirement based on the potential power differences between the SBFD and non-SBFD symbols.

	Proposal 4: Scheduling restriction on symbol level right after the SSB measurement within the SBFD slot is not necessary.

	Proposal 6: RAN4 further study how the accuracy requirement for both 48PRBs and 24PRBs for CSI-RS based L1-RSRP and L1-SINR.

	Proposal 7: RAN2 neighbour cell SBFD configuration is necessary for setting up proper CSI-RS based L3 requirements. RAN4 can send LS to RAN2 to ask for supporting the configuration.

# References

- R4-2504904 “WF on SBFD RRM”, Huawei, April ,2025

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024