3GPP TSG-RAN WG4 Meeting #115	R4-2506676

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.16.3

Source:	Samsung

Title:	Discussion on spatial channel model for demodulation performance requirements

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the SID [1] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved, which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#109 in Sep 2025 [2]. The objectives are as below:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In RAN4#113 meeting, High level TR skeleton for 38.753 and initial work split have been agreed, and obvious progresses in RAN4#114 and RAN4#114bis meeting have been achieved for test cases, parameter assumptions, CDL methodologies and AAV configurations [3][4]. However, there are still several issues need to be further clarified and discussed for the CDL modelling, and the discussion for TDL modelling also need to be more in-depth.

In this contribution, we share our views and analysis on the open issues for CDL-based model and TDL-based model.

- Discussion

  - Common Parameters for all Methodologies

Antenna element spacing

From RAN4#112 meeting to RAN4#114bis meeting, it seems we didn’t have agreements on the antenna element spacing. It’s no doubt different antenna element spacing will lead to different performance. In order to align and compare companies’ simulation results based on the same assumptions, we think it is necessary to confirm the setting of antenna element spacing.

Proposal 1: Clarify and confirm the setting of antenna element spacing

- BS antenna element spacing in horizontal dimension 0.5lambda

- BS antenna element spacing in vertical dimension 0.5lambda

- UE antenna element spacing in horizontal dimension 0.5lambda

  - TDL Based Methodologies

The channel model with spatial property has been discussed at the beginning of NR. Geometric based SCM(E) is defined in 38.901, in which the cluster delays are drawn randomly from the delay distribution, the cluster powers are calculated assuming a single slope exponential power delay profile, the AoA/AoD/ZoA/ZoD are determined by applying the inverse Gaussian/Laplacian function and random variable values with uniform distributions, the rays are randomly coupling within a cluster for both azimuth and elevation, the cross polarization power ratio (XPR) is log-Normal distributed, the initial phases are random uniform within (-). There are too many variables to align results based on this channel model. 38.901 CDL channel model is derived from SCM(E) with some parameters fixed, i.e., cluster delays, cluster powers, AoA/AoD/ZoA/ZoD and XPR are tabulated and fixed. Based on 38.901 CDL channel model, 38.901 TDL model is defined without spatial property (only cluster delays and powers with Rx and Tx radiation pattern effect are kept), and 38.101-4 TDL model is further simplified by removing some clusters.

Currently, the motivation of the SID [1] is that 38.101-4 TDL model does not have spatial property for RAN4 demodulation/CSI reporting requirements, which are needed for some scenarios e.g. High rank, PMI reporting cases.  It is obvious that spatial property is not equal to CDL channel model, since AE radiation patterns, the per sub-cluster AoA/AoD/ZoA/ZoD angles, and even Beam forming are redundant for the spatial property introduction. This target for introducing spatial property could also be achieved by extending current TDL model through introducing spatial clusters as mentioned in [5], for each cluster TX and RX steering corresponding to desired AoD/ZoD and AoA/ZoA.

The general multi-cluster MIMO channel impulse response matrix with dimensions Nrx× Ntx at some time instant is [5]

Hτ=nmpn,m∙Dn(rx)Hn,mDn(tx)∙δτ-τn,m

In which, steering matrix definitions:

- Dn(tx)=1001⊗Dθtx1(n),Ntx1⊗Dθtx2(n),Ntx2

- Dn(rx)=1001⊗Dθrx1(n),Nrx1⊗Dθrx2(n),Nrx2

TDL Steering model

For the steering model, below agreement is achieved in last RAN4#114 meeting [4].

Steering matrix definitions: - Dn(tx)=1001⊗Dθtx1(n),Ntx1⊗Dθtx2(n),Ntx2 - Dn(rx)=1001⊗Dθrx1(n),Nrx1⊗Dθrx2(n),Nrx2 - The steering matrix of size N× N as function of phase θ is defined as - Dθ,N=diag1ejθ⋯ejθ(N-1) - Assuming the antenna arrays employ uniform distance between elements in each linear dimension, the phase shift θ corresponds to AOA or AOD angle φ so that θφ,dλ=2π∙dλ∙sinφ, where d is the distance between adjacent antenna elements and λ is the carrier wavelength. - The steering phases θtx1(n), θtx2(n), θrx1(n), θrx2(n) may be specified either directly or by specifying the corresponding directional angles φtx1(n), φtx2(n), φrx1(n), φrx2(n).

In which the phase shift θ corresponds to AOA or AOD angle φ so that θφ,dλ=2π∙dλ∙sinφ. While, for the Tx and Rx phase shifts defined in the simulation assumption [6], [0, -π/2, -π, -3π/2] are selected for four clusters. When the antenna element spacing defined as 0.5lambda, the maximum value of phase shift θ should be π and the minimum value of phase shift θ should be -π, i.e., -3π/2 is an unattainable value.

Observation 1: When the antenna element spacing defined as 0.5lambda, the maximum value of phase shift θ should be π and the minimum value of phase shift θ should be -π, i.e., -3π/2 is an unattainable value.

Proposal 2: For extended TDL model with 4 clusters, the phase shift θ value -3π/2 should be replaced by other reachable value.

Cluster models

Three cluster models introduced in [4]. For cluster model 1, each cluster is associated with a TDL channel model with full delay profile, so the complexity of the channe model grows with the number of clusters. For cluster model 2, the delay profile of a single TDL channel model is split into multiple clusters, and the complexity of the model remains almost equal to the underlying TDL model. For cluster model 3, it combines the advantages of cluster model 1 and cluster model 2. Nevertheless, we assume the complexity difference between two cluster models is negligible compared with CDL channel model. Hence, the complexity is not a key issue for extended TDL models. Considering the cluster powers, the power difference between layers or codewords of cluster model 1 is clear, while the power difference between layers or codewords of cluster model 2 and cluster model 3 is ambiguous and need further calculation.

Observation 2: The complexity of cluster model 2 is similar as legacy TDL channel model, while the complexity of cluster model 1 grows with the number of clusters. Cluster model 3 combines the advantages of cluster model 1 and cluster model 2.

Observation 3: The power difference between layers or codewords of cluster model 1 is clear, while the power difference between layers or codewords of cluster model 2 and cluster model 3 is ambiguous and need further calculation.

Proposal 3: Follow the Way Forward “focus on option 2 and 1 for alignment, with 4 clusters for all cases, based on companies’ preference” to do performance evaluation and further down selection.

  - CDL Based Methodologies

Frequency Bands

Whether the same CDL channel model could be defined for all carrier frequencies is an important issue. According 38.901 description, at least the delay scaling is different for different carrier frequencies as

| Proposed Scaling Factor DSdesired in [ns] |  | Frequency [GHz] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | 6 | 15 | 28 | 39 | 60 | 70 |
| Indoor office | Short-delay profile | 20 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | Normal-delay profile | 39 | 30 | 24 | 20 | 18 | 16 | 16 |
|  | Long-delay profile | 59 | 53 | 47 | 43 | 41 | 38 | 37 |
| UMi Street-canyon | Short-delay profile | 65 | 45 | 37 | 32 | 30 | 27 | 26 |
|  | Normal-delay profile | 129 | 93 | 76 | 66 | 61 | 55 | 53 |
|  | Long-delay profile | 634 | 316 | 307 | 301 | 297 | 293 | 291 |
| UMa | Short-delay profile | 93 | 93 | 85 | 80 | 78 | 75 | 74 |
|  | Normal-delay profile | 363 | 363 | 302 | 266 | 249 | 228 | 221 |
|  | Long-delay profile | 1148 | 1148 | 955 | 841 | 786 | 720 | 698 |
| RMa & RMa O2I | Short-delay profile | 32 | 32 | N/A | N/A | N/A | N/A | N/A |
|  | Normal-delay profile | 37 | 37 | N/A | N/A | N/A | N/A | N/A |
|  | Long-delay profile | 153 | 153 | N/A | N/A | N/A | N/A | N/A |
| UMi / UMa O2I | Normal-delay profile | 240 |  |  |  |  |  |  |
|  | Long-delay profile | 616 |  |  |  |  |  |  |

For the AoA/AoD/ZoA/ZoD angles, we think different angles should be defined for different carrier frequencies. However, it seems tables CDLA/CDLB/CDLC/CDLD/CDLE are suitable for all carrier frequencies according the description in 38.901 section 7.7.1.

Observation 4: At least the delay scaling is different for different carrier frequencies.

Observation 5: For the AoA/AoD/ZoA/ZoD angles, it seems tables for CDLA/CDLB/CDLC/CDLD/CDLE are suitable for all carrier frequencies according the description in 38.901 section 7.7.1.

Proposal 4: RAN4 to study if the same CDL channel model/table can be defined for all carrier frequencies.

CDL Model for Alignment

After RAN4#114bis meeting, the number of CDL models are down selected to 2, one is the 38.901 CDL model without truncation, the other is truncated table.

- CDL Model for Alignment Way Forward - Truncated CDL Model (commonly referred to as ‘A4’), with confirmation of configuration table after this meeting and checking for issues. - Untruncated 38.901 Model (commonly referred to as ‘A2’) (Note: Companies to state whether using 20 rays per cluster or ray splitting)

For the tables, whatever for the untruncated or truncated tables, some tiny differences may exist between companies. We believe these tiny differences have no impacts on the simulation results, however we need to clearly describe our tables in the TR38.753 finally. Hence we need a clear rule to generate the tables in the same style. Per our understanding, round the powers/angles of each cluster to one decimal is reasonable, beacuse the rounding algorithm has a smaller error than the floor algorithm.

Proposal 5: A clear rule to generate the tables in the same style is needed. Per our understanding, round the powers/angles of each cluster to one decimal is reasonable.

CDL Table Truncation

For the truncated tables, according the truncated CDL Procedure described in Annex A [4], single AAV (subarray or single element), AAS orientation (downtilt, bearing, slant if appliable) are considered in order to do power-based cluster reduction. Therefore, different tables should be generated for different AAV solutions, different AE patterns, different downtilts, and even different slants. Currently, we may only need to consider these factors of BS sides, but with the enhancement on channel model, e.g., UE antenna radiation pattern/UE antenna placement locations modeling introduced in RAN1 as below agreements for Rel-19 channel model, more factors should be considered.

Agreement (RAN1#119) For UE antenna modeling of handheld devices, - Further study on default antenna placement locations for calibration purpose for 2, 3, 4, 6, and 8 antenna elements at least for FR1 and 6-24 GHz, including the need to define default antenna placement locations, and applicability of frequency ranges, e.g. FR1, FR2. - FFS on handling of antenna array panels Agreement (RAN1#119) For handheld UE device type antenna radiation pattern, further study details of antenna field pattern orientation with respect to the direction of antenna for each antenna candidate locations. - For example, maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location.

Observation 6: Single AAV (subarray or single element), AAS orientation (downtilt, bearing, slant if appliable) are considered in order to do power-based cluster reduction. Therefore, different tables should be generated for different AAV solutions, different AE patterns, different downtilts, and even different slants. Currently, we may only need to consider these factors of BS sides, but with the enhancement on channel model, e.g., UE antenna radiation pattern/UE antenna placement locations modeling introduced in RAN1 for Rel-19 channel model, more factors should be considered.

When we do performance evaluation for truncation table with considering many factors (named as separate truncation table solution), we also deploy some simulation cases for a unified truncation table without considering above factors (named as unified truncation table solution), i.e., the truncation only based on power of each cluster. The tables are for both separate truncation table solution and unified truncation table solution list in Annex section. Figure 2-1-Figure 2-4 are the comparison results for separate truncation table solution and unified truncation table solution.

Figure 2-1 Separate truncation table solution vs Unified truncation table solution for PDSCH 4T4R4layer

Figure 2-2 Separate truncation table solution vs Unified truncation table solution for PDSCH 8T8R8layer

Figure 2-3 Separate truncation table solution vs Unified truncation table solution for PMI 8T4R2layer

Figure 2-4 Separate truncation table solution vs Unified truncation table solution for PMI 8T4R4layer

Observation 7: Almost no performance difference between separate truncation table solution and unified truncation table solution for both PDSCH demodulation performance and PMI reporting performance.

Considering there is almost no performance difference between separate truncation table solution and unified truncation table solution, and the tables are easy to align for unified truncation table solution, we propose to use the unified truncation table solution without considering AAV (subarray or single element), AAS orientation (downtilt, bearing, slant if appliable) factors to generate tables in TR38.753.

Proposal 6: Prefer to use the unified truncation table solution without considering AAV (subarray or single element), AAS orientation (downtilt, bearing, slant if appliable) factors in TR38.753.

AAV Configuration for comparison

Way Forward Options: - Option 1 (previously 1Y): with Antenna Array Virtualisation (AAV) using subarray configuration - (M,N,P,Ms,Ns) = (8,2,2,8,1) for 4Tx CSI-RS Ports - (M,N,P,Ms,Ns) = (8,4,2,8,1) for 8Tx CSI-RS Ports - For interested companies: (M,N,P,Ms,Ns)= (16,8,2,8,1) for SU-MIMO eTypeII, for 32 TX, for comparison only - Option 3: with AAV with the following configuration - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

For comparison, both AAV option 1 and option 3 are meaningful. But we don’t think more Tx ports are needed since we are not trying to find cases with maximum performance gain.

Proposal 7: For comparison, both AAV option 1 and option 3 are meaningful. But we don’t think more Tx ports are needed since we are not trying to find cases with maximum performance gain.

SNR Normalisation with CDL

In last RAN4#114bis meeting, below way forward and agreements are reached for SNR normalisation.

Way Forward Assess results alignment at RAN4#115, FFS on the method to define normalisation method if required. Encourage companies to provide normalisation factor at next meeting and align accordingly. Agreement Simulation results to be provided with AAV gain (including AE pattern) normalized, such that AAV gain is taken away as part of the channel normalization.

With the introduction of the BS antenna radiation pattern and AAV beamforming in CDL based channel modelling, the normalization should be applied by a normalization factor and the normalization factor should be calculated considering the long-term average PDP and the beamforming effect for the spatial CDL channel, otherwise an additional gain will be added to the power of channel.

The final BS directivity pattern considering AE radiation pattern, combing weights and the value of down tilt angle. Therefore, compared with calculating the SNR normalization factor directly by channel response, it is somehow more difficult to divide the SNR normalization factor into AE PDP gain factor, BF gain factor and down tilt impact factor, etc. If the SNR normalization factor is not applied, it is hard to compare CDL based SCM options with legacy TDL cases since the different gains, and we may have to define specific requirement for different AAVs and down tilt values.

After round and round discussion, we have the common understanding that the normalisation factor should be measured during the simulation and different values may be measured by different companies. Even companies could provide simulation results with AAV gain (including AE pattern) normalized, we still have to define a value or formula for the normalisation factor finally in TR38.873. One possible solution is to define the average normalisation factor based on interested companies’ contribution. The second possible solution is to define a maximum theoretical value for each case.

Observation 8: The normalisation factor should be measured during the simulation and different values may be measured by different companies is the common understanding after round and round discussion.

Proposal 8: For the normalisation factor finally captured in TR38.873, one possible solution is to define the average normalisation factor based on interested companies’ contribution, another possible solution is to define a maximum theoretical value for each case.

  - Comparison of methodologies

From our companion contribution [7], our observations and proposals could be summarized as below.

Observation 9: 38.753 based CDL option 1Y has similar performance as option 3 with AAV gain (including AE pattern) normalized.

Observation 10: 38.753 based CDL options have similar PDSCH demodulation performance as legacy TDL300-10 XP Medium configuration.

Observation 11: For 4T4R 4layer PDSCH demodulation performance, there are almost no performance difference observed for 753 CDL A2 and A4.

Observation 12: For 8T8R 8layer PDSCH demodulation performance, there are obvious performance difference observed for 753 CDL A2 and A4, e.g., ~3dB for AAV option3 and 1dB for AAV option1Y.

Observation 13: For 8T4R2Layer PMI performance, obvious performance difference could be observed between 38.753 CDL A2 and A4, especially for eType-II codebook.

Observation 14: For 8T4R4Layer PMI performance, obvious performance difference could be observed between 38.753 CDL A2 and A4 with eType-II codebook, the difference of SNR working points is ~2dB.

For doppler 10Hz simulations

SNR normalization factor:

According the SNR normalization factor described in proposal 8, the SNR normalization factors from our simulation results could be summarized as below table, which are related with the number of antennas, AAV options, AE radiation patterns.

| Number of antennas | Models | Normalization factor (linear) | Normalization factor (dB) |
| --- | --- | --- | --- |
| 4Tx4Rx | A2-option3 | 0.88 | -0.56 |
|  | A4-separate-option3 | 0.43 | -3.64 |
|  | A4-unified-option3 | 0.43 | -3.68 |
|  | A2-option1Y | 5.08 | 7.06 |
|  | A4-separate-option1Y | 2.50 | 3.97 |
|  | A4-unified-option1Y | 2.47 | 3.93 |
| 8Tx8Rx | A2-option3 | 0.92 | -0.37 |
|  | A4-separate-option3 | 0.44 | -3.56 |
|  | A4-unified-option3 | 0.44 | -3.59 |
|  | A2-option1Y | 5.17 | 7.13 |
|  | A4-separate-option1Y | 2.49 | 3.95 |
|  | A4-unified-option1Y | 2.46 | 3.91 |
| 8Tx4Rx | A2-option3 | 0.86 | -0.64 |
|  | A4-separate-option3 | 0.43 | -3.70 |
|  | A4-unified-option3 | 0.42 | -3.73 |
|  | A2-option1Y | 5.14 | 7.11 |
|  | A4-separate-option1Y | 2.52 | 4.01 |
|  | A4-unified-option1Y | 2.49 | 3.97 |

- Conclusion

In this contribution, we provide analysis and views on SCM topic.

The observations and proposals could be summarized as:

For Common Parameters for all Methodologies

Proposal 1: Clarify and confirm the setting of antenna element spacing

- BS antenna element spacing in horizontal dimension 0.5lambda

- BS antenna element spacing in vertical dimension 0.5lambda

- UE antenna element spacing in horizontal dimension 0.5lambda

For TDL Based Methodologies

Observation 1: When the antenna element spacing defined as 0.5lambda, the maximum value of phase shift θ should be π and the minimum value of phase shift θ should be -π, i.e., -3π/2 is an unattainable value.

Proposal 2: For extended TDL model with 4 clusters, the phase shift θ value -3π/2 should be replaced by other reachable value.

Observation 2: The complexity of cluster model 2 is similar as legacy TDL channel model, while the complexity of cluster model 1 grows with the number of clusters. Cluster model 3 combines the advantages of cluster model 1 and cluster model 2.

Observation 3: The power difference between layers or codewords of cluster model 1 is clear, while the power difference between layers or codewords of cluster model 2 and cluster model 3 is ambiguous and need further calculation.

Proposal 3: Follow the Way Forward “focus on option 2 and 1 for alignment, with 4 clusters for all cases, based on companies’ preference” to do performance evaluation and further down selection.

For CDL Based Methodologies

Observation 4: At least the delay scaling is different for different carrier frequencies.

Observation 5: For the AoA/AoD/ZoA/ZoD angles, it seems tables for CDLA/CDLB/CDLC/CDLD/CDLE are suitable for all carrier frequencies according the description in 38.901 section 7.7.1.

Proposal 4: RAN4 to study if the same CDL channel model/table can be defined for all carrier frequencies.

Proposal 5: A clear rule to generate the tables in the same style is needed. Per our understanding, round the powers/angles of each cluster to one decimal is reasonable.

Observation 6: Single AAV (subarray or single element), AAS orientation (downtilt, bearing, slant if appliable) are considered in order to do power-based cluster reduction. Therefore, different tables should be generated for different AAV solutions, different AE patterns, different downtilts, and even different slants. Currently, we may only need to consider these factors of BS sides, but with the enhancement on channel model, e.g., UE antenna radiation pattern/UE antenna placement locations modeling introduced in RAN1 for Rel-19 channel model, more factors should be considered.

Observation 7: Almost no performance difference between separate truncation table solution and unified truncation table solution for both PDSCH demodulation performance and PMI reporting performance.

Proposal 6: Prefer to use the unified truncation table solution without considering AAV (subarray or single element), AAS orientation (downtilt, bearing, slant if appliable) factors in TR38.753.

Proposal 7: For comparison, both AAV option 1 and option 3 are meaningful. But we don’t think more Tx ports are needed since we are not trying to find cases with maximum performance gain.

Observation 8: The normalisation factor should be measured during the simulation and different values may be measured by different companies is the common understanding after round and round discussion.

Proposal 8: For the normalisation factor finally captured in TR38.873, one possible solution is to define the average normalisation factor based on interested companies’ contribution, another possible solution is to define a maximum theoretical value for each case.

Observation 9: 38.753 based CDL option 1Y has similar performance as option 3 with AAV gain (including AE pattern) normalized.

Observation 10: 38.753 based CDL options have similar PDSCH demodulation performance as legacy TDL300-10 XP Medium configuration.

Observation 11: For 4T4R 4layer PDSCH demodulation performance, there are almost no performance difference observed for 753 CDL A2 and A4.

Observation 12: For 8T8R 8layer PDSCH demodulation performance, there are obvious performance difference observed for 753 CDL A2 and A4, e.g., ~3dB for AAV option3 and 1dB for AAV option1Y.

Observation 13: For 8T4R2Layer PMI performance, obvious performance difference could be observed between 38.753 CDL A2 and A4, especially for eType-II codebook.

Observation 14: For 8T4R4Layer PMI performance, obvious performance difference could be observed between 38.753 CDL A2 and A4 with eType-II codebook, the difference of SNR working points is ~2dB.

- Reference

[1] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, RAN#104 meeting, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

[2] RP-250577, Revised SID: Study on spatial channel model for demodulation performance requirements, RAN#107 meeting, Nokia, BT Plc

[3] R4-2502378, Way Forward for [114][322]NR_SCM, RAN4#114 meeting, Nokia

[4] R4-2504756, Way Forward for [114bis][320] NR_SCM, RAN4#114bis meeting, Nokia

[5] R4-2500337, TP to TR38.753: TDL approaches and related Annex, RAN4#114 meeting, MediaTek Inc.

[6] R4-2504762, Simulation assumptions for SCM, RAN4#114bis meeting, Huawei, HiSilicon, MediaTek

[7] R4-2506677, Simulation results on spatial channel model, RAN4#115 meeting, Samsung

- Annex

Tables used for RAN4#115 meeting SCM simulation

A4-separate-option3

Cluster #; Cluster delay (ns); Power in [dB]; AOD in [°]; AOA in [°]; ZOD in [°]; ZOA in [°]. 1; 0; -7.4; -38.2; -96.9; 96.8; 98.1. 2; 132; -1.2; -21.8; 118.9; 98.5; 71.1. 3; 138; -5.5; -34.2; -124.4; 100.9; 67.6. 4; 402; 0.0; -5.9; 171.2; 99.2; 76.6. 5; 416; -10.4; 44.2; 51.9; 106.4; 62.8. 6; 502; -10.1; -50.6; 63.4; 94.5; 56.6. 7; 518; -13.7; 49.1; -42.1; 107.5; 69.8. 8; 776; -8.1; -44.2; 65.1; 104.6; 103.4. 9; 826; -9.8; -50.4; -63.4; 105.2; 50.1. 10; 1372; -11.7; -60.2; 79.0; 91.7; 51.7. 11; 1712; -16.2; 64.6; 26.3; 105.2; 121.0. 12; 2906; -16.9; 55.0; -1.6; 91.8; 61.6. Per-Cluster Parameters. Parameter; cASD in [°]; cASA in [°]; cZSD in [°]; cZSA in [°]; XPR in [dB]. Value; 1.4; 15.6; 3.6; 12.2; 7...

A4- separate-option1Y

Cluster #; Cluster delay (ns); Power in [dB]; AOD in [°]; AOA in [°]; ZOD in [°]; ZOA in [°]. 1; 0; -7.4; -38.2; -96.9; 96.8; 98.1. 2; 122; -1.2; -21.8; 118.9; 98.5; 71.1. 3; 126; -5.5; -34.2; -124.4; 100.9; 67.6. 4; 370; 0.0; -5.9; 171.2; 99.2; 76.6. 5; 384; -10.4; 44.2; 51.9; 106.4; 62.8. 6; 462; -10.1; -50.6; 63.4; 94.5; 56.6. 7; 478; -13.7; 49.1; -42.1; 107.5; 69.8. 8; 714; -8.1; -44.2; 65.1; 104.6; 103.4. 9; 762; -9.8; -50.4; -63.4; 105.2; 50.1. 10; 1264; -11.7; -60.2; 79.0; 91.7; 51.7. 11; 2478; -16.9; 62.2; -22.6; 94; 53.9. 12; 2678; -16.9; 55.0; -1.6; 91.8; 61.6. Per-Cluster Parameters. Parameter; cASD in [°]; cASA in [°]; cZSD in [°]; cZSA in [°]; XPR in [dB]. Value; 1.4; 15.6; 3.6; 12.2; 7..

A4-unified

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 164 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 170 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 500 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 516 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 622 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 644 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 732 | -14.1 | -73 | 43.1 | 92.3 | 50.8 |
| 9 | 964 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 10 | 1026 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 11 | 1702 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 12 | 2126 | -16.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |