3GPP TSG-RAN WG4 Meeting #115	R4-2506679

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.17.3

Source:	Samsung

Title:	Discussion on UE demodulation and CSI performance requirements for 8Rx with MMSE-IRC

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the revised WID [1] for “NR demodulation performance: Phase 5” has been approved. According the plan, this WI is targeting to be finished in RAN#109 meeting on Sep 2025.

Generally, the enhancements for UE demodulation performance focus on two scenarios, the first one is the inter-cell interference with MMSE-IRC receiver scenario, the second one is the intra-cell inter-user interference with MMSE-IRC receiver scenario.

RAN4#112 meeting is the first meeting for this topic, several high-level agreements are achieved [2]. Cover CQI reporting requirements under ICI for 8Rx is agreed and most parameters for simulation assumptions are agreed in RAN4#112bis [3] and RAN4#113 [4], more agreements about scenario coverage and test parameters are achieved in [5][6].

In this contribution, we provide our further analysis and views on the open issues for PDSCH demodulation performance parts and simulation assumptions for CQI part.

- Discussion

  - General

8Rx UE MMSE-IRC receiver coverage

In previous meetings, the criteria for decision if one or two sets PDSCH requirements to be introduced is agreed, but the value of threshold need further discussion.

8Rx UE MMSE-IRC receiver coverage Agreement: - Down select the [limit] to one of 0.5dB or 1.0dB. Remaining proposals: - Option 1: Introduce one more rule on the top of the agreed criteria: After reviewing the simulation results for all the test cases by the agreed criteria, if 2 sets of requirements are required for at least one test case, define two sets of PDSCH requirements for all cases, which are applicable for both baseline and simplified receivers.

As current definition of baseline/simplified 8Rx receiver is based on UE declaration, the corresponding requirements for each receiver should have obvious difference, otherwise the so-called declaration on different receiver is meaningless. Therefore, we prefer to set the threshold as at least 1dB.

Proposal 1: For the criteria for decision if one or two sets of PDSCH requirements are to be introduced, we prefer to set the threshold as 1.0dB.

For the further rule ” if 2 sets of requirements are required for at least one test case, define two sets of PDSCH requirements for all cases, which are applicable for both baseline and simplified receivers.” From the perspective of normative consistency, we agree with this proposal. However, from the necessity standpoint, we do not fully endorse it. We prefer to treat it case by case.

Proposal 2: For the further rule “if 2 sets of requirements are required for at least one test case, define two sets of PDSCH requirements for all cases, which are applicable for both baseline and simplified receivers”, we don’t think it’s necessary. We prefer to define requirements case by case.

  - Wideband CQI reporting requirements with inter-cell interference

CQI calculation algorithm

CQI calculation algorithm and Receiver coverage Agreement: - Companies are encouraged to provide suggestions on methodology for deciding CQI reporting requirements in next meeting. - Decision to have one or two sets of requirements is postponed to next meeting when more simulation results are available.

Due to similar reason as the issue for one or two sets of PDSCH requirements, we think two sets requirements should be based on obvious performance difference between baseline receiver and simplified receiver.

Proposal 3: For the requirements for CQI reporting, separate requirements for two receiver types should be defined once the gamma value has obvious difference, i.e., the gamma value difference is more than [0.1].

  - Applicability rule and release independent

Test applicability to skip 2/4Rx requirements

There is no conclusion on the issue for test applicability to skip 2/4Rx requirements. The way forward as below.

Test applicability to skip 2/4Rx requirements Proposals - Option 1: For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not. - Option 2: To consider if UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 3: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 4: Introduce test applicability under inter-cell interference and under intra-cell inter user interference to skip a 2/4Rx requirement if at least the following applies for each considered 8Rx test: - UE support both 8Rx and 2/4Rx - The UE passes an 8Rx requirement with same rank and interference configuration as the 2/4Rx requirement. - Option 5: RAN4 shall have the following applicability rules: - UE that passes Rank2 ICI test case with 8Rx shall skip Rank1 ICI requirements with 4Rx. - UE that passes Rank2 MU-MIMO test case with 8Rx shall skip Rank1/2 MU-MIMO requirements with 4Rx.

In order to balance the test coverage and test workload, for both demodulation performance tests and CQI reporting tests, we prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. We don’t think the condition with the same test configurations are valid, for example, for inter-cell interference scenario, for legacy 2Rx/4Rx test cases, only rank1 with MCS13 is defined. While for 8Rx, rank2 with MCS17 and rank4 with MCS13 test cases are defined.

Observation 1: For inter-cell interference scenario, only rank1 with MCS13 test cases are defined for legacy 2Rx/4Rx test, while rank2 with MCS17 and rank4 with MCS13 test cases are defined for 8Rx. It’s not valid to define test applicability rule to skip legacy cases with the same test configuration.

Proposal 4: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Release independency

For the release independency, when we introduce the performance requirements for 8Rx in Rel-18, below release independency is introduced in 38.307.

Table 5.4-1: Additional requirements of other release independent features Feature; Release independent from; Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced); Further information. RRM requirements for high speed train scenario; Rel-15 (NOTE 1); Table C.1-1; Rel-16 WI NR_HST introduced band independent RRM requirements: see Table C.1-1. UE demodulation requirements for high speed train scenario; Rel-15 (NOTE 1); Table C.2-1; Rel-16 WI NR_HST introduced band independent UE demodulation requirements: see Table C.2-1. RF requirements for 4Rx UEs; Rel-15; Table B.4.10-1, Table B.4.10-2. Transparent Tx diversity; Rel-15; Table B.4.11-1; Rel-17 WI NR_RF_TxD introduced transparent Tx diversity requirements: see Table B.4.11-1. UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference; Rel-15; Table B.3.3-1; Rel-17 WI NR_demod_enh2-Perf: see Table B.3.3-1. These requirements are optional for Rel-15 and Rel-16 UEs and can be executed based on UE declaration.. RRM enhancement for FR1 high speed train scenario enhancement; Rel-16 (NOTE 2); Table C.3-1; Rel-17 WI NR_HST_FR1_enh introduced band independent RRM enhancement: see Table C.3-1. HST-SFN CA demodulation enhancement for FR1 high speed train scenario enhancement; Rel-16 (NOTE 3); Table C.4-1; Rel-17 WI NR_HST_FR1_enh introduced band independent HST-SFN CA demodulation enhancement: see Table C.4-1. HST-DPS CA demodulation enhancement for FR1 high speed train scenario enhancement; Rel-15 (NOTE 4); Table C.4-1; Rel-17 WI NR_HST_FR1_enh introduced band independent HST-DPS CA demodulation enhancement: see Table C.4-1. PDSCH absolute physical layer throughput requirements with link adaptation; Rel-17; Table B.3.4-1; Rel-18 WI NR_demod_enh3-Perf: see Table B.3.4-1. These requirements are optional for Rel-17 and can be executed based on UE declaration.. RF requirements for 8Rx UE; Rel-17; Table B.4.x-1, Table B.4.x-2. Demodulation requirements for 8Rx UE; Rel-17; Table B.3.5-1. NOTE 1: Rel-15 UEs supporting the high speed train are assumed to read the Rel-16 high speed train scenario information, which is broadcast to all UEs. NOTE 2: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 3: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 4: Rel-15 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs..

Observation 2: When we introduce the demodulation performance requirements for 8Rx in Rel-18, the release independent from Rel-17 is defined.

Following this logic, although 8Rx is introduced in Rel-18, the demodulation requirement release independent is defined from Rel-17. We think it is reasonable to define the new requirements for 8Rx should be release independent from Rel-17.

Proposal 5: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-17.

- Conclusion

In this contribution, we provide analysis and views on NR demodulation performance: Phase 5.

The observations and proposals could be summarized as:

Proposal 1: For the criteria for decision if one or two sets of PDSCH requirements are to be introduced, we prefer to set the threshold as 1.0dB.

Proposal 2: For the further rule “if 2 sets of requirements are required for at least one test case, define two sets of PDSCH requirements for all cases, which are applicable for both baseline and simplified receivers”, we don’t think it’s necessary. We prefer to define requirements case by case.

Proposal 3: For the requirements for CQI reporting, separate requirements for two receiver types should be defined once the gamma value has obvious difference, i.e., the gamma value difference is more than [0.1].

Observation 1: For inter-cell interference scenario, only rank1 with MCS13 test cases are defined for legacy 2Rx/4Rx test, while rank2 with MCS17 and rank4 with MCS13 test cases are defined for 8Rx. It’s not valid to define test applicability rule to skip legacy cases with the same test configuration.

Proposal 4: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Observation 2: When we introduce the demodulation performance requirements for 8Rx in Rel-18, the release independent from Rel-17 is defined.

Proposal 5: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-17.

- Reference

[1] RP-241297, Revised WID on NR demodulation performance: Phase 5, RAN#104 meeting, China Telecom, NTT DOCOMO

[2] R4-2413570, WF on NR demodulation performance: Phase 5, RAN4#112 meeting, China Telcom

[3] R4-2416555, WF for [112bis][320] NR_demod_Ph5_Part2_UE, RAN4#112bis meeting, Nokia

[4] R4-2419784, Way forward for [113][322] NR_demod_Ph5_Part2_UE, RAN4#113 meeting, Nokia

[5] R4-2502275, Way forward for [114][324] NR_demod_Ph5_Part2_UE, RAN4#114 meeting, Nokia

[6] R4-2504694, Way forward for [114bis][322] NR_demod_Ph5_Part2_UE, RAN4#114bis meeting, Nokia