3GPP TSG-RAN WG4 Meeting #115	R4-2506681

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.24.6

Source:	Samsung

Title:	RAN4 demodulation and CSI reporting performance work scope for NR MIMO Phase 5

Document for:	Discussion

1		Introduction

Several new features introduced in Rel-19 NR MIMO Phase 5 [1] which can be categorized as following:

- Objective 1: UE-initiated/event-driven beam management

- Objective 2: CSI enhancement up to 128 CSI-RS ports

- Objective 3: UE reporting enhancement for CJT

- Objective 4: 3-antenna-port codebook-based UL transmissions

- Objective 5: Asymmetric DL sTRP/UL mTRP

As stated in the WID, RAN4 needs to specify necessary performance requirements for the enhancements listed above. RAN4 work plan including RF, RRM and performance part was endorsed in [2] which also provided in annex for reference.

In this contribution, overall RAN4 performance scope was analyzed with detailed proposals.

2		Discussion

2.1	Objective 1: UE-initiated/event-driven beam management

The detailed objective 1 as following:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

This objective targeting to achieve more timely beam reports with reduced reporting overhead. With UE-initiated/event-driven beam management, if the UE determines that e.g., current beam(s) quality becomes poor, UE can trigger beam reporting without the network needing to configure or trigger frequent reporting. Rel-19 MIMO supports both Mode A and Mode B for UE-initiated beam reporting, where upon determination of an event, they transmit a one-bit indication in a PUCCH resource to indicate a beam report. In Mode A, the UE transmits the beam report on a PUSCH indicated by a DCI in response to the one-bit indication, while in Mode B, the UE transmits the beam report on a type1 CG PUSCH associated to the PUCCH resource.

Clearly UE-initiated/event-driven beam management related enhancement related to RRM scope which already discussed over RRM session. There is no performance impact foreseen from both BS side and UE side.

Proposal 1: There is no performance impact (UE demodulation/CSI and BS demodulation) for objective 1: “UE-initiated/event-driven beam management”.

2.2	Objective 2: CSI enhancement up to 128 CSI-RS ports

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

In the current NR, each CSI resource can be configured with maximum 32 ports. Therefore, legacy MIMO CSI codebook such as Type I and Type II codebook only supports maximum 32 ports. There are two CSI enhancement in the past that allows support of more than 32 ports for multi-TRP operation. The first enhancement is the Rel-17 Type I codebook enhancement for Single-DCI Multi-TRP NCJT scheme 1a, i.e., SDM scheme that allows up to 2 TRPs, i.e., maximum total 64 ports across 2 TRPs. The other enhancement is Rel-18 Type II codebook enhancement for CJT that allows up to 4 TRPs, i.e., maximum total 128 ports across 4 TRPs.

To increase spectrum efficiency and capacity, it is targeted to support large antenna arrays by enhancement to CSI reporting and hybrid beamforming with up to 128 CSI-RS ports. In current specifications, the support for such large antenna arrays with large number of CSI-RS ports for CSI measurement and reporting is limited. It is also justified to facilitate more-than-4-layer DL reception, especially in TDD scenarios, via a 6/8Rx low complexity receiver by supporting a proper association between SRS ports and the two codewords. Considering the issue of coverage and higher downlink spectrum efficiency, in this objective, four CSI reporting enhancements are discussed for CSI support for up to 128 CSI-RS ports for FR1:

The first one is ‘Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources’ for the Type-I codebook enhancement. For UE supporting typeI-SinglePanel-r19, new UE function should be deployed to support up to 128 CSI-RS port codebook estimation and reporting. Consequently, we need to introduce new PMI reporting requirements for enhanced codebook ‘typeI-SinglePanel-r19’ (FR1 only).

The second one is ‘Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources’ which is on target to enhance Type-II codebook. For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II and Rel-18 Type-II Doppler for up to 128 CSI-RS ports are agreed in RAN1, as well as Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports. Hence, for UE supporting eTypeII-r19 and typeII-Doppler-r19, new UE function should be deployed to support up to 128 CSI-RS port codebook estimation and reporting. As a result, we need to introduce new PMI reporting requirements for enhanced codebook ‘eTypeII-r19’ and typeII-Doppler-r19 (FR1 only).

The third one is ‘CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources’ which is on target to enhance CRI-based CSI reporting. It is no doubt that this enhancement has impact on CSI reporting performance. However, considering no CRI-based CSI reporting requirements defined in legacy specification, we don’t prefer to introduce up to 128 ports CSI reporting requirements for CRI-based CSI reporting.

The last one is ‘SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers’ which is targeting to facilitate more-than-4-layer DL reception. Considering the basic rule is no enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback and DCI format, and no SRS related demodulation/CSI performance requirements are defined in current specification, we don’t think new requirements should be defined for this enhancement.

Proposal 2: Prefer to introduce new PMI reporting requirements for enhanced codebook ‘typeI-SinglePanel-r19’ (FR1 only).

Proposal 3: Prefer to introduce new PMI reporting requirements for enhanced codebook ‘eTypeII-r19’ (FR1 only).

Proposal 4: Prefer to introduce new PMI reporting requirements for enhanced codebook ‘typeII-Doppler-r19’ (FR1 only).

Proposal 5: Prefer not to introduce up to 128 ports CSI reporting requirements for CRI-based CSI reporting.

Proposal 6: Prefer not to define new demodulation/CSI reporting requirements for the enhancement on SRS port grouping and its association.

2.3	Objective 3: UE reporting enhancement for CJT

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

This objective targeting to the UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul. In Rel-18, Type-II CSI has been enhanced to accommodate CJT assuming ideal synchronization and backhaul. Scenarios such as inter-site CJT and a base station equipped with distributed remote radio heads (RRHs) require additional delay and phase/frequency calibration. For TDD, additional inter-TRP DL/UL calibration across TRPs is also beneficial to ensure proper DL/UL reciprocity holds. As the UE possesses more knowledge on DL channel condition in both FDD and TDD, the need for inter-TRP calibration reporting measured from CSI-RS is evident not only to expand the deployment scenarios, but also to offer additional robustness to CJT operation. For inter-TRP delay and frequency/phase calibration for coherent joint transmission (CJT), CSI-RS per TRP is transmitted to the UE to measure the CSI-RS and estimate the reporting quantity including inter-TRP delay offset, frequency offset, or phase offset. A CSI report associated with the reporting quantity is carried on a PUSCH. Accordingly, new UE function to estimate and report delay offset/frequency offset/phase offset are needed. In addition, for the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI. If UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report, new UE function to compensate it before CSI reporting is also needed.

At the same time, agreement to introduce new RRM requirements is achieved in RAN4 [3]

Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration? Agreement: - Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified)

Hence, whether to introduce new CSI reporting metric and requirements for CSI reporting performance need further discussion.

Observation 1: New UE function to estimate and report delay offset/frequency offset/phase offset are needed in order to support Rel-19 UE reporting enhancement for CJT.

Observation 2: If UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report, new UE function to compensate it before CSI reporting is also needed.

Observation 3: RAN4 agreement to define RRM core requirements for delay and frequency offset reporting achieved in RAN4#114bis meeting.

Proposal 7: For UE reporting enhancement for CJT, further discussion on whether to introduce new CSI reporting metric and requirements for CSI reporting performance.

At the same time, for PDSCH, some new transmission schemes are agreed for CJT scenarios in RAN1#116bis as following. It is obvious that different transmission schemes have different impact on PDSCH demodulation performance. Therefore, further discussion is needed to clarify if new PDSCH demodulation requirements should be defined for PDSCH transmission Scheme C/Scheme D/Scheme E.

Agreement For a UE indicated with two TCI states, regarding QCL assumptions for PDSCH, at least the following are supported: - Scheme C: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift} - Scheme D: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {average delay} Per Rel-18, the support of two TCI states is a UE capability Agreement For a UE indicated with two TCI states, regarding QCL assumptions for PDSCH, support the following QCL assumption for PDSCH: - Scheme E: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift, average delay}

Proposal 8: For UE reporting enhancement for CJT, further discussion on whether to introduce new PDSCH demodulation performance requirements for PDSCH transmission Scheme C/Scheme D/Scheme E.

2.4 Objective 4: 3-antenna-port codebook-based UL transmissions

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

NR supports 1-port, 2-port, 4-port and 8-port PUSCH. Nonetheless, current commercial mobile UEs are generally equipped with only 1 or 2 Tx antennas. To enhance the UL performance and anticipating advancements in hardware and design technology, the emergence of UEs equipped with 3 Tx antennas is a foreseeable development in the near future. Consequently, it is advantageous to augment the NR standards to support 3-port PUSCH supporting up to 3 layers. it is straightforward to introduce PUSCH demodulation performance requirements for UE which support 3Tx.

Proposal 9: For UE supporting 3Tx, introduce PUSCH demodulation performance requirements.

2.5 Objective 5: Asymmetric DL sTRP/UL mTRP

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

For asymmetric DL single-TRP and UL multi-TRP scenario where UL TRP may reduce or even turn off DL transmission, pathloss offset is indicated by TCI state for PUSCH, PUCCH, and SRS transmission, and is indicated by a PDCCH order for the PDCCH order triggered PRACH toward UL TRP facilitating pathloss calculation, and up to two closed loops are supported for SRS separate with PUSCH. This enhancement has no impact on demodulation/CSI reporting performance.

Proposal 10: There is no performance impact (UE demodulation/CSI and BS demodulation) for objective 5: “Asymmetric DL sTRP/UL mTRP”.

3		Conclusion

This contribution provides overall RAN4 work scope for Rel-19 NR MIMO Phase 5 WI on demodulation/CSI reporting performance part.

Proposal 1: There is no performance impact (UE demodulation/CSI and BS demodulation) for objective 1: “UE-initiated/event-driven beam management”.

Proposal 2: Prefer to introduce new PMI reporting requirements for enhanced codebook ‘typeI-SinglePanel-r19’ (FR1 only).

Proposal 3: Prefer to introduce new PMI reporting requirements for enhanced codebook ‘eTypeII-r19’ (FR1 only).

Proposal 4: Prefer to introduce new PMI reporting requirements for enhanced codebook ‘typeII-Doppler-r19’ (FR1 only).

Proposal 5: Prefer not to introduce up to 128 ports CSI reporting requirements for CRI-based CSI reporting.

Proposal 6: Prefer not to define new demodulation/CSI reporting requirements for the enhancement on SRS port grouping and its association.

Observation 1: New UE function to estimate and report delay offset/frequency offset/phase offset are needed in order to support Rel-19 UE reporting enhancement for CJT.

Observation 2: If UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report, new UE function to compensate it before CSI reporting is also needed.

Observation 3: RAN4 agreement to define RRM core requirements for delay and frequency offset reporting achieved in RAN4#114bis meeting.

Proposal 7: For UE reporting enhancement for CJT, further discussion on whether to introduce new CSI reporting metric and requirements for demodulation/CSI reporting performance.

Proposal 8: For UE reporting enhancement for CJT, further discussion on whether to introduce new PDSCH demodulation performance requirements for PDSCH transmission Scheme C/Scheme D/Scheme E.

Proposal 9: For UE supporting 3Tx, introduce PUSCH demodulation performance requirements.

Proposal 10: There is no performance impact (UE demodulation/CSI and BS demodulation) for objective 5: “Asymmetric DL sTRP/UL mTRP”.

Table 1: Overview on performance scope of Rel-19 MIMO Phase 5 WI

| Objectives | UE demodulation and CSI | BS demodulation |
| --- | --- | --- |
| Objective 1: UE-initiated/event-driven beam management | No impact | No impact |
| Objective 2: CSI enhancement up to 128 CSI-RS ports | PMI reporting requirements for enhanced codebook ‘typeI-SinglePanel-r19’, ‘eTypeII-r19’, ‘typeII-Doppler-r19’ (FR1 only) | No impact |
| Objective 3: UE reporting enhancement for CJT | FFS on whether to introduce new CSI reporting metric and requirements for CSI reporting performance (FR1 only) FFS on whether to introduce new PDSCH demodulation performance requirements for PDSCH transmission Scheme C/Scheme D/Scheme E. | No impact |
| Objective 4: 3-antenna-port codebook-based UL transmissions | No impact | 3Tx PUSCH demodulation requirements |
| Objective 5: Asymmetric DL sTRP/UL mTRP | No impact | No impact |

Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2412134, Work plan for Rel-19 NR MIMO Phase 5, Samsung

[3] R4-2504903, WF on RRM requirements for NR_MIMO_Ph5_Part2, RAN4#114bis, MediaTek.

Annex

Based on the assigned TU, general RAN4 work plan for Rel-19 MIMO Phase 5 WI is proposed:

| Meeting | Target Plan |  |
| --- | --- | --- |
| RAN4#112 (Aug, ’24) | - Agree overall work plan for core and performance part - Discuss and identify potential impact on UE RF requirements - Discuss and identify potential impact on RRM core requirements |  |
| RAN4#112-bis (Oct, ’24) | UE RF | - Initiate discussion on UE RF requirements of identified impact - Continue to check other impact on UE RF requirement |
|  | RRM Core | - Discuss and identify which RRM requirements need to be developed to support MIMO Phase 5 |
| RAN4#113 (Nov, ’24) | UE RF | - Continue to discuss UE RF requirements of identified impact |
|  | RRM Core | - Further discussions on UE RRM requirements impact for MIMO evolution features - Discuss the possible signalling impact (e.g. UE capabilities, network flag) |
| RAN4#114 (Feb, ’25) | UE RF | - Continue to discuss and define UE RF requirements of identified impact |
|  | RRM Core | - Continue to discuss and define RRM requirements of identified impacts |
| RAN4#114-bis (Apr, ’25) | UE RF | - Continue to discuss and define UE RF requirements of identified impact - Review draft CRs for endorsement if any |
|  | RRM Core | - Continue to discuss and define RRM requirements of identified impacts |
| RAN4#115 (May, ’25) | UE RF | - Try to agree RAN4 CR to finalize the RF core requirements |
|  | RRM Core | - Continue to discuss and decide the solution for the RRM requirements - Review draft CRs for endorsement if any |
|  | RRM perf | - Initial discussion on RRM performance part for MIMO evolution |
|  | Demod | - Initial discussion on demod/CSI requirements, discuss and identify which demod/CSI requirements need to be extended |
| RAN4#116 (Aug, ’25) | UE RF | - Agree RAN4 CR to finalize the RF core requirements |
|  | RRM Core | - Agree RAN4 CR to finalize the RRM core requirements |
|  | RRM perf | - Further discussion on RRM performance part for MIMO Phase 5 |
|  | Demod | - Further discussion on demod/CSI requirements, define scenario parameters in order to align simulation cases |
| Core part completion |  |  |
| RAN4#116-bis (Oct, ’25) | RRM perf | - Further discussion on RRM performance part for MIMO Phase 5, |
|  | Demod | - Further discussion on demod/CSI requirements, provide initial simulation results to align results from companies, CR split |
| RAN4#117 (Nov, ’25) | RRM perf | - Provide draft CR on TS38.133 for RRM performance part, endorsed if possible. |
|  | Demod | - Provide simulation results and continue to discuss demod/CSI requirements, provide draft CR, endorsed if possible. |
| RAN4#118 (Feb, ’26) | RRM perf | - Further work on the draft CRs and Agree RAN4 CR to finalize the RRM performance part |
|  | Demod | - Further work on the draft CRs and Agree RAN4 CR to finalize the demod/CSI part |
| Performance part completion |  |  |