3GPP TSG-RAN WG4 Meeting #115	R4-2506682

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.24.6

Source:	Samsung

Title:	Discussion on Rel-19 NR MIMO Phase 5 UE demodulation performance and CSI requirements

Document for:	Discussion

- Introduction

In the RAN plenary RAN#102 meeting the WID [1] for Rel-19 NR MIMO Phase 5 has been approved and further revised in RAN#105 meeting [2]. This is our first meeting on demodulation and CSI reporting performance requirements for Rel-19 NR MIMO Phase 5 WI. General impacts of each objective are described in [3] and the potential impacts on UE demodulation performance and CSI requirements are as below.

Overview on UE performance scope of Rel-19 MIMO Phase 5 WI

| Objectives | UE demodulation and CSI |
| --- | --- |
| Objective 1: UE-initiated/event-driven beam management | No impact |
| Objective 2: CSI enhancement up to 128 CSI-RS ports | PMI reporting requirements for enhanced codebook ‘typeI-SinglePanel-r19’, ‘eTypeII-r19’, ‘typeII-Doppler-r19’ (FR1 only) |
| Objective 3: UE reporting enhancement for CJT | FFS on whether to introduce new CSI reporting metric and requirements for CSI reporting performance (FR1 only) FFS on whether to introduce new PDSCH demodulation performance requirements for PDSCH transmission Scheme C/Scheme D/Scheme E. |
| Objective 4: 3-antenna-port codebook-based UL transmissions | No impact |
| Objective 5: Asymmetric DL sTRP/UL mTRP | No impact |

In this contribution, we provide our analysis and views on the requirement assumptions for objectives which have potential impacts.

In addition, from rapporteur perspective, the initial work plan for Rel-19 MIMO Phase 5 demodulation/CSI reporting performance part are provided in [4].

- Type-I codebook refinement up to 128 CSI-RS ports

In general, for Type-I codebook enhancement in Rel-19, two new codebook Types are supported as typeI-SinglePanel-r19 and typeI-MultiPanel-r19 targeting on FR1. Considering we didn’t define PMI reporting requirement for multi panel scenario before, we think it is not suitable to define PMI reporting requirements for typeI-MultiPanel-r19 at Rel-19. Hence, we only need to consider the assumptions typeI-SinglePanel-r19.

Duplex mode

Since the objective description in WID [1][2] clearly mentioned this objective is targeting for FR1. Both FR1 FDD and FR1 TDD should support related PMI reporting requirements for typeI-SinglePanel-r19.

Proposal 1: For typeI-SinglePanel-r19, the PMI reporting requirements of both FR1 FDD and FR1 TDD should be defined.

Number of Rx antennas

Since typical number of Rx antennas are 2 and 4 for FR1, the new PMI reporting requirements for typeI-SinglePanel-r19 should be defined for both 2 and 4 Rx antennas.

Proposal 2: For typeI-SinglePanel-r19, the PMI reporting requirements of both 2 and 4 Rx antennas should be defined.

Propagation channel and correlation configuration

As majority of legacy PMI reporting requirements of Type-I Single-Panel codebook are based on TDLA30-5 and XP High, we think it is a good starting point to use the same configuration.

Proposal 3: Use TDLA30-5 with XP high as the propagation channel and correlation configuration for typeI-SinglePanel-r19 test.

Number of CSI-RS ports

For typeI-SinglePanel-r19, 48, 64 and 128 CSI-RS ports are introduced in RAN1. Agreements in RAN1#116bis meeting as below. Accordingly, at least 64 ports PMI reporting requirements should be defined for typeI-SinglePanel-r19. At the same time, considering 4, 8, 16, 32 CSI-RS ports PMI reporting requirements are defined for legacy typeI-SinglePanel codebook, i.e., the requirements for maximum number of CSI-RS ports 32 is defined in current specification. We think the requirements for maximum number of Rel-19 CSI-RS ports 128 should also be supported.

Agreement For the Rel-19 Type-I codebook refinement for 48, 64, and 128 CSI-RS ports, the (N1,N2) values for P=64 are supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE capabilities

Proposal 4: For typeI-SinglePanel-r19, the PMI reporting requirements of basic feature P=64 and the maximum P=128 should be defined.

N1 and N2 configuration

The following (N1, N2) values agreed in RAN1#116 meeting.

Agreement For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II and Rel-18 Type-II Doppler for up to 128 CSI-RS ports, as well as Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, support the following (N1, N2) values: Total # CSI-RS ports across aggregated resources (=P); (N1, N2). 48; (8,3). (6,4). 64; (16,2). (8,4). 128; (16,4). (8,8).

Considering the meaningful deployment in practice, for 64 CSI-RS ports we prefer to set (N1, N2)= (8,4), and for 128 CSI-RS ports we prefer to set  (N1, N2)= (16,4).

Proposal 5: For typeI-SinglePanel-r19, the PMI reporting requirements for 64 CSI-RS ports set (N1, N2)= (8,4), and for 128 CSI-RS ports set  (N1, N2)= (16,4).

Number of aggregated NZP CSI-RS resources

For more than 32 CSI-RS ports, CSI-RS resources aggregation of K NZP CSI-RS resource agreed in RAN1#117 meeting.

Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding aggregation of K NZP CSI-RS resources to attain 32 < P (or PCSI-RS) ≤ 128, support only the following combinations of K and P (or PCSI-RS): - For P (or PCSI-RS) = 48, K = 2 (each resource 24 ports) and 3 (each resource 16 ports) - For P (or PCSI-RS) = 64, K = 2 (each resource 32 ports) and 4 (each resource 16 ports) - For P (or PCSI-RS) = 128, K = 4 (each resource 32 ports)

For 64 CSI-RS ports set K=2 (each resource 32 ports), and for 128 CSI-RS ports set K=4 (each resource 32 ports) for the test assumption of typeI-SinglePanel-r19.

Proposal 6: For typeI-SinglePanel-r19, the PMI reporting requirements for 64 CSI-RS ports set K=2 (each resource 32 ports), and for 128 CSI-RS ports set K=4 (each resource 32 ports).

typeI-codebookMode-r19

For the codebook Mode of Type-I codebook, Scheme-A and Scheme-B was introduced in RAN1#116bis meeting.

Agreement For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=1-4, support the following: - Scheme-A (based on Scheme1 in RAN1#116 agreement): Adding new (N1, N2) values for the Rel-15 Type-I single-panel codebook mode-1 (L=1) where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and for rank-3/4, follow legacy mechanisms for <16 ports - Scheme-B (based on Scheme2 in RAN1#116 agreement): Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and - W1 structure: - For each layer, reuse legacy Rel-16 eType-II SD basis with L=1 to determine the DFT-based SD basis candidates - For 1<RI≤4, L=1 SD basis vector is independently selected for different layers - The SD basis selection indication includes layer-common (q1,q2) and log2N1N2 bits for each layer - Note: This implies that each of the SD basis vectors is selected from a group of N1N2 orthogonal basis vectors - W2 structure: Layer-specific inter-polarization co-phasing with the alphabet {+1, +j, -1, -j} FFS (RAN1#116bis): For Rel-19 Type-I SP, whether to support Mode-C based on Scheme5 in RAN1#116 agreement with L=1 for RI=2-4 FFS (RAN1#116bis): For Rel-19 Type-I SP, whether inter-polarization amplitude for Mode-B can also be supported FFS: Discuss further if Rel-19 Type-I MP extension based on scheme 4 is needed

For Scheme-A, the RI=1-4 extends the (N1, N2) values of legacy Type-I codebook. There is limitation on the selection of the second beam, which reduces the implementation complexity. However, this also potentially degrades performance when more beams are available for larger than 32 ports. Meanwhile, for RI 5-8, the second, third and possible fourth beams are selected freely as long as they are orthogonal in at least one dimension. This increases the number of candidates precoders thus increases complexity. At the same time, the performance can be improved.

For Scheme-B RI=1-4, UE can select the beams and also the inter-polarization co-phasing freely for each layer, which improves performance while increases number of candidates precoders thus the implementation complexity. Meanwhile for Scheme-B RI=5-8, the beams can only be selected from N1*N2 orthogonal beams assuming a common offset (q1, q2). And the inter-polarization co-phasing is also limited for layers sharing the same beam. This can reduce the number of candidates precoders thus reduce the implementation complexity, however, the performance is also degraded potentially.

Considering Scheme-A follows the design principle of Rel-15 Type-I single-panel and the RI restriction for PMI reporting is usually assumed no more than 4, less implementation complexity scheme is preferred for Type-I single panel codebook. Therefore, Scheme-A is our first priority selection.

Proposal 7: For typeI-SinglePanel-r19, the Scheme-A is preferred for the PMI reporting requirements.

typeI-SinglePanel-ri-restriction-r19

The RI restriction as same as typeI-SinglePanel-ri-Restriction in R15 typeI-SinglePanel, it is an 8bit bitmap. Considering rank 2 is enough to verify the implementation of new codebooks, we prefer to set the RI restriction as 00000010.

Proposal 8: For typeI-SinglePanel-r19, set typeI-SinglePanel-ri-restriction-r19 as 00000010.

Other parameters

For other parameters not listed above, reuse the setting for Single PMI with 32TX TypeI-SinglePanel Codebook as the starting point.

Proposal 9: For other parameters of typeI-SinglePanel-r19, reuse the setting for Single PMI with 32TX TypeI-SinglePanel Codebook as the starting point.

- Type-II codebook refinement up to 128 CSI-RS ports

For Type-II codebook refinement up to 128 CSI-RS ports, eTypeII-r19, typeII-FePortSelection-r19 and typeII-Doppler-r19 are introduced in RAN1. Related new PMI reporting requirements are needed for UE supporting such kind of codebook types, however, since we didn’t define test cases for port selection codebook in legacy due to rare deployment, we still don’t plan to introduce new PMI reporting requirements for typeII-FePortSelection-r19. For eTypeII-r19 and typeII-Doppler-r19 codebooks, below test assumptions need further discussion separately.

  - Assumptions for eTypeII-r19

Duplex mode

Since the objective description in WID [1][2] clearly mentioned this objective is targeting for FR1. Both FR1 FDD and FR1 TDD should support related PMI reporting requirements for eTypeII-r19.

Proposal 10: For eTypeII-r19, the PMI reporting requirements of both FR1 FDD and FR1 TDD should be defined.

Number of Rx antennas

Since typical number of Rx antennas are 2 and 4 for FR1, the new PMI reporting requirements for eTypeII-r19 should be defined for both 2 and 4 Rx antennas.

Proposal 11: For eTypeII-r19, the PMI reporting requirements of both 2 and 4 Rx antennas should be defined.

Propagation channel and correlation configuration

As majority of legacy PMI reporting requirements of eType-II codebook are based on TDLA30-5 and XP Medium, we think it is a good starting point to use the same configuration.

Proposal 12: Use TDLA30-5 with XP Medium as the propagation channel and correlation configuration for eTypeII-r19 test.

Number of CSI-RS ports

For eTypeII-r19, 48, 64 and 128 CSI-RS ports are introduced in RAN1. Agreements in RAN1#116 meeting as below. Accordingly, at least 64 ports PMI reporting requirements should be defined for eType-II as 64 CSI-RS ports is the basic feature.

Agreement For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II and Rel-18 Type-II Doppler for up to 128 CSI-RS ports, as well as Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, support the following (N1, N2) values: Total # CSI-RS ports across aggregated resources (=P); (N1, N2). 48; (8,3). (6,4). 64; (16,2). (8,4). 128; (16,4). (8,8). The support of total # CSI-RS ports across aggregated resources (=P) and (N1, N2) are subject to UE capability. - For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II regular codebook, the (N1,N2) values for P=64 are supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE capabilities - For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler regular codebook, the (N1,N2) values for P=64 are supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE capabilities

Proposal 13: For eTypeII-r19, the PMI reporting requirements of basic feature P=64 should be defined.

N1 and N2 configuration

The following (N1, N2) values agreed in RAN1#116 meeting.

Agreement For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II and Rel-18 Type-II Doppler for up to 128 CSI-RS ports, as well as Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, support the following (N1, N2) values: Total # CSI-RS ports across aggregated resources (=P); (N1, N2). 48; (8,3). (6,4). 64; (16,2). (8,4). 128; (16,4). (8,8).

Considering the meaningful deployment in practice, for 64 CSI-RS ports we prefer to set (N1, N2)= (8,4).

Proposal 14: For eTypeII-r19, the PMI reporting requirements for 64 CSI-RS ports set (N1, N2)= (8,4).

Number of aggregated NZP CSI-RS resources

For more than 32 CSI-RS ports, CSI-RS resources aggregation of K NZP CSI-RS resource agreed in RAN1#117 meeting.

Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding aggregation of K NZP CSI-RS resources to attain 32 < P (or PCSI-RS) ≤ 128, support only the following combinations of K and P (or PCSI-RS): - For P (or PCSI-RS) = 48, K = 2 (each resource 24 ports) and 3 (each resource 16 ports) - For P (or PCSI-RS) = 64, K = 2 (each resource 32 ports) and 4 (each resource 16 ports) - For P (or PCSI-RS) = 128, K = 4 (each resource 32 ports)

For 64 CSI-RS ports set K=2 (each resource 32 ports) for the test assumption of eTypeII-r19.

Proposal 15: For eTypeII-r19, the PMI reporting requirements for 64 CSI-RS ports set K=2 (each resource 32 ports).

typeII-ri-restriction-r19

The RI restriction as same as Rel-16, Rel-17 and Rel-18 type-II codebooks, it is a 4bit bitmap. Considering rank 2 is enough to verify the implementation of new codebooks, we prefer to set the RI restriction as 0010.

Proposal 16: For eTypeII-r19, set typeII-ri-restriction-r19 as 0010 as the starting point.

numberOfPMI-SubbandsPerCQI-subband-r19

This parameter has the same meaning as legacy R in Rel-16/Rel-17/Rel-18 Type-II codebook, we propose to use 1 as legacy setting.

Proposal 17: For eTypeII-r19, set numberOfPMI-SubbandsPerCQI-subband-r19 as 1 as the starting point.

paramCombination-r19

This parameter is the codebook parameter combination. We assume the options for eTypeII-r19 are as same as legacy Rel-16 TypeII codebook as below. Hence it is a good selection to reuse 6 as the starting point.

Codebook parameter configurations for L,  β and pυ

| paramCombination-r16 | L | pυ |  | β |
| --- | --- | --- | --- | --- |
|  |  | υ ∈1,2 | υ ∈3,4 |  |
| 1 | 2 | ¼ | 1/8 | ¼ |
| 2 | 2 | ¼ | 1/8 | ½ |
| 3 | 4 | ¼ | 1/8 | ¼ |
| 4 | 4 | ¼ | 1/8 | ½ |
| 5 | 4 | ¼ | ¼ | ¾ |
| 6 | 4 | ½ | ¼ | ½ |
| 7 | 6 | ¼ | - | ½ |
| 8 | 6 | ¼ | - | ¾ |

Proposal 18: For eTypeII-r19, set paramCombination-r19 as 6 (L =4, pν =1/2, β=1/2) as the starting point.

  - Assumptions for typeII-Doppler-r19

Duplex mode

Since the objective description in WID [1][2] clearly mentioned this objective is targeting for FR1. Both FR1 FDD and FR1 TDD should support related PMI reporting requirements for typeII-Doppler-r19.

Proposal 19: For typeII-Doppler-r19, the PMI reporting requirements of both FR1 FDD and FR1 TDD should be defined.

Number of Rx antennas

Since typical number of Rx antennas are 2 and 4 for FR1, the new PMI reporting requirements for typeII-Doppler-r19 should be defined for both 2 and 4 Rx antennas.

Proposal 20: For typeII-Doppler-r19, the PMI reporting requirements of both 2 and 4 Rx antennas should be defined.

Propagation channel and correlation configuration

As legacy PMI reporting requirements of doppler codebook are based on TDLA30-20 and XP Medium, we think it is a good starting point to use the same configuration.

Proposal 21: Use TDLA30-20 with XP Medium as the propagation channel and correlation configuration for typeII-Doppler-r19 test.

Number of CSI-RS ports

For typeII-Doppler-r19, 48, 64 and 128 CSI-RS ports are introduced in RAN1. Agreements in RAN1#116 meeting as below. Accordingly, at least 64 ports PMI reporting requirements should be defined for typeII-Doppler-r19 as 64 CSI-RS ports is the basic feature.

Agreement For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II and Rel-18 Type-II Doppler for up to 128 CSI-RS ports, as well as Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, support the following (N1, N2) values: Total # CSI-RS ports across aggregated resources (=P); (N1, N2). 48; (8,3). (6,4). 64; (16,2). (8,4). 128; (16,4). (8,8). The support of total # CSI-RS ports across aggregated resources (=P) and (N1, N2) are subject to UE capability. - For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II regular codebook, the (N1,N2) values for P=64 are supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE capabilities - For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler regular codebook, the (N1,N2) values for P=64 are supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE capabilities

Proposal 22: For typeII-Doppler-r19, the PMI reporting requirements of basic feature P=64 should be defined.

N1 and N2 configuration

The following (N1, N2) values agreed in RAN1#116 meeting.

Agreement For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II and Rel-18 Type-II Doppler for up to 128 CSI-RS ports, as well as Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, support the following (N1, N2) values: Total # CSI-RS ports across aggregated resources (=P); (N1, N2). 48; (8,3). (6,4). 64; (16,2). (8,4). 128; (16,4). (8,8).

Considering the meaningful deployment in practice, for 64 CSI-RS ports we prefer to set (N1, N2)= (8,4).

Proposal 23: For typeII-Doppler-r19, the PMI reporting requirements for 64 CSI-RS ports set (N1, N2)= (8,4).

Number of aggregated NZP CSI-RS resources

For more than 32 CSI-RS ports, CSI-RS resources aggregation of K NZP CSI-RS resource agreed in RAN1#117 meeting.

Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding aggregation of K NZP CSI-RS resources to attain 32 < P (or PCSI-RS) ≤ 128, support only the following combinations of K and P (or PCSI-RS): - For P (or PCSI-RS) = 48, K = 2 (each resource 24 ports) and 3 (each resource 16 ports) - For P (or PCSI-RS) = 64, K = 2 (each resource 32 ports) and 4 (each resource 16 ports) - For P (or PCSI-RS) = 128, K = 4 (each resource 32 ports)

For 64 CSI-RS ports set K=2 (each resource 32 ports) for the test assumption of typeII-Doppler-r19.

Proposal 24: For typeII-Doppler-r19, the PMI reporting requirements for 64 CSI-RS ports set K=2 (each resource 32 ports).

typeII-ri-restriction-r19

The RI restriction as same as Rel-16, Rel-17 and Rel-18 type-II codebooks, it is a 4bit bitmap. Considering rank 2 is enough to verify the implementation of new codebooks, we prefer to set the RI restriction as 0010.

Proposal 25: For typeII-Doppler-r19, set typeII-ri-restriction-r19 as 0010 as the starting point.

numberOfPMI-SubbandsPerCQI-subband-r19

This parameter has the same meaning as legacy R in Rel-16/Rel-17/Rel-18 Type-II codebook, we propose to use 1 as legacy setting.

Proposal 26: For typeII-Doppler-r19, set numberOfPMI-SubbandsPerCQI-subband-r19 as 1 as the starting point.

The number of configured resource groups Kdopp

For the number of configured resource groups, each resource group comprises K NZP CSI-RS resources, where K={2,3,4}. The available values are 4, 8 and 12. We prefer to use 4 as the starting point, which is as same as the setting for Rel-18 doppler codebook.

Proposal 27: For typeII-Doppler-r19, the number of configured resource groups Kdopp set as 4 as the starting point.

- UE reporting enhancement for CJT

The coherent joint transmission (CJT) offers downlink spectral efficiency and coverage gain. In Rel-18, Type-II CSI has been enhanced to accommodate CJT assuming ideal synchronization and backhaul. Scenarios such as inter-site CJT and a base station equipped with distributed remote radio heads (RRHs) require additional delay and phase/frequency calibration. According to RAN1 progress in Rel-19, the CSI enhancement with UE measurement and reporting on delay offset, frequency offset and phase offset for CJT deployments under non-ideal synchronization and backhaul are agreed. As a result, the timing/frequency/phase misalignment among TRPs could be calibrated. It’s no doubt new UE functions for delay/phase/frequency offset measurement and reporting will be introduced in order to support the Rel-19 enhancement of CJT.

In addition, for the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI. If UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report, new UE function to compensate it before CSI reporting is also needed.

Proposal 28: The measurement and reporting of delay/phase/frequency offset for Rel-19 enhancement of CJT have impact on CSI reporting performance.

At the same time, some new PDSCH transmission schemes are agreed for CJT scenarios in RAN1#116bis as following. It is obvious that different transmission schemes have different impact on PDSCH demodulation performance. Therefore, further discussion is needed to clarify if new PDSCH demodulation requirements should be defined for PDSCH transmission Scheme C/Scheme D/Scheme E.

Agreement For a UE indicated with two TCI states, regarding QCL assumptions for PDSCH, at least the following are supported: - Scheme C: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift} - Scheme D: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {average delay} Per Rel-18, the support of two TCI states is a UE capability Agreement For a UE indicated with two TCI states, regarding QCL assumptions for PDSCH, support the following QCL assumption for PDSCH: Scheme E: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift, average delay}

Proposal 29: The new PDSCH transmission Scheme C/Scheme D/Scheme E for Rel-19 enhancement of CJT have impact on PDSCH demodulation performance.

- Conclusion

In this contribution, we provide analysis and views on UE demodulation performance and CSI requirements for Rel-19 NR MIMO Phase 5 WI.

The proposals and observations could be summarized as:

For typeI-SinglePanel-r19

Proposal 1: For typeI-SinglePanel-r19, the PMI reporting requirements of both FR1 FDD and FR1 TDD should be defined.

Proposal 2: For typeI-SinglePanel-r19, the PMI reporting requirements of both 2 and 4 Rx antennas should be defined.

Proposal 3: Use TDLA30-5 with XP high as the propagation channel and correlation configuration for typeI-SinglePanel-r19 test.

Proposal 4: For typeI-SinglePanel-r19, the PMI reporting requirements of basic feature P=64 and the maximum P=128 should be defined.

Proposal 5: For typeI-SinglePanel-r19, the PMI reporting requirements for 64 CSI-RS ports set (N1, N2)= (8,4), and for 128 CSI-RS ports set  (N1, N2)= (16,4).

Proposal 6: For typeI-SinglePanel-r19, the PMI reporting requirements for 64 CSI-RS ports set K=2 (each resource 32 ports), and for 128 CSI-RS ports set K=4 (each resource 32 ports).

Proposal 7: For typeI-SinglePanel-r19, the Scheme-A is preferred for the PMI reporting requirements.

Proposal 8: For typeI-SinglePanel-r19, set typeI-SinglePanel-ri-restriction-r19 as 00000010.

Proposal 9: For other parameters of typeI-SinglePanel-r19, reuse the setting for Single PMI with 32TX TypeI-SinglePanel Codebook as the starting point.

For eTypeII-r19

Proposal 10: For eTypeII-r19, the PMI reporting requirements of both FR1 FDD and FR1 TDD should be defined.

Proposal 11: For eTypeII-r19, the PMI reporting requirements of both 2 and 4 Rx antennas should be defined.

Proposal 12: Use TDLA30-5 with XP Medium as the propagation channel and correlation configuration for eTypeII-r19 test.

Proposal 13: For eTypeII-r19, the PMI reporting requirements of basic feature P=64 should be defined.

Proposal 14: For eTypeII-r19, the PMI reporting requirements for 64 CSI-RS ports set (N1, N2)= (8,4).

Proposal 15: For eTypeII-r19, the PMI reporting requirements for 64 CSI-RS ports set K=2 (each resource 32 ports).

Proposal 16: For eTypeII-r19, set typeII-ri-restriction-r19 as 0010 as the starting point.

Proposal 17: For eTypeII-r19, set numberOfPMI-SubbandsPerCQI-subband-r19 as 1 as the starting point.

Proposal 18: For eTypeII-r19, set paramCombination-r19 as 6 (L =4, pν =1/2, β=1/2) as the starting point.

For typeII-Doppler-r19

Proposal 19: For typeII-Doppler-r19, the PMI reporting requirements of both FR1 FDD and FR1 TDD should be defined.

Proposal 20: For typeII-Doppler-r19, the PMI reporting requirements of both 2 and 4 Rx antennas should be defined.

Proposal 21: Use TDLA30-20 with XP Medium as the propagation channel and correlation configuration for typeII-Doppler-r19 test.

Proposal 22: For typeII-Doppler-r19, the PMI reporting requirements of basic feature P=64 should be defined.

Proposal 23: For typeII-Doppler-r19, the PMI reporting requirements for 64 CSI-RS ports set (N1, N2)= (8,4).

Proposal 24: For typeII-Doppler-r19, the PMI reporting requirements for 64 CSI-RS ports set K=2 (each resource 32 ports).

Proposal 25: For typeII-Doppler-r19, set typeII-ri-restriction-r19 as 0010 as the starting point.

Proposal 26: For typeII-Doppler-r19, set numberOfPMI-SubbandsPerCQI-subband-r19 as 1 as the starting point.

Proposal 27: For typeII-Doppler-r19, the number of configured resource groups Kdopp set as 4 as the starting point.

For UE reporting enhancement for CJT

Proposal 28: The measurement and reporting of delay/phase/frequency offset for Rel-19 enhancement of CJT have impact on CSI reporting performance.

Proposal 29: The new PDSCH transmission Scheme C/Scheme D/Scheme E for Rel-19 enhancement of CJT have impact on PDSCH demodulation performance.

- Reference

[1] RP-234007, New WID: NR MIMO Phase 5, RAN#102 meeting, Samsung

[2] RP-242394, WID revision: NR MIMO Phase 5, RAN#105 meeting, Samsung

[3] R4-2506681, RAN4 demodulation and CSI reporting performance work scope for NR MIMO Phase 5, RAN4#115 meeting, Samsung

[4] R4-2412134, Work plan for Rel-19 NR MIMO Phase 5, RAN4#112 meeting, Samsung

[5] R4-2504903, WF on RRM requirements for NR_MIMO_Ph5_Part2, RAN4#114bis, MediaTek.