3GPP TSG-RAN WG4 Meeting #115	R4-2506684

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.28.7

Source:	Samsung

Title:	Discussion on UE demodulation requirements for NR LP-WUS and receiver

Document for:	Discussion

- Introduction

The Rel-19 WI on “New WID: Enhancements of network energy savings for NR” was approved in the RAN#102 meeting and revised in the RAN# meeting [2] along with the completion of the Rel-18 SI on low-power wake-up signal and receiver for NR [3], in which the performance part starts from RAN4#115 in May 2025 and targeting to complete by RAN#111 in March 2026. The objectives for performance part are as below:

- Specify the UE demodulation performance and test cases for LP-WUS/WUR (RAN4). - Study testability of above requirements - Specify RRM measurement performance requirement if identified, and corresponding test cases for the LP-WUS/WUR (RAN4).

In this contribution, we share our views and analysis on the general test scope and assumptions for demodulation requirements of NR LP-WUS and receiver.

- Discussion

Whether to introduce new demodulation requirements for LP-WUS

As specified in [1], OOK (On-Off Keying) with overlaid OFDM sequences over OOK symbol was decided as the waveform of LP-WUS.

In RAN1 #116, the following agreement was adopted to support both OOK-1 and OOK-4 for the LP-WUS OOK waveform generation. The key difference between OOK-1 and OOK-4 is coming from the number of OOK symbol that can be transmitted within 1 OFDM symbol. OOK-4 can transmit M OOK pulses within 1 OFDM symbol because it is required to performs pre-processing (e.g., Discrete Fourier Transform, Least Square) in time domain before subcarrier mapping. In a case that M value is 1, there is no difference between OOK-1 and OOK-4 in the receiver side. Hence OOK-4 provides scalability and flexibility on data rate and network overhead, and OOK-1 is relatively simpler. Consequently, new UE receiver function for LP-WUS should be supported in order to support LP-WUS feature. New demodulation requirements should be introduced for LP-WUS.

Agreement Support both OOK-1 and OOK-4 for LP-WUS. - FFS how OOK-1 and OOK-4 are specified - For OOK-4, M<=4, FFS supported values - The SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol - FFS different SCS.

Proposal 1: Introduce UE demodulation requirements for both OOK-1 and OOK-4 LP-WUS.

Frequency Range

For the frequency range of the test scope for Rel-19 LP-WUS, below WF was endorsed in [4] in order to support FR2. Thus, both FR1 and FR2 should be covered.

It is proposed to endorse the following in RAN#105 for Rel-19 LP-WUS/WUR WI regarding FR2 support - Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

Proposal 2: Introduce UE demodulation requirements for both FR1 and FR2 for LP-WUS.

Duplex mode

For the duplex mode, RAN4#111 meeting confirmed that no specific limitation on the bands as below. Therefore, LP-WUS should be supported for both FDD and TDD.

Issue 1-1-1: Operation bands for LP-WUS feature Agreements: RAN4 confirm LP-WUS is a general feature not limited to specific example band(s)

Proposal 3: Introduce UE demodulation requirements for both FDD and TDD for LP-WUS.

Channel Bandwidth and SCS

For the demodulation requirements in RAN4, 10MHz CHBW with SCS 15KHz for FR1 FDD, 40MHz CHBW with SCS 30KHz for FR1 TDD and 100MHz CHBW with SCS 120KHz for FR2 are usually used by majority test cases.

Proposal 4: For the channel bandwidth and SCS, take below assumptions for the starting points

- 10MHz CHBW with SCS 15KHz for FR1 FDD;

- 40MHz CHBW with SCS 30KHz for FR1 TDD;

- 100MHz CHBW with SCS 120KHz for FR2

Number of LP-WUS PRBs

For the number of LP-WUS PRBs, the agreement in RAN1#117 meeting for FR1 TDD as following is 11 PRBs for LP-WUS and LP-SS.

Agreement Update agreement in last meeting as below: From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X to be down-selected between = 11 and 12 PRBs - FFS the number of PRBs for 15kHz - FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz FFS: Whether the above is applicable to FR2

In addition, the same value that 11 PRBs for LP-WUS and LP-SS achieved in RAN1#118bis for FR1 FDD and FR2 as below. Therefore, the number of LP-WUS PRBs as 11 should be used for FR1 FDD, FR1 TDD and FR2.

Agreement From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 15kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5 MHz Agreement From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 60kHz and 120kHz for FR2.

Proposal 5: For the number of LP-WUS PRBs, the value 11 should be used for FR1 FDD, FR1 TDD and FR2.

Performance metric

The receiver characteristic of LP-WUS could be defined by the probability of miss-detection of LP-WUS and the probability of false alarm, i.e., miss-detection rate (MDR) and false-alarm rate (FAR). The SNR working points corresponding to separate thresholds of MDR and FAR could be used as the performance metric for LP-WUS. About the separate thresholds of MDR and FAR, we propose to use 1% as the starting points.

Proposal 6: For the performance metric, SNR working points corresponding to separate thresholds of MDR and FAR could be used as the performance metric for LP-WUS. Prefer to use  1% as the starting points for separate thresholds of MDR and FAR.

Time/frequency error

During SI, some cases were evaluated under the assumption that LP-WUR can achieve the perfect synchronization based on LP-SS. However, from practical perspective, the potential residual error after the synchronization based on designed LP-SS should be considered.

About the Time/frequency error, below agreements agreed in RAN1#118 meeting.

Agreement To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1 - T=1us(optional), 2us for OOK-4 with M=2 - T=0.5us(optional), 1us for OOK-4 with M=4 - Other values of SNR, T, M are up to company report. - Additionally, companies can submit results on SINR with details on how the interference was modelled. - Assume one-shot estimation - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified - RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report. - As a starting point, the time error is based on 5-10ppm residual frequency error. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified - The frequency error: up to company report - Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS. - Channel: AWGN and TDL-C 300ns is assumed. - Consider cross-correlation for 4 binary sequences under time and frequency error - Companies are encouraged to provide details on other additional simulation assumptions, high level description of their receiver algorithm, and assessment on power consumption Agreement For OOK-based LP-WUR, the LP-WUS design assumes the following: - As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift. - The frequency error: up to company report. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified Agreement As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift. - Note: Companies can use any value within the above range with justification on the value (including impact on power consumption)

In addition, below agreements agreed on RAN4 RF session in RAN4#112 meeting.

Issue 2-1-3: Time/frequency error Agreement: Residual frequency error: OFDM based receiver [5] ppm OOK based receiver [ 10 20] ppm

Furthermore, below agreements agreed on RAN4 RRM session in RAN4#112bis meeting.

Issue 2-1-3: Time/frequency error Agreement: Table 2: Cell-specific parameters Parameter; Unit; Cell 1; Cell 2. Frequency offset relative to UE frequency reference; Hz; OFDM based receiver [5] ppm OOK based receiver [5 10] ppm; N/A. Timing error; Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: T= 5us for OOK-1 T= 2us for OOK-4 with M=2 T= 1us for OOK-4 with M=4.

Taking these background details into account, we suggest considering the timing/frequency errors. The proposed timing error as T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4, and proposed frequency errors as OFDM based receiver [5] ppm, OOK based receiver [10] ppm.

Proposal 7: Suggest considering the timing/frequency errors as the test assumptions for LP-WUS performance requirements. The proposed timing error as T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4, and proposed frequency errors as OFDM based receiver [5] ppm, OOK based receiver [10] ppm.

Propagation condition

Although the transmit signal is different with legacy, the propagation condition is the same as legacy. Hence, we prefer to use the popular propagation conditions in demodulation requirements.

Proposal 8: For propagation condition assumption of LP-WUS test requirement, use TDLA30-10 and TDLC300-100 as the starting points.

- Conclusion

In this contribution, we provide analysis and views on the general test scope for demodulation requirements of Rel-19 LP-WUS. The proposals could be summarized as:

Proposal 1: Introduce UE demodulation requirements for both OOK-1 and OOK-4 LP-WUS.

Proposal 2: Introduce UE demodulation requirements for both FR1 and FR2 for LP-WUS.

Proposal 3: Introduce UE demodulation requirements for both FDD and TDD for LP-WUS.

Proposal 4: For the channel bandwidth and SCS, take below assumptions for the starting points

- 10MHz CHBW with SCS 15KHz for FR1 FDD;

- 40MHz CHBW with SCS 30KHz for FR1 TDD;

- 100MHz CHBW with SCS 120KHz for FR2

Proposal 5: For the number of LP-WUS PRBs, the value 11 should be used for FR1 FDD, FR1 TDD and FR2.

Proposal 6: For the performance metric, SNR working points corresponding to separate thresholds of MDR and FAR could be used as the performance metric for LP-WUS. Prefer to use  1% as the starting points for separate thresholds of MDR and FAR.

Proposal 7: Suggest considering the timing/frequency errors as the test assumptions for LP-WUS performance requirements. The proposed timing error as T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4, and proposed frequency errors as OFDM based receiver [5] ppm, OOK based receiver [10] ppm.

Proposal 8: For propagation condition assumption of LP-WUS test requirement, use TDLA30-10 and TDLC300-100 as the starting points.

- Reference

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC (moderator, RAN VC), 3GPP TSG RAN Meeting #102, Edinburgh, GB, December 11-15, 2023.

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024

- TR38.869, Study on low-power wake up signal and receiver for NR, Release 18.

- RP-242361, Way Forward on LP-WUS for FR2, vivo, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024