3GPP TSG-RAN WG4 Meeting #115	R4-2506705

# Malta, MT, 19th – 23rd May, 2025

Agenda item:  7.27.3.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES OD-SSB

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN4 discuss the RRM impacts of OD-SSB with issues and agreement captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

## Scenarios

In previous RAN1 meeting, RAN1 discuss the timing relation and sent a LS to RAN4 [2] as follows:

Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted

Besides, RAN1 also achieve following agreements which can help further clarify the applicable scenario in RAN4 discussion.

Agreement - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists Conclusion The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19 - Scenario #3A and Case #1 - Scenario #3A and Case #2 Above does not impact discussion on SSB periodicity adaptation in time domain Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

In last RAN4 meeting, RAN4 discussed above cases, with following progress:

Issue 1-2-2: Requirement for RAN1 Alt Time-C1 Agreement: - RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario. - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification. - FFS for other procedures. Issue 1-2-3: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: Define the requirement based on OD-SSB periodicity. - Option 2: Not to define requirement. Issue 1-2-4: Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies) Agreement RAN4 to first discuss the RAN4 impact based on RAN2 LS R2-2503015. - Option 1: RAN4 to define requirement for different frequencies - Option 2: RAN4 not to define requirements for different frequencies

For Alt Time C2, based on discussion in last meeting, companies mentioned that the uneven distribution of the SSB may cause additional complexity of UE implementations. Thus, as baseline, we can define requirements based on OD-SSB periodicity, which does not require UE to combine the OD-SSB and AO-SSB measurement.

Proposal 1: For Alt Time C2, RAN4 to define baseline requirements based on OD-SSB.

For the case when OD-SSB and AO-SSB are within different frequencies, there is no much discussion in RAN4 since more concrete conclusion in RAN1/2 are needed. Based on the discussion in last RAN1/2 meeting, the progress are summarized as follows:

RAN1#120bis Agreement For the case when the center frequency locations of always-on SSB and on-demand SSB are different, at least the following is supported for QCL between AO-SSB and OD-SSB - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications RAN2#129bis R2-2503019 In RAN2#129b, RAN2 discussed how to configure L3 measurement when always-on SSB (A-SSB) and on-demand SSB (OD-SSB) have different center frequency, and made the following Working Assumption: Working assumption: - When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. Note: The WA was made from RRC signaling perspective. RAN2 relies on RAN4/1 to decide whether to support the scenario when A-SSB and OD-SSB have different center frequencies or not.

Based on above conclusions, the scenarios are much clearer for RAN4 requirements discussion. For L3 measurement (before and after SCell activation), based on RAN2 WS, our understanding is that UE will be configured with additional servingCell MO on OD-SSB, and upon OD-SSB is activated, UE is no longer required to measure the AO-SSB but to measure OD-SSB. The reasons are as follows:

- Based on previous RAN1 agreements, UE is not required to both AO-SSB and OD-SSB. Since OD-SSB is transmitted, UE shall measurement OD-SSB.

- Based on current RAN2 framework, when event is triggered, UE shall report results for all serving Cell. Currently, there is no way to tell whether the results is from AO-SSB or OD-SSB.

Observation 1: Based on RAN2 discussion, UE is only required to measure OD-SSB upon OD SSB is activated.

Observation 2: From L3 measurement perspective, when AO-SSB and OD-SSB are in different frequencies, upon OD-SSB is activated, UE is only required to measure OD-SSB.

Based on observation 2, from L3 requirements perspective, previous agreements for case#1 can be applied which is very much similar.

For SCell activation requirements, it is straightforward to define SCell activation requirements based on OD-SSB. For known conditions, at least for the case when with same QCL assumption, the known conditions can be based on either AO-SSB or OD-SSB.

Observation 3: For SCell activation requirements, RAN4 to define SCell activation requirements based on OD-SSB. For known conditions, at least for the case when with same QCL assumption, the known conditions can be based on either AO-SSB or OD-SSB.

Based on analysis above, we think RAN4 shall define requirements for scenario when AO-SSB and OD-SSB have different center frequencies.

Proposal 2: RAN4 to define requirements for scenario when AO-SSB and OD-SSB have different center frequencies.

Proposal 3: For the scenario when AO-SSB and OD-SSB have different center frequencies:

- For Scenario 2: Upon OD-SSB is activated, UE is only required to measure OD-SSB and is not required to measure AO-SSB.

- For Scenario 2A: Define SCell activation requirements based on OD-SSB. For known conditions, at least for the case when with same QCL assumption, the known conditions can be based on either AO-SSB or OD-SSB.

## OD-SSB processing time.

In last RAN4 meeting, RAN4 achieve following agreements on OD-SSB processing time.

Agreement UE applies the additional processing timing only in deactivated SCell measurement. - Note: The additional UE processing timing is not applied to SCell activation procedure. The additional processing timing is T_processing, the value of T_processing: - Introduce different values with UE capability: [2 or 3]ms and 5ms.

In last RAN1 meeting, RAN1 also send an LS [4] to RAN1 about the OD-SSB deactivation timeline.

RAN1 has made the following agreement for MAC-CE based deactivation of on-demand SSB (OD-SSB) transmission on SCell. Agreement - Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification. - (Working assumption) T=T_min - RAN4 to confirm the above two working assumptions - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication RAN1 respectfully asks RAN4 if the two working assumptions can be confirmed to agreement.

From our understanding, above working assumptions can be confirmed.

Proposal 4: RAN4 to confirm RAN1 Working assumptions on OD-SSB deactivation.

## Deactivated SCell L3 measurement (case 1)

For deactivated SCell, we start from case#1, and some issues are common for both case#1 and case#2. RAN4 achieved following agreements in last RAN4 meeting:

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure - Background - The deactivated SCell measurement requirement in FR1 is as follow. DRX cycle; T SSB_measurement_period_intra. No DRX; Ceil(5 x Kp) x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra. - The OD-SSB based deactivated SCell measurement procedure is shown as follow. Sub-issue 1: The start point of T2 Wayforward The start point of T2 is - Option 1: - The time point of OD-SSB deactivation - Option 2: - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify. - Option 2a: - The time point of OD-SSB activation Sub-issue 2: The minimum interval(T2) between multiple FMWs Wayforward - Option 1: 5* measCycleSCell - Option 1a: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) - Option 1b: 5*max (measCycleSCell, DRX cycles) - Option 2: Minimum measurement period defined in existing spec. - Option 3: The time period of known Scell condition but with OD-SSB periodicity for FR1 Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2 Wayforward - Option 1: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: The UE is assumed to perform fast mode measurement without cell identification period. - Option 3: No requirement for UE within T2.

For measurement period in fast mode, it is agreed that UE shall perform measurement based on OD-SSB periodicity regardless of measCycleSCell and DRX cycle. The initial motivation to considering T2 is to avoid too frequency fast measurement to avoid severe power consumption issues, e.g. UE is performing deactivated SCell measurement based on each SSB.

Observation 4: The motivation of define a side conditions of T2 is to avoid too frequency fast measurement to avoid severe power consumption issues.

Thus, the very simply principles as discussed in previous meetings by several UE vendors is that UE is not required to perform “fast measurement” more frequently than the minimum interval. And the length can refer to the minimum measurement period defined in existing spec (measurement period with 160ms measCycle).

Regarding the starting point and ending point of T2, the current option 1/2/3 are illustrated in following example.

Fig.1 T2 for OD-SSB activation in different option 1.

As commented during last meeting, for option 1, it could be observed that the last FMW could be very long time ago, thus the OD-SSB is activated very soon again, there is no UE power consumption concerns as the FMW is not frequent at all. For option 2, the problem is that Tend is the time point defined in the spec which may not exist in real UE implementation. Thus, we think option 3 is clearer and can serve the purpose that UE is not required to perform too frequency fast measurement.

Observation 5: The drawbacks of different definition of starting point of T2:

Option 1: The time point of OD-SSB deactivation

Drawbacks: UE will not start fast measurement even the last FMW is very long time ago.

Option 2: The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

Drawbacks: Tend is the time point defined in the spec which may not exist in real UE implementation.

Proposal 5: The starting point of T2 is define as the time point of OD-SSB activation (or first OD-SSB).

Proposal 6: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement when the interval is smaller than T2. And the length is the minimum measurement period defined in existing spec (i.e. measurement period with 160ms measCycle).

Another aspect is when the Cell identification could be saved. As commented before, it as separate dsicussion from the T2 dsicussion. Regardign the T2 length, many companies mentiond that it could be 5*measCycle Scell, thus it could be 800 ms for FR1. However, UE can mainly the time for Cell search much longer than that. Instead, the value for known conditions can be referred to.

Proposal 7: When the time interval is smaller than X, UE is assumed to perform fast mode measurement without cell identification period, where X is

      - max(5*measCycleSCell,  5*DRX cycles) for FR1

      - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

## Deactivated SCell L3 measurement – other issues

For neighbor cell measurement, RAN4 achieved following agreements in last meeting:

Issue 1-4-8: OD-SSB based Neighbor cell measurement before OD-SSB activation Wayforward - RAN4 to discuss whether to perform measurement on neighbour cells’ in the same frequency of deactivated SCell before OD-SSB activation for Case#1.

For neighbor cell measurement, we identified another issue based on the agreements in last meeting. For some use cases, gNB may triggered OD-SSB to check the status of the SCell or for SCell activation. On the same frequency layers, there may be no other neighbour cells or neighbour cells are also OD-SSB SCell and the SSB is muted. We following RAN4 agreements achieved in previous meeting, we would like to check which is the right understanding:

Understanding 1: For case#1, UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered, but to perform measurement on neighbour cell as legacy.

Understanding 2: For case#1, UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered for both serving Cell and neighbour cell.

@RAN4#112 R4-2413885 Issue 2-2-2: Case 1 - OD-SSB based deactivated SCell measurement before OD-SSB indication - Agreement: - In (Case #1 Scenario #2) and (Case #1, Scenario #2A), UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered. - Note: How to differentiate OD-SSB SCell and normal SCell depends on other WGs’ discussion.

Proposal 8: RAN4 to discuss which is the right understanding for case 2:

- Understanding 1: For case#1, UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered, but to perform measurement on neighbour cell as legacy.

- Understanding 2: For case#1, UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered for both serving Cell and neighbour cell.

Another issue is whether to introduce OD-SSB based L3 measurement reporting. UE can trigger the measurement report if there is valid measurement report and UE can stop fast measurement. The common understanding is gNB is expecting a timely report upon activating OD-SSB with fast measurement window. By legacy case, what we can imagine is gNB to artificially configured a very low threshold to trigger the event and reconfigure it after report to avoid UL overhead and wrongly triggered event. The is very clumsy and situation will be more severe for case#2. We think we can have similar approach as introduced in Rel-18 RRM “report on SCell activation”, and the workload is manageable in RAN2 since framework can be reused.

Issue 7-3: OD-SSB based L3 reporting Wayforward: - RAN4 to discuss whether to introduce an OD-SSB based L3 measurement reporting.

Observation 6: There is no existing method to trigger L3 report after OD-SSB FMW to report latest report and help gNB to deactivated OD-SSB.

Proposal 9: RAN4 to send LS to RAN2 to support L3 report triggered by OD-SSB activation that UE triggers report after measurement based on OD-SSB FMW.

Another issue is about whether to define requirements for multiple OD-SSB carriers. From our understanding, if the feature is supported, it unreasonable to restrict that only one carrier can support OD-SSB and all other carriers are with legacy SSB. If network want to have energy saving gain, most likely all carries within the same bands will support OD-SSB; otherwise, the Rx chain is active the energy saving is negligible if only one of the carriers is supporting OD-SSB.

Observation 7: The energy saving gain will be severely impacted if it is restricted that only one carrier can use OD-SSB.

And if we look at pervious RAN2 agreements, it is already supported by RAN2 that NW can send OD-SSB indication for multiple SCells simultaneously.

RAN2 Agreements @ RAN2#127bis NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Thus, multiple OD-SSB carriers shall be supported with/without legacy SSB carriers, and whether to prioritize OD-SSB carrier(s) measurement shall be discussed separately with a unified an approach.

Observation 8: Multiple OD-SSB carriers are already supported by other WG.

Proposal 10: RAN4 to define requirements for multiple OD-SSB carriers with/without legacy SSB carriers.

Regarding the prioritization discussion, RAN4 achieve following agreements in last meeting:

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW Sub-issue 2: CSSF Agreement - When NW configures single OD-SSB carrier with multiple legacy SSB carriers, CSSF =1 for OD-SSB layer - Extend the FMW window time by one OD-SSB periodicity - Longer measurement delay of other frequency layers with AO-SSBs are expected

Based on above agreements, when considering multiple OD-SSB carriers with/without legacy SSB carriers, when fast measurement window for each OD-SSB carriers are non-overlapped (i.e. OD-SSB are activated in turns). The above agreements can apply to each OD-SSB carriers. Another scenario is whether to consider the case that OD-SSB are activated together in multiple layers. Companies commented that there is no NW energy saving gain, as UE anyway needs to measurement these carriers in turns. However, one benefits is that there is no need to precisely schedule the OD-SSB in different carriers to guarantee that there are non-overlapped. For this case, if CSSF is considered, there is no further burden on UE implementations that UE can takes turns to perform fast measurement.

Proposal 11: For deactivated SCell measurement when UE is configured with multiple OD-SSB carriers with/without legacy SSB carriers:

When fast measurement window for each OD-SSB carriers are non-overlapped (i.e. OD-SSB are activated in turns):

- RAN4#114bis agreements applies to each OD-SSB carriers that:

- CSSF =1 for each OD-SSB layer

  - Extend the FMW window time by one OD-SSB periodicity

  - Longer measurement delay of other frequency layers with AO-SSBs and OD-SSB after FMW are expected

When fast measurement window for each OD-SSB carriers are overlapped (i.e. OD-SSB are activated simultaneously):

- CSSF are calculated based on these OD-SSB carriers.

- Longer measurement delay of other frequency layers with AO-SSBs and OD-SSB after FMW are expected

## Scenario 3B

Regarding scenario 3B, RAN1 achieved following agreements in last meeting.

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

It could be observed that for scenario 3B, the MAC-CE could be used for parameters updating. The parameters could be updated are as follows per RAN1 agreements so far:

- SSB periodicity of OD-SSB

- SSB positions within OD-SSB (for case1 and case 2 same frequency)

- N of od-SSB burst to be transmitted after OD-SSB is indicated

Observation 9: Following parameters could be updated per RAN1 agreements for far for Scenario 3B:

- SSB periodicity of OD-SSB

- SSB positions within OD-SSB (for case1 and case 2 same frequency)

- N of od-SSB burst to be transmitted after OD-SSB is indicated

Thus, RAN4 needs to evaluate the requirements impacts for above Scenario 3B.

For L1 measurement, same principle can be reused from SSB adaption discussion, which is shown as follows:

Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details Agreement: - For L1-RSRP/L1-SINR/CBD - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB. - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation. Issue 1-2-1: L1 requirements at transition Agreement - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline. - No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs.

Based on above agreements, it means UE will use the new SSB periodicity for L1 measurement after the first SSB, where the processing time needs to be considered (e.g. 3ms + UE capabilities) as agreed in last meeting. Thus, for OD-SSB parameters updating in terms of periodicity and SSB positions, the measurement/evaluation period based on SSB parameters (i.e. periodicity and/or positions) apply from Tfirst_SSB, where Tfirst_SSB the uncertainty time from the timepoint when MAC-CE for updating the transmission parameter is received till the first SSB that UE is able to process based on timeline defined in RAN1 and the addition processing time defined in RAN4.

Proposal 12: For Scenario 3B for OD-SSB parameter updating for L1 measurement, apply same principles as SSB adaption that:

- The measurement/evaluation period based on updated SSB parameters (i.e. periodicity and/or positions) apply from Tfirst_SSB.

- Tfirst_SSB the uncertainty time from the timepoint when MAC-CE for updating the transmission parameter is received till the first SSB that UE is able to process based on timeline defined in RAN1 and the addition processing time defined in RAN4.

For L1 measurement impact in Scenario, there is on common issue for OD-SSB and SSB adaptation which is also analysed in our paper for SSB adaption in last meeting [5]. For L1 measurement restriction requirements, when SSB-based L1-RSRP and CSI-RS are overlapped, UE is only required to measure one of them. Based on existing requirements, it is up to UE to choose either SSB or CSI-RS for measurement. However, OD-SSB parameters updating in Scenario 3, SSB are occasionally triggered by DCI. Since the Cell specific CSI-RS is configured based on normal SSB pattern without collision, the newly triggered the SSB (different periodicity or different position) may colliding the CSI-RS and gNB may not guarantee that both SSB and CSI-RS are available to UE when they are overlapping. Thus, if UE choose CSI-RS for measurement, it may cause wrong measurement results (e.g. wrong L1-RSRP results/ wrongly triggered BFD). Thus, it should be discussed whether legacy measurement restriction can apply when adapted SSB colliding with CSI-RS.

Observation 10: Based on legacy measurement restriction when CSI-RS and SSB collides, UE can choose either of them for measurement, which implicitly assumed that both CSI-RS and SSB are available.

Observation 11: When OD-SSB parameters updating in Scenario 3, the dynamic change of SSB periodicity and positions may collides with CSI-RS and the CSI-RS may be dropped.

Proposal 13: For OD-SSB parameters updating in Scenario 3, RAN4 discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB.

For Scenario 3B, the impacts on L3 measurements shall be considered. As mentioned above, gNB could update the parameters of OD-SSB in Scenario 3B. One question is whether the same requirements/behaviour defined for scenario 2 can be reuse. From our understanding, there is no much benefits to apply the FMW after the SCell is activated. The reason are as follows:

- In deactivated SCell measurement, UE performs measurement in MeasCycleSCell, and the FMW can help gNB to get fresh measurement results to speed up the SCell activation.

- After the SCell is activated, UE performs measurement based on SMTC which is already very frequent compared with MeasCycleSCell. What’s more, data scheduling in the SCell shall be considered and FMW may cause unnecessary scheduling restrictions.

Proposal 14:

For Scenario 3B for OD-SSB parameter updating for L3 measurement:

- For serving Cell measurement: UE perform serving Cell measurement based the Max (SMTC, OD-SSB periodicity)

- For neighbour cell measurement: Legacy requirements apply

## Others

For OD-SSB based SCell activation requirements, it is FFS whether to define OD-SSB based Rel-18 SCell activation with the L3 reporting requirement.

Issue 2-1-2: Fast SCell Activation Agreement - RAN4 to only update the known cell condition considering OD-SSB for Rel-17 fast SCell activation. - The requirement is based on A-TRS of Rel-17 fast SCell activation requirement. - The details can be discussed in the CR directly. - It doesn’t impact the WI’s completion. Issue 2-1-3: Deactivated PUCCH SCell activation Agreement - RAN4 to define OD-SSB based deactivated PUCCH SCell activation requirement. - The details can be discussed in the CR directly. - It doesn’t impact the WI’s completion. Issue 2-1-4: Rel-18 SCell Activation with the L3 reporting during activation Agreement - FFS RAN4 to define OD-SSB based Rel-18 SCell activation with the L3 reporting requirement. - The details can be discussed in the CR directly. - It doesn’t impact the WI’s completion.

From our understanding, there is clear benefits for the scenario. In scenario 2, gNB may trigger OD-SSB measurement time to time, and UE has historical measurement results for the deactivated SCell. When gNB activate the SCell, UE can report the valid the report to gNB and Rel-18 requirements can apply.

Proposal 15: RAN4 to define OD-SSB based Rel-18 SCell activation with the L3 reporting requirement considering at least following case:

- SCell activation with the L3 reporting based on deactivated SCell measurement based on OD-SSB (Scenario 2).

Another issue is about the relation to SSB-less operation which has been mentioned from the beginning, but not clarified so far. The latest status is summarized as follows [6]:

Issue 1-4-12: SSB-less SCell and OD-SSB SCell differentiation - Proposals - Option 1: Ericsson - UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation. Otherwise, UE shall follow the SSB-less SCell activation procedure. - Option 2: Intel - When the UE supports fast SCell activation for intra-band contiguous CA or inter-band CA and indicating support of scellWithoutSSB or scellWithoutSSB-InterBandCA-r18 and the conditions meet fast SCell activation conditions, the Rel-19 requirements do not apply.

To avoid confusions for the case when UE supports both SSB-less and OD-SSB, which requirements applies shall be clarified. UE shall follow the OD-SSB based SCell activation requirements once NW indicates the OD-SSB transmission together with SCell activation (i.e. scenario 2A). Otherwise, UE shall follow the SSB-less SCell activation requirements.

Proposal 16: RAN4 to clarify which requirements apply when UE supports both SSB-less and OD-SSB as:

- UE shall follow the OD-SSB based SCell activation requirements once NW indicates the OD-SSB transmission during with SCell activation. (i.e. scenario 2A). Otherwise, UE shall follow the SSB-less SCell activation requirements.

Regarding the feature list, the feature list table in Annex is proposed for the agreed two UE capabilities.

Proposal 17: Agree on the feature list table attached.

# Conclusions

Proposal 1: For Alt Time C2, RAN4 to define baseline requirements based on OD-SSB.

Observation 1: Based on RAN2 discussion, UE is only required to measure OD-SSB upon OD SSB is activated.

Observation 2: From L3 measurement perspective, when AO-SSB and OD-SSB are in different frequencies, upon OD-SSB is activated, UE is only required to measure OD-SSB.

Observation 3: For SCell activation requirements, RAN4 to define SCell activation requirements based on OD-SSB. For known conditions, at least for the case when with same QCL assumption, the known conditions can be based on either AO-SSB or OD-SSB.

Proposal 2: RAN4 to define requirements for scenario when AO-SSB and OD-SSB have different center frequencies.

Proposal 3: For the scenario when AO-SSB and OD-SSB have different center frequencies:

- For Scenario 2: Upon OD-SSB is activated, UE is only required to measure OD-SSB and is not required to measure AO-SSB.

- For Scenario 2A: Define SCell activation requirements based on OD-SSB. For known conditions, at least for the case when with same QCL assumption, the known conditions can be based on either AO-SSB or OD-SSB.

Proposal 4: RAN4 to confirm RAN1 Working assumptions on OD-SSB deactivation.

Observation 4: The motivation of define a side conditions of T2 is to avoid too frequency fast measurement to avoid severe power consumption issues.

Observation 5: The drawbacks of different definition of starting point of T2:

Option 1: The time point of OD-SSB deactivation

Drawbacks: UE will not start fast measurement even the last FMW is very long time ago.

Option 2: The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

Drawbacks: Tend is the time point defined in the spec which may not exist in real UE implementation.

Proposal 5: The starting point of T2 is define as the time point of OD-SSB activation (or first OD-SSB).

Proposal 6: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement when the interval is smaller than T2. And the length is the minimum measurement period defined in existing spec (i.e. measurement period with 160ms measCycle).

Proposal 7: When the time interval is smaller than X, UE is assumed to perform fast mode measurement without cell identification period, where X is

      - max(5*measCycleSCell,  5*DRX cycles) for FR1

      - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

Proposal 8: RAN4 to discuss which is the right understanding for case 2:

- Understanding 1: For case#1, UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered, but to perform measurement on neighbour cell as legacy.

- Understanding 2: For case#1, UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered for both serving Cell and neighbour cell.

Observation 6: There is no existing method to trigger L3 report after OD-SSB FMW to report latest report and help gNB to deactivated OD-SSB.

Proposal 9: RAN4 to send LS to RAN2 to support L3 report triggered by OD-SSB activation that UE triggers report after measurement based on OD-SSB FMW.

Observation 7: The energy saving gain will be severely impacted if it is restricted that only one carrier can use OD-SSB.

Observation 8: Multiple OD-SSB carriers are already supported by other WG.

Proposal 10: RAN4 to define requirements for multiple OD-SSB carriers with/without legacy SSB carriers.

Proposal 11: For deactivated SCell measurement when UE is configured with multiple OD-SSB carriers with/without legacy SSB carriers:

When fast measurement window for each OD-SSB carriers are non-overlapped (i.e. OD-SSB are activated in turns):

- RAN4#114bis agreements applies to each OD-SSB carriers that:

- CSSF =1 for each OD-SSB layer

  - Extend the FMW window time by one OD-SSB periodicity

  - Longer measurement delay of other frequency layers with AO-SSBs and OD-SSB after FMW are expected

When fast measurement window for each OD-SSB carriers are overlapped (i.e. OD-SSB are activated simultaneously):

- CSSF are calculated based on these OD-SSB carriers.

- Longer measurement delay of other frequency layers with AO-SSBs and OD-SSB after FMW are expected

Observation 9: Following parameters could be updated per RAN1 agreements for far for Scenario 3B:

- SSB periodicity of OD-SSB

- SSB positions within OD-SSB (for case1 and case 2 same frequency)

- N of od-SSB burst to be transmitted after OD-SSB is indicated

Proposal 12: For Scenario 3B for OD-SSB parameter updating for L1 measurement, apply same principles as SSB adaption that:

- The measurement/evaluation period based on updated SSB parameters (i.e. periodicity and/or positions) apply from Tfirst_SSB.

- Tfirst_SSB the uncertainty time from the timepoint when MAC-CE for updating the transmission parameter is received till the first SSB that UE is able to process based on timeline defined in RAN1 and the addition processing time defined in RAN4.

Observation 10: Based on legacy measurement restriction when CSI-RS and SSB collides, UE can choose either of them for measurement, which implicitly assumed that both CSI-RS and SSB are available.

Observation 11: When OD-SSB parameters updating in Scenario 3, the dynamic change of SSB periodicity and positions may collides with CSI-RS and the CSI-RS may be dropped.

Proposal 13: For OD-SSB parameters updating in Scenario 3B, RAN4 discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB.

Proposal 14:

For Scenario 3B for OD-SSB parameter updating for L3 measurement:

- For serving Cell measurement: UE perform serving Cell measurement based the Max (SMTC, OD-SSB periodicity)

- For neighbour cell measurement: Legacy requirements apply

Proposal 15: RAN4 to define OD-SSB based Rel-18 SCell activation with the L3 reporting requirement considering at least following case:

- SCell activation with the L3 reporting based on deactivated SCell measurement based on OD-SSB (Scenario 2).

Proposal 16: RAN4 to clarify which requirements apply when UE supports both SSB-less and OD-SSB as:

- UE shall follow the OD-SSB based SCell activation requirements once NW indicates the OD-SSB transmission during with SCell activation. Otherwise, UE shall follow the SSB-less SCell activation requirements.

Proposal 17: Agree on the feature list table attached.

# References

[1] R4-2504906 WF on RRM requirements for Netw_Energy_NR_enh_Part1

[2] R1-2501633 LS on time location of on-demand SSB for SCell

[3] R2-2503019 LS on on-demand SSB

[4] R1-2503108 LS to RAN4 on time instance B for on-demand SSB SCell operation

[5] R4-2503867 Discussion on RRM impact for Rel-19 NES on adaptation

[6] R4-2503625 Topic summary for [114bis][215] Netw_Energy_NR_enh_Part1

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. Netw_Energy_NR_enh | 54-1 | Lower bound on OD-SSB periodicity for measurement in fast measurement window | A lower bound on OD-SSB periodicity of [10] ms for measurement in deactivated SCell fast measurement window | At least one among 61-1/ 61-2/61-2a/ 61-3/ 61-4/61-4a | Yes | n/a | There is no lower bound on OD-SSB periodicity for for measurement in deactivated SCell fast measurement window | Per BC | n/a | n/a | Yes | If UE does not indicate this capability, there is no lower bound on OD-SSB periodicity for for measurement in deactivated SCell fast measurement window. | Optional with capability signalling |
|  | 54-2 | Additional processing timing for OD-SSB activation | Additional OD-SSB processing time for OD-SSB reception. | At least one among 61-3/ 61-4/61-4a | Yes | n/a | UE is not able to receive OD-SSB before the processing time | Per UE | n/a | n/a | Yes | Candidate value :{[2 or 3]ms, 5ms}. The additional processing time does not apply when OD-SSB activation comes together with SCell activation. | Optional with capability signalling |