3GPP TSG-RAN WG4 Meeting # 115	R4-2506706

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on OD-SSB based SCell Activation Delay Requirement for Deactivated PUCCH SCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-Core |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce requirements for OD-SSB based PUCCH SCell activation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce requirements for OD-SSB based PUCCH SCell activation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.X1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

### 8.3.X1	SCell Activation Delay Requirement for Deactivated PUCCH SCell based on OD-SSB

The requirements in this clause shall apply for the UE configured with one downlink SCell and when PUCCH is configured for the SCell being activated.

For a UE supporting [OD-SSB operation], if UE receives SSB activation command together with on-demand SSB activation command, UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated on the PUCCH SCell as required by clause 8.3.12, expect that:

-	For Tactivation_time, TFirstSSB, TFirstSSB_MAX, TSMTC_MAX, Trs and TL1-RSRP, measure are replaced as:

-	TFirstSSB: when there is no always-on SSB, TFirstSSB is the time to the end of the first complete SSB burst indicated by the on-demand periodicity of the SCell being activated, after slot n + THARQ+3msNR slot length; when there is always-on SSB and the certer frequency of always-on SSB and on-demand SSB are the same, TFirstSSB is the time to the end of the first complete SSB burst indicated by the periodicity of the union of AO-SSB and OD-SSB of the SCell being activated, after slot n + THARQ+3msNR slot length.

-	TFirstSSB_MAX: when there is no always-on SSB, TFirstSSB_MAX is the time to the end of the first complete SSB burst indicated by the on-demand periodicity of the SCell being activated, after slot n + THARQ+3msNR slot length; when there is always-on SSB and the certer frequency of always-on SSB and on-demand SSB are the same, TFirstSSB_MAX is the time to the end of the first complete SSB burst indicated by the periodicity of the union of AO-SSB and OD-SSB of the SCell being activated, after slot n + THARQ+3msNR slot length.

-	TSMTC_MAX: when there is no always-on SSB, TSMTC_MAX is the on-demand periodicity of the SCell being activated; when there is always-on SSB and the certer frequency of always-on SSB and on-demand SSB are the same, TSMTC_MAX is the periodicity of the union of AO-SSB and OD-SSB of the SCell being activated.

-	Trs: when there is no always-on SSB, Trs is the periodicity of the SCell being activated; when there is always-on SSB and the certer frequency of always-on SSB and on-demand SSB are the same, Trs is the periodicity of the union of AO-SSB and OD-SSB of the SCell being activated.

-	TL1-RSRP, measure: For SSB-based L1-RSRP measurement, when there is no always-on SSB, L1-RSRP measurement delay apply based on the on-demand periodicity of the SCell being activated; when there is always-on SSB and the certer frequency of always-on SSB and on-demand SSB are the same, L1-RSRP measurement delay apply based the periodicity of the union of AO-SSB and OD-SSB of the SCell being activated.

Editor’s notes: FFS the case when there is always-on SSB and the certer frequency of always-on SSB and on-demand SSB different.

### <End of Change 1>