3GPP TSG-RAN WG4 Meeting # 115	R4-2506709

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.27.3.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES on adaptation

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN4 discussed the RRM impacts of OD SIB1 in last meeting with the agreements captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

The agreements for RRM impacts for OD-SIB1 is as follows.

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Way forward: - Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. (HW, OPPO, Apple, Nokia, Xiaomi, CATT, MTK, vivo) - Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. (CMCC, QC, LGE) - Option C: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paging time. (E///, LGE, MTK) Issue 2-1-2: RRM requirements impacts for OD-SIB1 – timing Agreement - Clarify in transmit timing requirements to include UL WUS transmission (PRACH) for OD-SIB1: - reference cell is the NES cell - NTA for UL-WUS is defined as 0 - NTA offset is the [n-TimingAdvanceOffset] from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133 - Note: The IE can be updated based on other WG’s progress.

Based on the latest progress in RAN1/2 discussion, when UE decides to perform cell reselection to an NES Cell, it will first transmit WUS(PRACH) to NES Cell based on the WUS configuration obtained from Cell A, and receives SIB1 transmitted from NES Cell within a configured time window. Thus, for RRM requirements impacts, the paging interruption may need to be updated to include the potential paging interruption caused by WUS transmission. The existing paging interruption requirements are shown as follows:

#### 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell. These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

Based on the discussion in last meeting, one of the options is to define the UE behaviour that UE postpone the UL-WUS and prioritize paging. From our understanding, the UE behaviour is very clear. As commented by companies, as UE needs to enter connected mode after paging reception, when Cell reselection conditions are met, it is more reasonable to reselect the qualified NES Cell as soon as possible, and enter connected mode from the NES Cell. It does not make much sense to ask UE to prioritize the paging on Cell A. Thus, a proper way to clarify the impact due to OD-SIB1 is to that: When the target Cell is an [NES OD-SIB1 Cell], TSI-NR also includes the time for UL WUS transmission and RAR reception for OD-SSB1 reception.

Proposal 1: For RRM requirements impacts of OD-SIB1, RAN4 to update the paging interruption requirements as:

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. When the target Cell is an [NES OD-SIB1 Cell], TSI-NR also includes the time for UL WUS transmission and RAR reception for OD-SSB1 reception.

# Conclusions

Proposal 1: For RRM requirements impacts of OD-SIB1, RAN4 to update the paging interruption requirements as:

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. When the target Cell is an [NES OD-SIB1 Cell], TSI-NR also includes the time for UL WUS transmission and RAR reception for OD-SSB1 reception.

# References

[1] R4-2504907 WF on RRM requirements for Netw_Energy_NR_enh_Part2