3GPP TSG-RAN WG4 Meeting # 115	R4-2506712

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.30.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM requirements for Rel-19 XR

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 discussed the RRM impacts of the WI with agreements and issues captured in the WF [1]. In this contribution, we further provide our views on remaining issues.

# Discussion

## 2.1 Indication

In RAN4#113 meeting, there was very controversial discussion on DCI-based indication and RRC-based indication. RAN4 reply the LS in [2]. In the LS reply, RAN4 confirm the DCI-based solution, and at the same time, RAN4 is also discussing the need of semi-static based solution. As commented many times in previous meetings, there is clear benefits of semi-static solution compared with DCI-based solution.

Benefits of semi-static solution:

- It is less complicated to UE implementation that UE can have a stable long-term measurement pattern.

- Semi-static schemes can support the case that MG occasion skipping without impacting RRM performance. If NW can indicate the positions of MG occasions to be skipped, UE can arrange the measurement pattern to guarantee that there is no impact on measurement delay.

- A single RRC signal has less overhead than DCI which is per-occasion indication.

And it can also be observed from the discussion and also captured in chairman’s notes, all UE vendors support that semi-static solution can ease UE implementation complexity. One of the major concerns is that whether the RRC-based solution can support the XR service as it is not that flexible as DCI-based solution. It could be observed from the study phase that benefits come from that some XR frame will collides with MG inevitably due to non-integer frame interval. These patterns are not that frequently changed, and semi-static approach can cancel the MG which will collide with XR frame in following several occasions. And with that information, UE can re-arrange the measurement pattern using the available MG.

Observation 1: Compared with DCI-based solution, semi-static approach has following benefits:

- It is less complicated to UE implementation that UE can have a stable long-term measurement pattern.

- Semi-static schemes can support the case that MG occasion skipping without impacting RRM performance. If NW can indicate the positions of MG occasions to be skipped, UE can arrange the measurement pattern to guarantee that there is no impact on measurement delay.

- A single RRC signal has less overhead than DCI which is per-occasion indication.

However, though the discussion in RAN4 is controversial, we didn’t observe very concrete argument why the semi-static approach is not needed. The arguments are mainly about working procedure wise (e.g. other WG has made decision though we do not agree with the statement). The only technical concern is whether it will lead to UE only supporting semi-status approach if it is allowed.

Unfortunately, we don’t have chance to discussed the issue in last RAN4 meetings, the status are the same as the meeting before last meeting:

## Issue 6-1: Semi-static schemes <Way forward>: - Further discussion based on recommended options - Option 1: Do not consider semi-static solutions - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution. - Option 2a: RRC pattern-based - Option 2b: MAC CE bitmap for skipping indication - Option 2c: RRC configuration patterns with MAC CE activation - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

As we all know, the semi-static solution is not a pure RAN4 discussion. If it is agreed to be supported, at least RAN2 needs to discuss the MAC-CE/RRC impacts. However, if this issue is keeping untouched in RAN4, other working group will have no time for further discussion even it is agreed to be introduced.

Thus, it is proposed that RAN4 to conclude whether to introduce semi-static indication by RAN4#115.

Proposal 1: RAN4 to conclude whether to introduce semi-static indication by RAN4#115.

Proposal 2: RAN4 to consider following approach to move forward:

- The DCI-based indication shall be supported by default if UE support MG skipping.

- RAN4 to introduce semi-static indication to skip measurement gap as additional UE capability.

## 2.2 Type of MG

In last meeting, companies spent most of TU to discuss the applicable type of MG for MG skipping. The progress is summarized as follows:

## Issue 2-1: Types of measurement gaps In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG. <Agreement>: - Gaps which shall not be cancelled - MUSIM - Positioning - For other gap types: - The companies bring the issues and solutions on the RAN4#115. If the solution is not agreeable in RAN4#115, the gap type is not part of Rel-19 - The discussion in RAN4#115 starts with Type 2 MG, then other gap types based on incoming contributions if time allows.

From our understanding, there is no significant technical reason why certain type of gap cannot be cancelled. It will cause extra complexity to gNB and UE implementation. For instance, if UE supports NCSG, then how to understand the applicability rules as companies proposed. Does it mean as long as NCSG is configured, the MG skipping feature can not be enabled?

Observation 2: Artificially precluded certain type of gaps will cause unnecessary implementation complexity to both gNB and UE.

The only technical issue is for concurrent gap. There is dropping rules when colliding happens. Companies mentioned that whether the skipping of a certain MG occasion will change the dropping situation. For instance, MG1 and MG2 are collided and MG2 is dropped due to the existing dropping rules. If MG 1 is skipped by the XR operation, does it make MG2 valid again? From our understanding, the dropping rules when MG collides are rather static and it is determined when MGs are configured. It makes no sense to dynamic change the dropping situation when MG skipping happens. The most reasonable way is to apply the existing dropping rule first and the MG skipping only applies to the MG which is not dropped, and the MG skipping won’t change status of the MG occasion w.r.t dropped or not.

Proposal 3: MG skipping applies to all type of MG except for MUSIM and Position which is precluded in last meeting.

Proposal 4: For concurrent gap, existing dropping rules applies and the MG skipping only applies to the MG which is not dropped, and the MG skipping won’t change status of MG occasion(s) w.r.t dropped or not.

## 2.3 Requirements impacts

Regarding how to capture impact of measurement skipping, following options are considered:

## Issue 3-2: Formulas for measurement delay extension <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - FFS: CSSFinter

It is agreed to following NR-U approach to extend the measurement period requirements. Regarding the detailed formula, option 2 is preferred. For option 1, the extended measurement occasions are not scaled by CSSF, it somehow meanings UE shall prioritize the measurement on certain carriers. However, when a certain MG is skipped, we cannot determine on which carriers the MG is used for measurement, which is up to UE implementation. And for the same reason, the delay shall be extended for all frequency which are candidates to be measured using the case.

Proposal 5: Use following formula to define the delay extension. The all candidate carriers using the MG shall be extended.

- Option 2:

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of skipped MG occasions in the measurement period.

- the delay shall be extended accordingly for all frequencies which are candidates to be measured using the gap.

## 2.4 Applicable range of measurement gap skipping

Regarding the applicable CC for MG cancelling, one important aspect should be considered is the supporting of per-FR gap or per-UE gap. RAN1 achieved following agreements in last meeting,

Conclusion It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction. - Note: Applicable range of measurement gap/restriction is based on RAN4 specifications

Based on our understanding about the highlighted sentence in RAN1 agreement, the applicable range is up to RAN4 for further discussion. From UE implementation perspective, if UE is capable of per-FR gap, and if the per-FR gap is cancelled, UE shall be ready for data reception/transmission in the MG in the FR and has no impacts on UE behavior in the other FR. However, different from legacy RRM requirements which only relies on whether UE supporting per-FR gap, the situation for MG cancellation is a bit different. If gNB is not implementing the per-FR gap, and only per-UE gap is configured to a UE which is capable of per-FR gap, there could be misunderstanding between UE and gNB. UE may keep measurement in the other FR and gNB may think all CCs are ready for data scheduling. Thus, it should be relied on whether per-FR gap is actually configured by gNB instead of whether UE is supporting per-FR gap.

Proposal 6: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

# Conclusions

Observation 1: Compared with DCI-based solution, semi-static approach has following benefits:

- It is less complicated to UE implementation that UE can have a stable long-term measurement pattern.

- Semi-static schemes can support the case that MG occasion skipping without impacting RRM performance. If NW can indicate the positions of MG occasions to be skipped, UE can arrange the measurement pattern to guarantee that there is no impact on measurement delay.

- A single RRC signal has less overhead than DCI which is per-occasion indication.

Proposal 1: RAN4 to conclude whether to introduce semi-static indication by RAN4#115.

Proposal 2: RAN4 to consider following approach to move forward:

- The DCI-based indication shall be supported by default if UE support MG skipping.

- RAN4 to introduce semi-static indication to skip measurement gap as additional UE capability.

Observation 2: Artificially precluded certain type of gaps will cause unnecessary implementation complexity to both gNB and UE.

Proposal 3: MG skipping applies to all type of MG except for MUSIM and Position which is precluded in last meeting.

Proposal 4: For concurrent gap, existing dropping rules applies and the MG skipping only applies to the MG which is not dropped, and the MG skipping won’t change status of MG occasion(s) w.r.t dropped or not.

Proposal 5: Use following formula to define the delay extension. The all candidate carriers using the MG shall be extended.

- Option 2:

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of skipped MG occasions in the measurement period.

- the delay shall be extended accordingly for all frequencies which are candidates to be measured using the gap.

Proposal 6: Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

# References

[1] R4-2502700 WF on RRM requirements for XR_Ph3

[2] R4-2420198 Response LS on gaps/restrictions that are caused by RRM measurements