3GPP TSG-RAN WG4 Meeting # 115	R4-2506715

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.33.6

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impacts for IoT NTN TDD mode

Document for:	Discussion

# Introduction

The new WID of NTN IoT TDD mode was approved in [1] and was revised in [2]. RAN4 studied the impact due to periodic pattern from RAN4#113 meeting. The progress of last RAN4 meetings are captured in [3]. In this paper, we provide our views on the potential impacts on RRM impacts.

# Discussion

The latest progress last RAN4 meeting is summarized as follows:

## Issue 0: General aspects Agreement (online): For TDD NB-IoT in NTN, discuss whether to define the RRM requirements for: - Earth moving and fixed cell, normal coverage with SNR ≥ -1dB Agreement: Further discuss the following questions in the next meeting: - Q1: Is the measurement/evaluation solely based on NPSS, NSSS or NRS? - Q2: How many samples of the reference signal(s) were needed in a normal coverage scenario with SNR ≥ [-6, -1] dB? - Q3: Is the UE required to monitor the reference signals during DL NB-IoT subframes that are in the middle of ongoing UL transmissions? Question Q1; Question Q2; Question Q3. Issue 1: Cell reselection (4.6A.2). Issue 2: Cell measurement in connected mode (8.14A). Issue 3: RRC re-establishment (6.5A). Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A). Issue 5: RLM (7.23A). ## Issue 1: Cell reselection (4.6A.2) Agreement (online): For the cell reselection for TDD NB-IoT in NTN, - The requirements are applicable only if UE is provided with the active periods of the neighbour cells configured for the measurements. - For earth fixed cell and earth moving cell: - The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable if it is confirmed that the current requirement is achievable without requiring more than one NPSS/NSSS sample for half the DRX cycle length [due to the TDD pattern] - Further discuss the values if -1dB will be used. Ksatellite = 1 for the cells measured in DL opportunities of a neighbour satellite that are in non-overlapping with DL opportunities to be measured in the serving satellite

One important remaining issue is to align the understanding on the basic assumptions on number of samples for each RRM procedure, thus RAN4 could update the requirements at higher SNR conditions. Based on discussion in last meeting, there is no intention for the group to conduct the measurement campaign again at -1 dB SNR. Thus, we provide our analysis based on the previous RAN4 discussion.

As we commented in last RAN4 meeting, the challenging case for mobility performance is the cell detection time, especially in IDLE mode. In last meeting, it is already agreed that existing requirements without extension can apply and whether to have enhanced requirement at -1dB. Based on the contributions about the simulation results in previous releases [4][5][6], the simulation results at higher SNR level are summarized in following tables:

Table I. Simulation results at higher SNR [4][5][6]

| R4-164150 | @-3 dB 7 NPSS +1 NSSS |
| --- | --- |
| R4-164089 | @-2 dB 11 NPSS + 2 NSSS |
| R4-164143 | @ -4.6 dB 1110 ms |

Considering the -1 dB conditions and the channel model, UE can at least achieve the same performance as the value listed in Table I. Considering certain implementation margin, we propose the number of NPSS and NSSS for Cell detection at -1 dB as 22 NPSS+4 NSSS.

Observation 1: Cell detection at -1 dB can be achieved with 22 NPSSS +4 NSSS.

For Cell measurement, based on the agreement in [7], RAN4 defined the requirements based on NRS. 400 ms with 30 NRS samples are assumed for measurement period. As mentioned above, the bottleneck of the mobility performance is the cell detection delay. As for cell measurement, the delay is shorter and the delay will not be significantly reduced in at higher SNR level.

Observation 2: For Cell measurement (NRS based or NSSS based), it is not the bottleneck of the mobility performance and the delay will not be significantly reduced in at higher SNR level.

Thus, the assumed number of samples for measurement is proposed as 30 and 8 for NRS and NSSS respectively.

Regarding the Rmax which may have impacts on RLM requirements, RAN1 achieve following agreements in last meeting:

Agreement For the issue of handling NPDCCH postponing, down-select among the following options: - Option 1: No new periodicities are introduced for NPDCCH search space monitoring - FFS: Changes in postponing / overlapping rules, if any - FFS: Introduction of constraints on the configuration of search space parameters (e.g. Rmax, G…), if any - Option 2: New periodicities are introduced for NPDCCH search space monitoring - FFS: Details of the new periodicity, e.g. the existing periodicity is scaled by a fixed factor F, explicit signalling of the new periodicity, reusing the NB-IoT TN TDD periodicities, etc.

It is still FFS whether to limit the number of NPDCCH repetition. Thus, RAN4 shall wait for further RAN1 agreements on this.

Observation 3: Whether to introduce NPDCCH repetition constraints are under discussion in RAN1.

Regarding Q3, RAN1 reached following working assumption in RAN1#120 meeting:

Working assumption Uplink transmission gaps do not apply in NB-IoT NTN TDD.

From our understanding, Q3 is a RAN1 specific question. If we follow HF-FDD principle, UE is not required to monitor. If we follow TDD principle, there is no such constraints. Based on above RAN1 WS, UE is allowed to monitor DL for synchronization. However, from RRM requirements perspective, Q3 is not essential to define the RRM requirements. It should be clarified in RAN1 if necessary.

Thus, based on the analysis above, the table for Q1~	Q3 are provided as follows.

Table II. Assumptions for each RRM requirements.

|  | Question Q1 | Question Q2 | Question Q3 |
| --- | --- | --- | --- |
| Issue 1: Cell reselection (4.6A.2) | Detection: NPSS+NSSS | Detection: 22 NPSS+4 NSSS | No impacts on defining requirements. Could be clarified in RAN1 if needed. |
| Issue 2: Cell measurement in connected mode (8.14A) | Detection: NPSS+NSSS Measurement: NRS or NSSS | Detection: 22 NPSS+4 NSSS NSSS: 8 NRS: 30 |  |
| Issue 3: RRC re-establishment (6.5A) | Detection: NPSS+NSSS Measurement: NRS | Detection: 22 NPSS+4 NSSS Measurement: 30 NRS |  |
| Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A) | NA | NA |  |
| Issue 5: RLM (7.23A) | NRS | Rmax ≤ 64: 10/5 for OOS/INS Rmax> 64: FFS |  |

Proposal 1: The assumptions on for each procedure are provided in Table II.

RAN1 send an LS to RAN4 [9], which is to ask about whether there is any issues identified by RAN4 about following RAN1 working assumption.

Working assumption For precompensation, from RAN1 perspective: - The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed. - FFS: whether spec impact is in RAN1, RAN4 or both

From our understanding, since there is sufficient vacant time period before every 8 consecutive UL subframes, UE can perform pre-compensation before it without pre-compensation gap, and the segmented operation is not needed.

Proposal 2: RAN4 to confirm RAN1 working assumption. And the segmented pre-compensation is not needed.

# Conclusions

Observation 1: Cell detection at -1 dB can be achieved with 22 NPSSS +4 NSSS.

Observation 2: For Cell measurement (NRS based or NSSS based), it is not the bottleneck of the mobility performance and the delay will not be significantly reduced in at higher SNR level.

Observation 3: Whether to introduce NPDCCH repetition constraints are under discussion in RAN1.

Proposal 1: The assumptions on for each procedure are provided in Table II.

Proposal 2: RAN4 to confirm RAN1 working assumption. And the segmented pre-compensation is not needed.

# References

[1] RP-242415 New WID on introduction of IoT-NTN TDD mode

[2] RP-243293 Revised WID on introduction of IoT-NTN TDD mode

[3] WF on RRM requirements for IoT_NTN_TDD

[4] R4-164150 Cell detection times in NB-IoT

[5] R4-164089 Discussion on NB-IoT cell search

[6] R4-164143 Cell detection simulation results for NB-IoT

[7] R4-78AH-0212 Minutes of Meeting on NB-IoT RRM AH

[8] R4-162872 Way forward on NB-IoT RLM

[9] R1-2503142 LS on precompensation for NB-IoT NTN TDD mode